1.	Introduction	4

1.1.	What is PageGate	4

1.2.	Features & Capabilities	4

1.2.1.	General	4

1.2.2.	Commandline/Ascii	5

1.2.3.	Email Gateway	6

1.2.4.	Serial Interface	6

1.2.5.	TAP Interface	6

1.2.6.	Web Gateway	6

1.2.7.	GUI Client	7

1.3.	Shareware Information	7

1.4.	Purchasing	8

1.5.	Contact Information	8

1.6.	Version Information	9

1.7.	End User License Agreement	9

2.	Installation	11

2.1.	Overview	11

2.2.	PageGate Paging Server	11

2.2.1.	System Requirements	11

2.2.2.	Pre-installation	11

2.2.3.	Upgrading	12

2.2.4.	Installation Instructions	12

2.2.5.	Post-installation	14

2.2.6.	Advanced Installation	15

2.2.7.	Installation troubleshooting	15

2.3.	PageGate Admin	16

2.3.1.	System Requirements	16

2.3.2.	Pre-installation	16

2.3.3.	Installation Instructions	16

2.3.4.	Post-installation	18

2.3.5.	Installation troubleshooting	18

2.4.	PageGate Monitor	18

2.4.1.	System requirements	18

2.4.2.	Pre-installation	19

2.4.3.	Installation Instructions	19

2.4.4.	Post-installation	21

2.4.5.	Installation troubleshooting	21

2.5.	PageGate GUI Client	21

2.5.1.	System Requirements	21

2.5.2.	Pre-installation	21

2.5.3.	Installation Instructions	22

2.5.4.	Post-installation	24

2.5.5.	Installation troubleshooting	24

3.	Running the first time	25

3.1.	One-time configuration	25

3.2.	Starting & Stopping PageGate	25

3.3.	Where to find Help	26

3.4.	5 steps to sending your first message	26

3.5.	Monitoring PageGate	27

4.	Modules	28

4.1.	Overview	28

4.2.	PG Admin	28

4.2.1.	Overview	28

4.2.2.	Starting & Stopping	28

4.2.3.	Using	28

4.2.4.	Objects	28

4.3.	PG Monitor	42

4.3.1.	Overview	42

4.3.2.	Starting & Stopping	42

4.3.3.	Configuration	42

4.3.4.	Using	42

4.4.	Service Manager	43

4.4.1.	Overview	43

4.4.2.	Starting & Stopping	43

4.5.	Scheduler	44

4.5.1.	Overview	44

4.5.2.	Starting & Stopping	44

4.6.	Connectors	44

4.6.1.	Overview	44

4.6.2.	Starting & Stopping	44

4.6.3.	Multiple Dialers	44

4.6.4.	Direct connect	45

4.6.5.	Dedicated Carriers	45

4.6.6.	Touch Tone Delivery	45

4.7.	GetAscii (commandline/ascii)	46

4.7.1.	Overview	46

4.7.2.	Settings	46

4.7.3.	Advanced Polling	46

4.7.4.	Starting & Stopping	49

4.7.5.	Using sendpage.exe	49

4.7.6.	Using ascii text files	49

4.7.7.	Ad-Hoc	50

4.7.8.	MultiPage	50

4.7.9.	Additional Notes	51

4.8.	GetMail (email gateway)	51

4.8.1.	Overview	51

4.8.2.	Settings	52

4.8.3.	Ad-Hoc Mail Settings	52

4.8.4.	Dial-Up Networking Settings	53

4.8.5.	Domain POP Mail Settings	53

4.8.6.	Starting & Stopping	54

4.8.7.	Using PageGate with a registered domain name, a static IP address, and a dedicated internet connection.	54

4.8.8.	Using PageGate with a registered domain name and a dialup connection	55

4.8.9.	Using PageGate in conjunction with another mail server (i.e. MS Exchange, Unix based server, etc.)	56

4.8.10.	Using PageGate without a registered domain name and using POP mail collection	56

4.8.11.	MultiPage	58

4.8.12.	POP Mail Settings	58

4.8.13.	Email Aliases	59

4.8.14.	Email Filtering	59

4.8.15.	Additional Notes	59

4.9.	GetSerial	60

4.9.1.	Overview	60

4.9.2.	Settings	60

4.9.3.	Starting & Stopping	62

4.10.	GetTap	62

4.10.1.	Overview	62

4.10.2.	Settings	62

4.10.3.	Starting & Stopping	63

4.11.	GetWeb (webpage gateway)	63

4.11.1.	Overview	63

4.11.2.	Settings	63

4.11.3.	Starting & Stopping	64

4.11.4.	Web server configuration	64

4.11.5.	Using the webgate.exe CGI	65

4.12.	PageGate GUI Client	71

4.12.1.	Overview	71

4.12.2.	Settings	71

4.12.3.	Starting & Stopping	71

4.12.4.	Additional Notes	71

5.	Concepts	72

5.1.	Ad-Hoc	72

5.2.	Carrier	72

5.3.	Failover	72

5.4.	Group	72

5.5.	Hold Messages	72

5.6.	Init String	73

5.7.	Interfaces	73

5.8.	Log Files	74

5.9.	Message Content	74

5.10.	Message Length	74

5.11.	Modularity/Scalability	74

5.12.	MultiPage	74

5.13.	Notify Only	75

5.14.	Numeric (digital) Pagers	75

5.15.	On-Call Group	75

5.16.	Pager (Beeper)	75

5.17.	Recipient	75

5.18.	Reliapage	75

5.19.	Repeating Pages	76

5.20.	Reports	76

5.21.	Scheduled Pages	76

5.22.	Touch-Tone Messages	76

6.	Technical Information	78

6.1.	Adding Modules to PageGate	78

6.2.	Advanced Installation	78

6.3.	Integration	79

6.4.	Run Styles	81

6.5.	Uninstalling PageGate	82

7.	Troubleshooting	83

7.1.	Installation	83

7.2.	Init Strings	83

7.3.	Connecting to a carrier	84

7.3.1.	Make sure your modem is operating correctly	84

7.3.2.	Make sure you have a dial tone	84

7.3.3.	Make sure you are connecting to a modem	84

7.3.4.	Make sure you are connecting to a paging terminal	85

7.3.5.	Check the PageGate Dialer Log	85

7.4.	Misc	86

7.4.1.	I receive the error 'Unable to access modem'	86

7.4.2.	I receive the message 'Warning unable to access the database'	86

7.4.3.	I receive the error 'Program halted.  Database error...'	86

7.4.4.	When I start the PG Admin program, I get the error 'Error 35602 key is not unique in collection...'	87

7.4.5.	I have a US Robotics (3 Com) modem. I have tried the init string but still get inconsistent results.	87

7.4.6.	I am certain that everything is set correctly, but PageGate won't dial with my modem.	87

7.4.7.	I have a 56K modem and am unable to connect to the paging carrier.	87

7.4.8.	My modem does not hang up after sending a message.	88

7.4.9.	My pager keeps receiving garbled messages, why?	88

7.4.10.	PageGate says the message was sent, but it is never received on the pager, why?	88

7.4.11.	When I try to install, I get a "missing runtime DLL" message.	88

7.4.12.	When I try to send a message through a web page nothing happens. My server is NT 4.0 and my web server is Microsoft IIS.	88

7.4.13.	Messages aren't being acknowledged by either the scheduler or the dialers.	88

7.4.14.	PageGate stopped sending messages at the end of the month	89

7.4.15.	I'm using commandline/ascii interface and I am trying to send message from notepad.	89

7.4.16.	With Windows 2000, I can't get GetMail to work	89

7.4.17.	With Windows 2000, I can't get PageGate to use my modem	89

8.	FAQs	90

8.1.	How does your software treat email messages with graphic encapsulation e.g. attachments, jpg, MIME ?	90

8.2.	Will PageGate support more than one modem per server ?	90

8.3.	Will PageGate queue messages on the PageGate server for when dialout lines are busy?	90

8.4.	Can I send the same messages to multiple pagers ?	90

8.5.	Can PageGate run as a service an NT server?	90

8.6.	Will PageGate work with a direct connection to the paging terminal (a direct serial connection, no modem) ?	90

8.7.	Can PageGate be modified to adhere to a different paging protocol, that requires '' character before inputting text ?	91

8.8.	What operating systems operating systems will PageGate work on?	91

8.9.	Can PageGate's modules be run on separate machines?	91

8.10.	What mail servers does PageGate work with?	91

8.11.	How do I get technical support ?	91

8.12.	What web servers will PageGate work with?	91

# 1.	Introduction

## 1.1.	What is PageGate

PageGate is a sophisticated wireless messaging (paging) application.  The heart of the application is a high throughput paging engine that has the ability to support multiple dialers (modems), direct cable connections, and internet connections for message delivery.

PageGate supports several front-end interfaces (gateways) that can be used individually or in conjunction with each other. This allows PageGate to receive messages from many sources, and deliver the messages to wireless messaging devices (including alphanumeric pagers, numeric pagers, mobile phones, PIMs, etc.)   The front-end interfaces include:

PageGate GUI Client - a GUI (Graphical User Interface) windows client that enables an entire network to send messages from its Windows based workstations.

Commandline/Ascii interface - enables many 'off the shelf' and custom applications to send messages to PageGate.  This is also the choice interface for many integrators that need wireless messaging functionality.

Email gateway interface - enables email messages to be accepted by PageGate.  This interface includes the capabilities of a full functional SMTP/POP3 mail server.

CGI web interface - allows messages to be sent directly from web pages to PageGate

Serial port interface - enables PageGate extract data from an incoming stream of data over a serial port.

TAP interface - enables PageGate to accept incoming messages via modem (or direct connection) by way of the TAP paging protocol.

PageGate supports several Protocols for message delivery:

TAP - Telocator Alphanumeric Protocol - the most widely used protocol by carriers for dialup modem message delivery.

UCP - Universal Computer Protocol - another widely used dialup protocol

TONE - DTMF (Touch Tone) message delivery for numeric pagers

GSM - Support for direct message delivery to a GSM modem or phone via a serial connection

SNPP - Simple Network Paging Protocol - a new internet based paging protocol that is being adopted by many carriers

SMTP - Simple Mail Transport Protocol - a protocol designed for email, but used by many carriers to accept messages over the internet.

WCTP - Wireless Communications Transfer Protocol - another new internet based paging protocol that is being adopted by many carriers

PageGate's modularity (PageGate's modules can be run across several separate machines), scalability (direct cable connection and support for multiple dialers), multiple front-end interfaces, and high-end features (group paging, scheduled pages, repeating pages, on-call groups, ad-hoc paging, etc.), and multiple delivery protocols makes it a perfect solution for most wireless messaging needs.

## 1.2.	Features & Capabilities

### 1.2.1.	General

Group paging - messages can be sent to a predefined group of recipients

Multiple recipients - allows a single message to be sent to more than one recipient and/or group

Intelligent message queuing - messages for the same carrier are grouped together and sent during a single communications session

Multiple delivery methods - PageGate can be connected to a paging terminal by a direct cable connection, through a modem, or via the internet

Message splitting - Long messages can be intelligently split into multiple smaller messages

Settable character limits - The maximum number of characters to be sent in a message is settable by carrier and individual recipient

On-call groups - A schedule can be setup for the recipients in a group, allowing PageGate to send messages to only the recipients scheduled to receive messages at that point in time.

Multiple protocol support - PageGate supports the following protocols: TAP, UCP, TONE, GSM, SNPP, SMTP, WCTP for message delivery.

Message logging - PageGate keeps general and detailed logs to help monitor the activity of all of its modules.

Real-time Monitoring - PageGate's PG Monitor program allows for real-time monitoring of all of PageGate's modules from the paging server, or any workstation connected via TCP/IP

Modular / Scalable - PageGate's modular architecture allows the application to be run across several computers for high throughput applications.

Reliapage - Provides a way for recipients to be aware that a message has been missed.

Requeue messages - Missed messages can be requeued for re-transmission to the paging carrier

High throughput - PageGate's intelligent queuing of messages and multiple dialers allows for the delivery of a large number of messages per hour.

Failover support - If messages fail delivery, they can be automatically rerouted out a different interface, or to another recipient.

Extensive Reporting - Reports can be printed and previewed for all messages sent and received.

Message Holding - Messages can be held overnight and delivered in the morning.

### 1.2.2.	Commandline/Ascii

DOS Commandline - messages can be sent from a DOS prompt

Windows Commandline - messages can be sent from a Windows commandline

Polling - a directory can be polled for Ascii text files containing message information

Advanced Polling - message recipients can be defined by text file names or locations.  Multiple files/folders can be scanned for message information

Multiple file type support - Additions to files can be detected, changes in files can be detected, message information can be parsed out of most file formats

Ad-Hoc recipients - messages can be sent by using PIN numbers (recipients don't need to be setup inside PageGate).  Only available with 5000 user license or greater

Multi-Page recipients - multiple recipients can be specified within a single message

Message Tracking - messages sent via this interface can be tracked via status directories

3rd party application support - messages can be sent from any application that can issue a commandline command (shell command) or create an Ascii text file.

Custom Application Support - this is a popular interface for developers who want an easy way to add paging to their application.

### 1.2.3.	Email Gateway

Email Server - PageGate's email interface is a full email server.  Along with its ability to send and receive email for paging, the server can act as a regular SMTP/POP mail server for company email.

Forwarding - each recipient can have their messages forwarded to an additional email account

Ad-Hoc recipients - messages can be sent by using PIN numbers (recipients don't need to be setup inside PageGate).  Only available with 5000 user license or greater

Multi-Page recipients - multiple recipients can be specified within a single email message

Email Outcome - the success or failure of messages sent can be automatically returned to the sender via email

Numeric Email Notification - numeric pagers can receive notification of the arrival of email messages

Full Module Integration - integrates with all other PageGate modules for forwarding of messages received via any other interface

Filtering - PageGate can be setup to only send messages that are received from specific email addresses or domains.  Also PageGate can be setup to block messages that come from specific email addresses or domains.

### 1.2.4.	Serial Interface

Serial Interface - PageGate's serial port interface accepts data in over a serial port and extracts messages from it.

Direct support - the serial interface can support a direct serial cable connection to the computer's COM port

Modem support - the serial interface can support a modem connected to the serial port

Message data - the recipient, sender and message can be extracted from the received data, or each can be hard coded

Communications - the serial interface supports a whole range of communication speeds, protocols, and flow control options.

Lookup Tables - lookup tables can be used to automatically substitute text data in the place of codes received over the serial connection.

### 1.2.5.	TAP Interface

Tap Interface - PageGate can accept incoming messages via modem or direct serial connection, via the industry standard TAP protocol.

Routing - incoming messages can be re-routed to different recipients or groups.

Logging - standalone applications (like NotePager Pro) can send messages through PageGate to enable message logging and tracking in a central location.

Centralization - standalone applicatons (like NotePager Pro) can be setup to use PageGate as its only carrier.  Messages will be sent through PageGate and delivered to any number of actual carriers and delivery methods.

### 1.2.6.	Web Gateway

Automatic Web Page Generation - web pages can be created automatically by using template files

Custom Web Page Support - messages can be sent from within custom web pages

Flexible Web Server Support - PageGate can be used with any windows based web server

Ad-Hoc recipients - messages can be sent by using PIN numbers (recipients don't need to be setup inside PageGate).  Only available with 5000 user license or greater

Multi-Page recipients - multiple recipients can be specified within a single message

Drop-Down List Support - web pages with drop down lists of recipients is supported, and is automatically updated

Recipient Verification - users are notified immediately if a message is sent to an invalid recipient

### 1.2.7.	GUI Client

Local and network recipient lists - on each workstation you can choose recipients from the network recipient list, or from a smaller local recipient list that can be defined for each workstation

Scheduled messages - messages can be scheduled to be sent at a future date/time

Repeating messages - messages can be scheduled to repeat at predefined interval

Multiple recipients - allows a single message to be sent to more than one recipient and/or  group

Pre-programmed messages - on each workstation, up to 100 pre-programmed messages can be defined

Real-time message status log - each workstation can track the progress of each message from start to finish

Real-time message status counters - displays current number of messages pending, sent, or failed from each workstation

Message character counter - displays the length of the current message

Message history log - displays a history of all messages sent from the workstation

Point and click operation - an intuitive user interface makes the PageGate Client very easy to use

Spell check - a spell check option is available on systems running MS Word

Recipient cueing - Recipients may be selected by typing the first few letters of their name

Restricted recipient lists - PageGate Client users can be restricted to only be able to use a local recipient list

Reporting - Print or preview reports for messages that are pending or already sent.

## 1.3.	Shareware Information

PageGate defaults into a 'shareware mode' until it is registered (purchased). Registration information can be entered into PageGate's Admin program by choosing the 'Registration' object.  The shareware version of PageGate only allows for two active recipients at any given time, and the server will stop itself every 3-5 days. PageGate's Admin program can be used to restart the paging engine after this time period.

If a less restrictive evaluation license is necessary in order to evaluate PageGate in full, contact our sales department to discuss the availability of a thirty-day evaluation key without the above restrictions.

Shareware is based on the try it before you buy it concept. This software is not free. You may try it. If you like the software and would like a complete registered version you must contact sales@notepage.com. The registration fee allows us to continue to develop quality products, notify you of updates, and provide technical support. NotePage, Inc. accepts credit cards (Visa, MasterCard, Discover & American Express), Purchase Orders, checks drawn on US banks, and wire transfers. If you have any questions or concerns regarding registration please contact sales@notepage.com. If you use any of NotePage, Inc.'s software and do not register after the trial period it is considered software piracy and is illegal.

## 1.4.	Purchasing

PageGate can be ordered directly from NotePage, Inc:

The most current PageGate pricing can be found on our website:

www.notepager.net/pricing.htm

www.notepager.com/pricing.htm

www.notepage.net/pricing.htm

NotePage, Inc.

PO Box 296

Hanover, MA 02339

Phone (781) 829-0500

Fax (781) 582-1869

Accepted Payment forms are:

Pre-payment

COD Company Check (continental US only)

MasterCard

Visa

Discover

Purchase Orders

All payments are in US Dollars

PageGate can be ordered on-line at:

www.notepager.net/purchase.htm

www.notepager.com/purchase.htm

www.notepage.net/purchase.htm

Accepted Payment forms are:

MasterCard

Visa

Discover

American Express

Purchase Orders

PageGate can be purchased through a NotePage authorized reseller, who can also be contracted for the installation of our products.  Please contact us for a reseller in your area.

## 1.5.	Contact Information

NotePage, Inc.

291 Rockland Street

Suite 13

Hanover, MA  02339

USA

Mailing Address

PO Box 296

Hanover, MA  02339

web 	www.notepage.net

email	sales@notepage.net

phone	(781) 829-0500

fax	(781) 582-1869

## 1.6.	Version Information

PageGate Help File

For PageGate version 4.0

Copyright 1997-2002 NotePage, Inc.

## 1.7.	End User License Agreement

END-USER LICENSE AGREEMENT FOR PageGate

IMPORTANT PLEASE READ THE TERMS AND CONDITIONS OF THIS LICENSE AGREEMENT CAREFULLY BEFORE CONTINUING WITH THIS PROGRAM INSTALL:

NotePage, Inc.'s End-User License Agreement ("EULA") is a legal agreement between you (either an individual or a single entity) and NotePage, Inc. for the NotePage software product(s) identified above which may include associated software components, media, printed materials, and "online" or electronic documentation ("SOFTWARE PRODUCT"). By installing, copying, or otherwise using the SOFTWARE PRODUCT, you agree to be bound by the terms of this EULA. This license agreement represents the entire agreement concerning the program between you and NotePage, Inc., (referred to as "licensor"), and it supersedes any prior proposal, representation, or understanding between the parties.  If you do not agree to the terms of this EULA, do not install or use the SOFTWARE PRODUCT.

The SOFTWARE PRODUCT is protected by copyright laws and international copyright treaties, as well as other intellectual property laws and treaties. The SOFTWARE PRODUCT is licensed, not sold.

1. GRANT OF LICENSE.  The SOFTWARE PRODUCT is licensed as follows:

(a) Installation and Use. NotePage, Inc. grants you the right to install and use copies of the SOFTWARE PRODUCT on your computer running a validly licensed copy of the operating system for which the SOFTWARE PRODUCT was designed [e.g., Windows 95®, Windows NT®, Windows 98®, Windows 2000 ®].

(b) Backup Copies. You may also make copies of the SOFTWARE PRODUCT as may be necessary for backup and archival purposes.

2. DESCRIPTION OF OTHER RIGHTS AND LIMITATIONS.

(a) Maintenance of Copyright Notices.You must not remove or alter any copyright notices on any and all copies of the SOFTWARE PRODUCT.

(b) Distribution. You may not distribute registered copies of the SOFTWARE PRODUCT to third parties.  Evaluation versions available for download from NotePage, Inc.'s websites may be freely distributed.

(c) Prohibition on Reverse Engineering, Decompilation, and Disassembly. You may not reverse engineer, decompile, or disassemble the SOFTWARE PRODUCT, except and only to the extent that such activity is expressly permitted by applicable law notwithstanding this limitation.

(d)  Rental. You may not rent, lease, or lend the SOFTWARE PRODUCT.

(e) Support Services. NotePage, Inc. may provide you with support services related to the SOFTWARE PRODUCT ("Support Services").  Any supplemental software code provided to you as part of the Support Services shall be considered part of the SOFTWARE PRODUCT and subject to the terms and conditions of this EULA.

(f) Compliance with Applicable Laws. You must comply with all applicable laws regarding use of the SOFTWARE PRODUCT.

3. TERMINATION

Without prejudice to any other rights, NotePage may terminate this EULA if you fail to comply with the terms and conditions of this EULA. In such event, you must destroy all copies of the SOFTWARE PRODUCT in your possession.

4. COPYRIGHT

All title, including but not limited to copyrights, in and to the SOFTWARE PRODUCT and any copies thereof are owned by NotePage, Inc. or its suppliers. All title and intellectual property rights in and to the content which may be accessed through use of the SOFTWARE PRODUCT is the property of the respective content owner and may be protected by applicable copyright or other intellectual property laws and treaties.  This EULA grants you no rights to use such content.  All rights not expressly granted are reserved by NotePage.

5. NO WARRANTIES

NotePage, Inc. expressly disclaims any warranty for the SOFTWARE  PRODUCT. The SOFTWARE PRODUCT is provided "As Is" without any express or implied warranty of any kind, including but not limited to any warranties of merchantability, noninfringement, or fitness of a particular purpose. NotePage does not warrant or assume responsibility for the accuracy or completeness of any information, text, graphics, links or other items contained within the SOFTWARE PRODUCT.  NotePage makes no warranties respecting any harm that may be caused by the transmission of a computer virus, worm, time bomb, logic bomb, or other such computer program. NotePage further expressly disclaims any warranty or representation to Authorized Users or to any third party.

6. LIMITATION OF LIABILITY

In no event shall NotePage, Inc. be liable for any damages (including, without limitation, lost profits, business interruption, or lost information) rising out of 'Authorized Users' use of or inability to use the SOFTWARE PRODUCT, even if NotePage, Inc. has been advised of the possibility of such damages.  In no event will NotePage be liable for loss of data or for indirect, special, incidental, consequential (including lost profit), or other damages based in contract, tort or otherwise. NotePage, Inc. shall have no liability with respect to the content of the SOFTWARE PRODUCT or any part thereof, including but not limited to errors or omissions contained therein, libel, infringements of rights of publicity, privacy, trademark rights, business interruption, personal injury, loss of privacy, moral rights or the disclosure of confidential information.

# 2.	Installation

## 2.1.	Overview

PageGate is designed to be a highly modular application.  This design allows PageGate's modules to be run across multiple computers.  The following installation information covers installation of PageGate on a single computer.  For information on installing and running PageGate across multiple computers, please refer to the section titled 'Advanced Installation'.

In a PageGate Server installation, the following modules are installed:

PageGate Server - This is the control module for PageGate.  It is constantly running in the background whenever the PageGate application is running.  It monitors, starts, and stops the other PageGate modules.

PageGate Scheduler - This module processes all messages sent by PageGate.  It manages scheduled and repeating messages, in addition to scheduling the delivery of each message by a dialer.

PageGate Dialers - The dialers are responsible for the actual delivery of the messages to a paging carrier(s).

PageGate Interfaces - These modules (GetAscii, GetMail, GetWeb, and GUI) accept messages from various sources, into the PageGate application.

PageGate Admin - The administration program is used to setup, control, and maintain the PageGate application.

PageGate Monitor - The monitoring application allows for real-time monitoring of the PageGate application from the local machine, or from any other workstation that is connected to the paging server via TCP/IP.

## 2.2.	PageGate Paging Server

### 2.2.1.	System Requirements

Minimum requirements to run PageGate:

Operating System:  Windows 95, 98, ME, NT, 2000, XP

Processor:  Pentium 200Mhz

Memory:  64 Meg of free memory (typically 96 Meg for Windows 95/98/ME,  128 Meg for Windows NT,  256 Meg for Windows 2000, or XP - on a bare system)

Storage:  50 Meg of free disk space (for application and database)

Communications:  300 baud or faster modem for dialup connections, a serial cable for direct connections, or a dedicated internet connection.

### 2.2.2.	Pre-installation

There are a few things that should be verified and setup before the installation of PageGate:

1. If you are upgrading from a previous registered version of PageGate, be sure to obtain a new registration key for PageGate BEFORE installing over your existing version of PageGate.

2. Be sure your hardware meets the minimum specs for PageGate version 4.x

3. Be sure the computer that PageGate is to be installed on is configured for your network.  The monitoring program uses TCP/IP.  Although TCP/IP is not required for running PageGate, it is recommended that it be installed on the PageGate server so the monitoring program can be used.

4. Be sure that you have sufficient permissions (if applicable) to install new programs on the chosen computer.

5. If PageGate is to be run as a Windows NT/2000/XP system service, be sure you have an account setup with 'log on as a service' rights.  Please refer to the section titled 'Run Styles'  for more information.

6. In some cases, you will be asked to restart your computer after the installation of PageGate.  If so, this must be done before PageGate is run.  Be sure that you install PageGate at a time when this can be done.

7. ALWAYS be sure to have a complete backup of your system before installing ANY new software! The installation of PageGate is NOT and exception to this rule.

### 2.2.3.	Upgrading

When PageGate is upgraded from a previous version, all the previous settings and information will be automatically upgraded too (as long as the new version of PageGate is installed  to the same location as the older version).

Upgrading from PageGate version 3.x

1.  IMPORTANT - If you are currently running a registered version of PageGate, be sure to obtain a registration key for version 4.x BEFORE continuing.  Your version 3.x key will not work in version 4.x.

2. If you are running PageGate 3.x as a system service (NT service), switch it back to run as an application before upgrading.  After upgrading, you can switch it back to run as a service.

3. Use the regular installation instructions found later in this section, but be sure to install version 4.x to the same folder that 3.x was installed to.

4. After the installation of version 4.x is complete, run the PG Admin program before starting the PageGate server.  This will upgrade the database structure.

Upgrading from PageGate version 4.x to a newer version of 4.x

1. If you are running PageGate 4.x as a system service (NT service), switch it back to run as an application before upgrading.  After upgrading, you can switch it back to run as a service.

2. Install the newer version of 4.x over your existing version.

3. After the installation of version 4.x is complete, run the PG Admin program before starting the PageGate server.  This will upgrade the database structure.

### 2.2.4.	Installation Instructions

#### 2.2.4.a.	CD-Rom Installation

1.  Insert the PageGate CD-Rom into your computer.

2.  PageGate's setup routine will (on most computers) automatically start.  If the setup program starts, go to step 3 below.  If not, click on the Windows start button, and then choose 'Run' from the menu.  Type in 'd:\setup' without the quotes (in this example the CD-Rom drive letter is 'd'. If your CD-Rom drive letter is different, use it instead).  Next hit the 'OK' button.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install PageGate for the first time, you should choose the first option.

4.  The next setup screen will give you the option to review PageGate's Readme and Release Notes files.  If you choose to display either file, they will be shown when you hit 'Next'.

5.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

6.  On the next screen you can specify a location where you want PageGate installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

7.  You will now be prompted for the name of the group that the PageGate icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

8.  The setup program is now ready to install PageGate.  Click on the 'Next' button to start the installation process.

9.  The setup program will now copy the PageGate program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

10.  The next screen will tell you that PageGate has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

11.  In some cases, your computer will ask to be restarted after the installation of PageGate.  If you are prompted to do so, you must restart your computer before running PageGate.

#### 2.2.4.b.	Single executable (.exe) installation

1.  Run the executable file that you downloaded (pgv4.exe).

2.  The setup program will present you with options for which part of PageGate you want to install.  To install PageGate for the first time, you should choose the first option.

3.  The next setup screen will give you the option to review PageGate's Readme and Release Notes files.  If you choose to display either file, they will be shown when you hit 'Next'.

4.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

5.  On the next screen you can specify a location where you want PageGate installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

6.  You will now be prompted for the name of the group that the PageGate icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

7.  The setup program is now ready to install PageGate.  Click on the 'Next' button to start the installation process.

8.  The setup program will now copy the PageGate program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

9.  The next screen will tell you that PageGate has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

10.  In some cases, your computer will ask to be restarted after the installation of PageGate.  If you are prompted to do so, you must restart your computer before running PageGate.

#### 2.2.4.c.	Single archive (.zip) installation

1.  To extract the PageGate installation file from the archive (.zip) file, you will need a program such as WinZip or PKZip.  Extract the contents of the .zip file that you have downloaded (pgv4.zip) into a temporary directory by using one of these utilities.

2.  Run the file named pgv4.exe from the location where you extracted PageGate's installation file.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install PageGate for the first time, you should choose the first option.

4.  The next setup screen will give you the option to review PageGate's Readme and Release Notes files.  If you choose to display either file, they will be shown when you hit 'Next'.

5.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

6.  On the next screen you can specify a location where you want PageGate installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

7.  You will now be prompted for the name of the group that the PageGate icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

8.  The setup program is now ready to install PageGate.  Click on the 'Next' button to start the installation process.

9.  The setup program will now copy the PageGate program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

10.  The next screen will tell you that PageGate has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

11.  In some cases, your computer will ask to be restarted after the installation of PageGate.  If you are prompted to do so, you must restart your computer before running PageGate.

#### 2.2.4.d.	Promptless Automatic Installation

The PageGate Server can be installed unattended.  This helps network administrators to automate the installation of the this program.

1.  Start the PageGate setup program as usual (see above).

2.  When the first installation screen is shown, choose 'Exit' and copy the 'PGSetup' folder that has now been created under your system's temporary directory, (usually c:\windows\temp or c:\temp) to another location.

3.  Exit out of the installation program.

4.  Use the following commandline syntax to run a promptless install.

Note: promptless installations installs the PageGate applications to their default locations only.

<path>pgssetup.exe /S

The <path> option should be replace with the path to where you copied the 'PGSetup' folder in step #2 above.

Here is an example of a promptless install of the PageGate server if the PGSetup folder was copied to C:\

C:\PGSetup\pgssetup.exe /S

Notes:

The PageGate server can be installed normally by running pgssetup.exe.  After the installation finishes, the install files are automatically removed from your system.  If you want the setup files to remain after installation, use a /K in the commandline.

### 2.2.5.	Post-installation

1.  If you installed from CD-Rom, remove the CD and keep it in a safe place.  You will need it again if you ever need to reinstall PageGate, or want to install the PageGate Administration program, the PageGate Monitor program, or the PageGate GUI Client on a workstation.

2.  If you installed PageGate from a single downloaded file (pgv4.exe or pgv4.zip), be sure to make a copy of the file and store it somewhere safe. You will need it again if you ever need to reinstall PageGate, or want to install the PageGate Administration Program, the PageGate Monitor program, or the PageGate GUI Client on a workstation.

3.  After PageGate is successfully installed, please continue to the section 'Running PageGate for the first time' for important information on the setup and configuration of PageGate.

### 2.2.6.	Advanced Installation

PageGate's flexibility and scalability is due to its modular design.  The modularity of PageGate allows each of its modules to be run on a separate computer.  Here is an example:

- The PageGate Scheduler module and the PageGate Database run on the production file server.

- The GetMail module runs on a separate server

- The GetWeb module runs on the web server

- 1 to 16 Dialer modules run on a communications server

- The PageGate Administration and Monitoring programs run on an administrator's workstation

- The PageGate GUI Client runs on several workstations on the LAN

In contrast, all of the above modules can run on a single workstation or server.

Steps to install PageGate's modules on additional computers:

1.  Run PageGate's setup program.  Install the PageGate application on the new workstation or server (refer to the regular installation instructions for details).

2.  Run PageGate on the new computer.

3.  The first time PageGate is run, you will be given the 'PageGate Setup Screen' (this screen can be accessed later by choosing 'Settings' from the 'Program' object in the PG Admin program).

4.  Set the 'Program Directory' to where you installed PageGate on the local machine.

5.  Set the 'Database Directory' to the same location as in the existing PageGate installation (this would need to be a shared folder on the network).

6.  Set the 'Log Directory' to the same location as in the existing PageGate installation (this would need to be a shared folder on the network).

7.  Choose which of PageGate's modules are to run on this computer.  Be sure to disable the modules you choose on any existing PageGate servers.  If you are installing PageGate simply to use the Administration program, leave all the module boxes unchecked.

8.  Apply your changes.  PageGate will now start..

9.  Use the above procedure on each computer that will run one or more of PageGate's modules.

### 2.2.7.	Installation troubleshooting

Problem:  Any number of different problems

Solution:  Be sure that there are no other programs running when you install PageGate.  There may be programs running even if you don't see them in your task bar.  Pressing <ctrl><alt><del> will bring up a list of running programs in Windows 95 and 98.  In Windows NT you can choose 'Task Manager' after hitting <ctrl><alt><del> for similar information.  Stop any program that you believe is causing the conflict.  WARNING: this should only be done by an experienced user.  WARNING: Randomly ending tasks can lock up your computer, lose data or worse.  Please consult with and experienced technician before killing any processes.  A clean reboot can sometimes also help.  Holding down the shift key while windows is starting will stop some programs from starting that automatically load at startup.  Try re-running the PageGate setup program.

## 2.3.	PageGate Admin

### 2.3.1.	System Requirements

Minimum requirements to run PageGate Admin:

Operating System:  Windows 95, 98, ME, NT, 2000, XP

Processor:  Pentium

Memory:  8 Meg of free memory (typically 64 Meg for Windows 95/98/ME, 128 Meg for Windows NT, 2000, or XP - on a bare system)

Storage:  10 Meg of free disk space

### 2.3.2.	Pre-installation

There are a few things that should be verified and setup before the installation of PG Admin:

1.  Be sure your hardware meets the minimum specs for PageGate Admin

2.  If you are installing PG Admin on any other computer than the one  running the PageGate server application, be sure the computer is configured for your network.  PG Admin must access the PageGate database through your LAN (Local Area Network).

3.  Be sure that you have sufficient permissions (if applicable) to install new programs on the chosen computer.

4.  In some cases, you will be asked to restart your computer after the installation of PG Admin.  If so, this must be done before PG Admin is run.  Be sure that you install PG Admin at a time when this can be done.

5.  ALWAYS be sure to have a complete backup of your system before installing ANY new software! The installation of PG Admin is NOT and exception to this rule.

### 2.3.3.	Installation Instructions

#### 2.3.3.a.	CD-Rom installation

Please note that the PG Admin program is automatically installed when the PageGate Server is installed.  You only need to follow these instructions if you are installing PG Admin on an additional workstation.

1.  Insert the PageGate CD-Rom into your computer.

2.  PageGate's setup routine will (on most computers) automatically start.  If the setup program starts, go to step 3 below.  If not, click on the Windows start button, and then choose 'Run' from the menu.  Type in 'd:\setup' without the quotes (in this example the CD-Rom drive letter is 'd'. If your CD-Rom drive letter is different, use it instead).  Next hit the 'OK' button.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install PG Admin you should choose the third option.

4.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

5.  On the next screen you can specify a location where you want PG Admin installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

6.  You will now be prompted for the name of the group that the PG Admin icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

7.  The setup program is now ready to install PG Admin.  Click on the 'Next' button to start the installation process.

8.  The setup program will now copy the PG Admin program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

9.  The next screen will tell you that PG Admin has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

10.  In some cases, your computer will ask to be restarted after the installation of PG Admin.  If you are prompted to do so, you must restart your computer before running PG Admin.

#### 2.3.3.b.	Single executable (.exe) installation

Please note that the PG Admin program is automatically installed when the PageGate Server is installed.  You only need to follow these instructions if you are installing PG Admin on an additional workstation.

1.  Run the executable file that you downloaded (pgv4.exe).

2.  The setup program will present you with options for which part of PageGate you want to install.  To install PG Admin you should choose the third option.

3.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

4.  On the next screen you can specify a location where you want PG Admin installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

5.  You will now be prompted for the name of the group that the PG Admin icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

6.  The setup program is now ready to install PG Admin.  Click on the 'Next' button to start the installation process.

7.  The setup program will now copy the PG Admin program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

8.  The next screen will tell you that PG Admin has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

9.  In some cases, your computer will ask to be restarted after the installation of PG Admin.  If you are prompted to do so, you must restart your computer before running PG Admin.

#### 2.3.3.c.	Single archive (.zip) installation

Please note that the PG Admin program is automatically installed when the PageGate Server is installed.  You only need to follow these instructions if you are installing PG Admin on an additional workstation.

1.  To extract the PageGate installation file from the archive (.zip) file, you will need a program such as WinZip or PKZip.  Extract the contents of the .zip file that you have downloaded (pgv4.zip) into a temporary directory by using one of these utilities.

2.  Run the file named pgv4.exe from the location where you extracted PageGate's installation file.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install PG Admin you should choose the third option.

4.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

5.  On the next screen you can specify a location where you want PG Admin installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

6.  You will now be prompted for the name of the group that the PG Admin icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

7.  The setup program is now ready to install PG Admin.  Click on the 'Next' button to start the installation process.

8.  The setup program will now copy the PG Admin program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

9.  The next screen will tell you that PG Admin has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

10.  In some cases, your computer will ask to be restarted after the installation of PG Admin.  If you are prompted to do so, you must restart your computer before running PG Admin.

### 2.3.4.	Post-installation

1.  If you installed from CD-Rom, remove the CD and keep it in a safe place.  You will need it again if you ever need to reinstall PageGate, or want to install either the PageGate Monitor program, or the PageGate GUI Client on a workstation.

2.  If you installed PG Admin from a single downloaded file (pgv4.exe or pgv4.zip), be sure to make a copy of the file and store it in a safe place. You will need it again if you ever need to reinstall PageGate, or want to install the PageGate Administration program, the PageGate Monitor program, or the PageGate GUI Client on a workstation.

### 2.3.5.	Installation troubleshooting

Problem:  Any number of different problems

Solution:  Be sure that there are no other programs running when you install PageGate.  There may be programs running even if you don't see them in your task bar.  Pressing <ctrl><alt><del> will bring up a list of running programs in Windows 95 and 98.  In Windows NT you can choose 'Task Manager' after hitting <ctrl><alt><del> for similar information.  Stop any program that you believe is causing the conflict.  WARNING: this should only be done by an experienced user.  WARNING: Randomly ending tasks can lock up your computer, lose data or worse.  Please consult with and experienced technician before killing any processes.  A clean reboot can sometimes also help.  Holding down the shift key while windows is starting will stop some programs from starting that automatically load at startup.  Try re-running the PageGate setup program.

## 2.4.	PageGate Monitor

### 2.4.1.	System requirements

Minimum requirements to run PageGate Monitor:

Operating System:  Windows 95, 98, ME, NT, 2000, XP

Processor:  Pentium

Memory:  4 Meg of free memory (typically 64 Meg for Windows 95/98/ME, 128 Meg for Windows NT, 2000, or XP - on a bare system)

Storage:  10 Meg of free disk space

Communications:  TCP/IP for communications with PageGate server

### 2.4.2.	Pre-installation

There are a few things that should be verified and setup before the installation of PG Monitor:

1.  Be sure your hardware meets the minimum specs for PG Monitor

2.  Be sure the computer that PG Monitor is to be installed on is configured for your network.  The monitoring program uses TCP/IP.  Although TCP/IP is not required for running PageGate, it is recommended that it be installed on the PageGate server so the monitoring program can be used.

3.  Be sure that you have sufficient permissions (if applicable) to install new programs on the chosen computer.

4.  In some cases, you will be asked to restart your computer after the installation of PG Monitor.  If so, this must be done before PG Monitor is run.  Be sure that you install PG Monitor at a time when this can be done.

5.  ALWAYS be sure to have a complete backup of your system before installing ANY new software! The installation of PG Monitor is NOT and exception to this rule.

### 2.4.3.	Installation Instructions

#### 2.4.3.a.	CD-Rom installation

Please note that the PG Monitor program is automatically installed when the PageGate Server is installed.  You only need to follow these instructions if you are installing PG Monitor on an additional workstation.

1.  Insert the PageGate CD-Rom into your computer.

2.  PageGate's setup routine will (on most computers) automatically start.  If the setup program starts, go to step 3 below.  If not, click on the Windows start button, and then choose 'Run' from the menu.  Type in 'd:\setup' without the quotes (in this example the CD-Rom drive letter is 'd'. If your CD-Rom drive letter is different, use it instead).  Next hit the 'OK' button.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install PG Monitor you should choose the fourth option.

4.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

5.  On the next screen you can specify a location where you want PG Monitor installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

6.  You will now be prompted for the name of the group that the PG Monitor icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

7.  The setup program is now ready to install PG Monitor.  Click on the 'Next' button to start the installation process.

8.  The setup program will now copy the PG Monitor program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

9.  The next screen will tell you that PG Monitor has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

10.  In some cases, your computer will ask to be restarted after the installation of PG Monitor.  If you are prompted to do so, you must restart your computer before running PG Monitor.

#### 2.4.3.b.	Single executable (.exe) installation

Please note that the PG Monitor program is automatically installed when the PageGate Server is installed.  You only need to follow these instructions if you are installing PG Monitor on an additional workstation.

1.  Run the executable file that you downloaded (pgv4.exe).

2.  The setup program will present you with options for which part of PageGate you want to install.  To install PG Monitor you should choose the fourth option.

3.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

4.  On the next screen you can specify a location where you want PG Monitor installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

5.  You will now be prompted for the name of the group that the PG Monitor icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

6.  The setup program is now ready to install PG Monitor.  Click on the 'Next' button to start the installation process.

7.  The setup program will now copy the PG Monitor program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

8.  The next screen will tell you that PG Monitor has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

9.  In some cases, your computer will ask to be restarted after the installation of PG Monitor.  If you are prompted to do so, you must restart your computer before running PG Monitor.

#### 2.4.3.c.	Single archive (.zip) installation

Please note that the PG Monitor program is automatically installed when the PageGate Server is installed.  You only need to follow these instructions if you are installing PG Monitor on an additional workstation.

1.  To extract the PageGate installation file from the archive (.zip) file, you will need a program such as WinZip or PKZip.  Extract the contents of the .zip file that you have downloaded (pgv4.zip) into a temporary directory by using one of these utilities.

2.  Run the file named pgv4.exe from the location where you extracted PageGate's installation file.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install PG Monitor you should choose the fourth option.

4.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

5.  On the next screen you can specify a location where you want PG Monitor installed.  The default install location is C:\Program Files\PageGate\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

6.  You will now be prompted for the name of the group that the PG Monitor icons should be added to in the Windows Start Menu.  The default group is 'PageGate'.

7.  The setup program is now ready to install PG Monitor.  Click on the 'Next' button to start the installation process.

8.  The setup program will now copy the PG Monitor program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

9.  The next screen will tell you that PG Monitor has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

10.  In some cases, your computer will ask to be restarted after the installation of PG Monitor.  If you are prompted to do so, you must restart your computer before running PG Monitor.

### 2.4.4.	Post-installation

1.  If you installed from CD-Rom, remove the CD and keep it in a safe place.  You will need it again if you ever need to reinstall PageGate, or want to install either the PageGate Monitor program, or the PageGate GUI Client on a workstation.

2.  If you installed PG Monitor from a single downloaded file (pgv4.exe or pgv4.zip), be sure to make a copy of the file and store it in a safe place. You will need it again if you ever need to reinstall PageGate, or want to install the PageGate Administration program, the PageGate Monitor program, or the PageGate GUI Client on a workstation.

3.  TCP/IP must be installed and operational on both the PageGate server computer and the workstation running PG Monitor.  The Windows ping utility should be used to verify connectivity.

### 2.4.5.	Installation troubleshooting

Problem:  Any number of different problems

Solution:  Be sure that there are no other programs running when you install PageGate.  There may be programs running even if you don't see them in your task bar.  Pressing <ctrl><alt><del> will bring up a list of running programs in Windows 95 and 98.  In Windows NT you can choose 'Task Manager' after hitting <ctrl><alt><del> for similar information.  Stop any program that you believe is causing the conflict.  WARNING: this should only be done by an experienced user.  WARNING: Randomly ending tasks can lock up your computer, lose data or worse.  Please consult with and experienced technician before killing any processes.  A clean reboot can sometimes also help.  Holding down the shift key while windows is starting will stop some programs from starting that automatically load at startup.  Try re-running the PageGate setup program.

## 2.5.	PageGate GUI Client

### 2.5.1.	System Requirements

Minimum requirements to run the PageGate GUI Client:

Operating System:  Windows 95, 98, ME, NT, 2000, or XP

Processor:  Pentium

Memory:  8 Meg of free memory (typically 64 Meg for Windows 95/98/ME, 128 Meg for Windows NT, 2000, or XP - on a bare system)

Storage:  10 Meg of free disk space

Communications:  Network access to the PageGate database

### 2.5.2.	Pre-installation

There are a few things you should verify before installing the PageGate GUI Client:

1. The PageGate paging server must be installed and operational before the PageGate GUI Client is installed.

2. The workstation must have network access to the paging database (pagegate.mdb).  The

workstation must have read, write, and change permissions to this file.

3. If you plan to use the spell-checking feature, Microsoft Word 97 or newer must be installed on the workstation.

4. If application or database passwords have been defined  in the PG Admin program, they will be needed to access the application and the PageGate database.

### 2.5.3.	Installation Instructions

#### 2.5.3.a.	CD-Rom installation

1.  Insert the PageGate CD-Rom into your computer.

2.  PageGate's setup routine will (on most computers) automatically start.  If the setup program starts, go to step 3 below.  If not, click on the Windows start button, and then choose 'Run' from the menu.  Type in 'd:\setup' without the quotes (in this example the CD-Rom drive letter is 'd'. If your CD-Rom drive letter is different, use it instead).  Next hit the 'OK' button.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install the PageGate GUI Client you should choose the second option.

4.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

5.  On the next screen you can specify a location where you want the PageGate GUI Client installed.  The default install location is C:\Program Files\PageGate Client\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

6.  You will now be prompted for the name of the group that the PageGate GUI Client icons should be added to in the Windows Start Menu.  The default group is 'PageGate Client'.

7.  The setup program is now ready to install the PageGate GUI Client.  Click on the 'Next' button to start the installation process.

8.  The setup program will now copy the PageGate GUI Client program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

9.  The next screen will tell you that the PageGate GUI Client has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

10.  In some cases, your computer will ask to be restarted after the installation of the PageGate GUI Client.  If you are prompted to do so, you must restart your computer before running the PageGate GUI Client.

#### 2.5.3.b.	Single executable (.exe) installation

1.  Run the executable file that you downloaded (pgv4.exe).

2.  The setup program will present you with options for which part of PageGate you want to install.  To install the PageGate GUI Client you should choose the second option.

3.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

4.  On the next screen you can specify a location where you want the PageGate GUI Client installed.  The default install location is C:\Program Files\PageGate Client\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

5.  You will now be prompted for the name of the group that the PageGate GUI Client icons should be added to in the Windows Start Menu.  The default group is 'PageGate Client'.

6.  The setup program is now ready to install the PageGate GUI Client.  Click on the 'Next' button to start the installation process.

7.  The setup program will now copy the PageGate GUI Client program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

8.  The next screen will tell you that the PageGate GUI Client has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

9.  In some cases, your computer will ask to be restarted after the installation of the PageGate GUI Client.  If you are prompted to do so, you must restart your computer before running the PageGate GUI Client.

#### 2.5.3.c.	Single archive (.zip) installation

1.  To extract the PageGate installation file from the archive (.zip) file, you will need a program such as WinZip or PKZip.  Extract the contents of the .zip file that you have downloaded (pgv4.zip) into a temporary directory by using one of these utilities.

2.  Run the file named pgv4.exe from the location where you extracted PageGate's installation file.

3.  The setup program will present you with options for which part of PageGate you want to install.  To install the PageGate GUI Client you should choose the second option.

4.  The following screen will display PageGate's End User License Agreement.  Please read it carefully and proceed with the installation of PageGate ONLY if you agree with ALL the terms of this agreement.

5.  On the next screen you can specify a location where you want the PageGate GUI Client installed.  The default install location is C:\Program Files\PageGate Client\. You can change the installation path by hitting the 'Browse' button.  To accept the install location, hit 'OK'.

6.  You will now be prompted for the name of the group that the PageGate GUI Client icons should be added to in the Windows Start Menu.  The default group is 'PageGate Client'.

7.  The setup program is now ready to install the PageGate GUI Client.  Click on the 'Next' button to start the installation process.

8.  The setup program will now copy the PageGate GUI Client program and support files to your computer.  If you encounter any problems during this part of the installation, please refer to the 'Installation Troubleshooting' section of the manual.

9.  The next screen will tell you that the PageGate GUI Client has been successfully installed.  Hit the 'Finish' button to exit PageGate's setup program.

10.  In some cases, your computer will ask to be restarted after the installation of the PageGate GUI Client.  If you are prompted to do so, you must restart your computer before running the PageGate GUI Client.

#### 2.5.3.d.	Promptless Automatic Installation

The PageGate GUI Client can be installed unattended.  This helps network administrators to automate the installation of the this program.

1.  Start the PageGate setup program as usual (see above).

2.  When the first installation screen is shown, stop there and copy the 'PGSetup' folder that has now been created under your system's temporary directory, (usually c:\windows\temp or c:\temp) to another location.

3.  Exit out of the installation program.

4.  Use the following commandline syntax to run a promptless install.

Note: promptless installations installs the PageGate applications to their default locations only.

<path>pgcsetup.exe /S

The <path> option should be replace with the path to where you copied the 'PGSetup' folder in step #2 above.

Here is an example of a promptless install of the PageGate GUI Client if the PGSetup folder was copied to C:\

C:\PGSetup\pgcsetup.exe /S

Notes:

The PageGate client can be installed normally by running pgcsetup.exe.  After the installation finishes, the install files are automatically removed from your system.  If you want the setup files to remain after installation, use a /K in the commandline.

### 2.5.4.	Post-installation

1.  If you installed from CD-Rom, remove the CD and keep it in a safe place.  You will need it again if you ever need to reinstall PageGate, or want to install either the PageGate Monitor program, or the PageGate GUI Client on a workstation.

2.  If you installed NotePager Net from a single downloaded file (pgv4.exe or pgv4.zip), be sure to make a copy of the file and store it somewhere safe. You will need it again if you ever need to reinstall PageGate, or want to install the PageGate Administration program, the PageGate Monitor program, or the PageGate GUI Client on a workstation.

### 2.5.5.	Installation troubleshooting

Problem:  Any number of different problems

Solution:  Be sure that there are no other programs running when you install PageGate.  There may be programs running even if you don't see them in your task bar.  Pressing <ctrl><alt><del> will bring up a list of running programs in Windows 95 and 98.  In Windows NT you can choose 'Task Manager' after hitting <ctrl><alt><del> for similar information.  Stop any program that you believe is causing the conflict.  WARNING: this should only be done by an experienced user.  WARNING: Randomly ending tasks can lock up your computer, lose data or worse.  Please consult with and experienced technician before killing any processes.  A clean reboot can sometimes also help.  Holding down the shift key while windows is starting will stop some programs from starting that automatically load at startup.  Try re-running the PageGate setup program.

# 3.	Running the first time

## 3.1.	One-time configuration

When PageGate is run for the first time, you will be required to enter some initial setup information.  This information can be modified later by using PageGate's Administration program (PG Admin).  To start PageGate, choose 'PageGate Server' from your Windows start menu.  The default location would be 'Start' > 'Programs' > 'PageGate' > 'PageGate Server'.  The first window you will see is the 'PageGate Setup' window.  Here you need to make some choices about where to keep some of PageGate's files, and which of PageGate's modules you will be using on this computer.

Here are the settings:

Program directory - This tells PageGate where its program files reside on the local computer.  In most cases this would be where PageGate was installed (default: C:\Program Files\PageGate\).

Database Directory - This is where PageGate will keep its database.  If you are upgrading from a previous version of PageGate, be sure to enter the same database location as the prior version.  The database will be automatically upgraded.  If you plan to use the PageGate GUI Client, or to run some of PageGate's modules on another computer, or to administer PageGate from another computer, you should put PageGate's database in a shared network folder.  IMPORTANT: If you specify a shared network folder, the other PageGate modules/programs MUST HAVE FULL RIGHTS TO THIS FOLDER, and all folders below it in the directory structure.

Log Directory - The log directory holds all of PageGate's log files.  These logs may be used often by the administrator of PageGate.  If the PageGate Administration program is to be run from another workstation, the logs should be kept in a shared network folder that both the PageGate server and the administrator's workstation have full access to.

Run on this server - Here you can choose which of PageGate's modules to run on this computer (please  refer to 'Advanced Installation' for information on running PageGate across multiple computers).  The scheduler module and at least one dialer must be run somewhere in order for PageGate to operate.  The GetAscii, GetMail, and GetWeb modules are optional, but must be setup on a computer to receive messages via their respective interfaces.

After making your selections, apply your changes.

If you are upgrading from a previous version of PageGate, you will be prompted for a new registration key.  You will need to obtain a new key  from NotePage, Inc. for this version if you are upgrading from PageGate version 2.x (please refer to the section labeled 'Purchasing').

If a new PageGate database is created, you may be prompted to restart PageGate at this point.

## 3.2.	Starting & Stopping PageGate

PageGate can be started by choosing 'PageGate Server' from the Windows start menu.  Typically it can be found at 'Start' > 'Programs' > 'PageGate' > 'PageGate Server'.

After PageGate is started, a pager icon will appear in the Windows system tray on the bottom right-hand corner of the screen (next to the clock).  This icon is used to stop PageGate, start the PG Admin program, or start the PG Monitor program.  Clicking on the pager with the left mouse button will display a menu with these options.

If PageGate is running as an application, or as a Windows 9x service, the first menu option, 'Restore', will simply remove the pager icon from the system tray, and open a window on your desktop.  Use the minimize button to return the icon to the system tray and remove the PageGate window.  Quitting the PageGate window by using the 'X' control or by using the 'Quit PageGate' button will stop the entire PageGate Server application.

If PageGate is running as a Windows NT system service, the first menu option will be 'Start' instead of 'Restore'.  This option will start the PageGate system service if it is not already running.

The second menu item is 'PG Admin'.  Choosing this item will start PageGate's Administration program (if it is not already running).

The third menu item is 'PG Monitor'.  Choosing this item will start PageGate's real-time monitoring program.

If PageGate is running as an application or as a Windows 9x service, the last option is 'Quit'.  Selecting 'Quit' will stop the entire PageGate application.  If PG Admin or PG Monitor are running, they will need to be exited individually.

If PageGate is running as a Windows NT system service, the last option is 'Stop'.  This will stop the PageGate Server service from running.  If the PG Admin or PG Monitor programs are running, they will need to be quit out of individually.

If PageGate has been setup to run as a Windows 9x or Windows NT service, it will start automatically each time the computer is restarted.  Otherwise, you must start PageGate manually from the Windows start menu. (or add it to the computer's startup group).

## 3.3.	Where to find Help

There are several places to find help on the operation of PageGate.

This help file is probably the best resource for most questions. It is accessible from the 'PageGate' group in your Windows Start Menu, or by clicking on the 'Help' object in the PG Admin program.

PageGate also implements an extensive context sensitive help system.  In the bottom right corner of each entry screen in the PG Admin program, you will find a question mark (?).  When you click on it, your mouse pointer will change to a question mark too.  Next click on any entry field (or label) to receive help information about that specific field.

All of PageGate's manuals are also available on NotePage's websites in HTML format. (found under the 'Support' section):

www.notepage.net

www.notepager.net

www.notepager.com

All of PageGate's manuals are also available to download in .doc format (readable by WordPad, MS Word, etc.), from the 'Support' section of our website:

www.notepage.net

www.notepager.net

www.notepager.com

## 3.4.	5 steps to sending your first message

In order to configure PageGate, you first must have some information about your computer and paging device you want to send messages to:

1.  The PIN/ID number of the pager

2.  The phone number for your paging service provider's paging terminal

3.  The communication settings needed to connect to your paging service provider's terminal

4.  An init string for your modem to help your modem to communicate with the paging carrier's modem (usually necessary for 9600 baud and faster modems)

5.  The Communications port your modem is using on your computer

Notes regarding the above items:

The PIN/ID number is, in some cases, the same number used to send a voice or touch-tone message to the pager (sometimes this number includes the area code numbers too).  In other cases, the PIN/ID number is a totally different number.  To save time, it is best to verify your PIN/ID number with your paging service provider before configuring PageGate.

The paging terminal's phone number, in almost all cases, is NOT the same phone number you would use to send a voice or touch-tone message to the pager.  You must get this number from your paging service provider.  Be persistent.  The first representative you talk to may not have this information readily available.

The communications settings are the baud rate, parity, data bits, and stop bits used by your paging service provider's modems.  In most cases, these are the settings used: 1200 baud, even parity, 7 data bits, and 1 stop bit.  You should verify these settings with your paging service provider.

The init string for your modem can sometimes be a little tricky to obtain.  Most modems that communicate at 9600 baud or faster need an init string to reliably communicate with paging terminals.  Some modems don't require an init string at all.  You can find init strings that work with most modems on our web page (www.notepage.net) and later in this manual.  Because of the lack of command string standards within the modem community, you may need to do some research to find a workable init string.

The Communications port for your modem is the number for the serial port your modem uses.  Ports 1-4 are most commonly used.  Make sure your modem is operational before trying to use it with PageGate.  PageGate requires EXCLUSIVE use of your modem.  No other applications (such as internet dialers or fax programs) can share the modem with PageGate.

Steps:

1.  Start PageGate and PG Admin, if they are not already running.

2.  In Admin, choose 'Connectors' > 'Dialer1' > 'Settings'.  Enter an init string and dialer prefix if necessary.  Enter the correct 'Port' for your modem.  Apply your changes.

3.  Right-click on 'Carriers' and choose 'Add'.  For your paging carrier, enter its name, protocol, phone number, baud rate, parity, data bits, and stop bits.  Apply your changes.

4.  Right-click on 'Recipients' and choose 'Add'.  For the recipient (paging device), enter the recipient name, full name, PIN/ID, and carrier.  Apply your changes.

5.  Right-click on the new recipient you just created (under the 'Recipient' object).  Choose 'Send Message' from the menu.  Enter who the message is from and then a short test message, and then hit the 'Send' button.

If all your settings are correct, shortly you should receive the message on your paging device.  If not, recheck your settings and refer to the section 'Troubleshooting connecting to a carrier'.

## 3.5.	Monitoring PageGate

While setting up PageGate and experimenting with its capabilities, PageGate's behavior can be most easily monitored by using the PG Monitor program.  This program is installed along with the PageGate server application. It can be started from the PageGate group in the Windows Start Menu, or by clicking on the pager in you Windows system tray (down by the clock), if the PageGate server is already running.  Whenever the PageGate server is running, it will send status and logging information to the PG Monitor program.  For more information about the operation of PG Monitor, see the section under the 'Modules' chapter in this manual labeled 'PG Monitor'.

# 4.	Modules

## 4.1.	Overview

The PageGate application consists of several modules, each having its own unique responsibilities.  Some modules (the interfaces in particular) are optional and need only be run if the user desires.  The following section describes each of the modules and their function.

## 4.2.	PG Admin

### 4.2.1.	Overview

The PageGate Administration program (PG Admin) is used for setting, configuring, and controlling all aspects of the PageGate application.  PG Admin can be run from the paging server computer, or it can be setup and run on an administrator's workstation.  PG Admin takes an object-oriented approach to administering the server, most of its functions and settings can be accessed by manipulating the objects in its object-tree.  Many of the functions of PG Admin are only accessible by right-clicking on these objects.

### 4.2.2.	Starting & Stopping

There are two ways to start the PG Admin program.  If the PageGate Server is running, there should be a pager icon in the Windows system tray (bottom right corner of the screen, next to the clock).  Click on the pager icon and you will see a popup menu.  Choose 'PG Admin' from the menu.  PG Admin can also be started from the Windows start menu.  Choose 'Start' > 'Programs' > 'PageGate' > 'PageGate Admin'.  PG Admin will start.

To exit the PG Admin program, either click on the 'X' control on the PG Admin window, or right-click on the 'PageGate' object and choose 'Exit' from the popup menu.

### 4.2.3.	Using

PageGate's Administration program uses an object-oriented approach to administering PageGate.  The PG Admin program's screen is broken into two halves.  The left half is a tree-view of PageGate's objects.  The right half gives details about the currently selected object.  Much of PG Admin's functionality can be accessed by using the right mouse button and clicking on one of PageGate's objects.  For example, to send a message to a single recipient (assuming the recipient has already been setup), simply right-click on the recipient in the object-tree and choose 'Send Message' from the popup menu.  Similarly, a message can be sent to all recipients or a group of recipients by right-clicking on the recipient's or group's object.

### 4.2.4.	Objects

#### 4.2.4.a.	PageGate

Description - The PageGate object is the container for all of the objects belonging to the PageGate application.  To display the objects beneath the PageGate object, click on the '+' to the left of it, or double-click on the word 'PageGate'.

Functions (accessible by right-clicking on the object)

Exit - This exits the PG Admin program.  This will NOT stop the rest of the paging server, only the administration program.

Settings

(None)

#### 4.2.4.b.	Program

Description - The program object is found beneath the PageGate object.  The Program object contains settings and information that relate to the operation of the entire PageGate application.

Functions (accessible by right-clicking on the object)

Exit - This exits the PG Admin program.  This will NOT stop the rest of the paging server, only the administration program.

Settings

Program directory - This is the full path to the location of PageGate's program files.

Database directory - This is the full path to the location where PageGate's database is kept (pagegate.mdb).  This should only be a local path (i.e. C:\Program Files\PageGate\) if the entire PageGate application is to be run on a single computer, including the PG Admin program.  Otherwise, this path should be a UNC (Universal Naming Convention) path pointing to a shared network folder (i.e. \\PagingServer\PagingDatabase\).

Log directory - This is the full path to the location where PageGate's log files are kept. This should only be a local path (i.e. C:\Program Files\PageGate\) if the entire PageGate application is to be run on a single computer, including the PG Admin program.  Otherwise, this path should be a UNC pointing to a shared network folder (i.e. \\PagingServer\PagingLogs\).

Run PageGate as - This sets the style in which the PageGate application is run.  On a Windows 95 or 98 computer, PageGate can run as an application or as a Windows 9x service.  On a Windows NT, 2000, or XP computer, PageGate can run as an application or as a Windows NT service.  See the section 'PageGate Run Styles' for more information.

Run on this server - PageGate is made up of several modules, which can be run on separate computers.  This section allows you to choose which modules are to be run on this computer.  Please see the section 'Modules' for more information on each of PageGate's modules, or 'Advanced Installation' for information on installing PageGate across multiple computers.

Global Log - If logging is enabled, this log shows a summary of the events generated from the PageGate application, including all messages sent.

Refresh - displays the most current entries in the global log

Print - sends a copy of the log to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

Password Protect - Four passwords can be set here for the PageGate application.

Master Password - when set gives access to everything in the PG Admin program, and access to the setup screen in the GUI Client program.

Admin Password - when set gives access to recipient and group settings, status and reports in the PG Admin program, and access to the setup screen in the GUI client program.

Report Password - when set gives access only to status screens and reports in the PG Admin program.

Client Password - when set is required to start the GUI Client program.

#### 4.2.4.c.	Database

Description - The database object is found beneath the PageGate object.  It contains settings, status information, and tools that relate to the PageGate database.

Settings

Purge After - This sets the number of days that delivered messages are kept in the PageGate database before they are deleted.  This setting also controls when old input files processed by the GetAscii interface are removed.

Recipient Extra Field Labels - Four extra pieces of information can be stored with each recipient and group.  This is where you can set the labels that are shown with each piece of information.

Status - Shows statistics of the PageGate database

Tools - Procedures to import data, export data and rebuild the PageGate database

Import - these objects allow you to import information into the PageGate database from comma delimited files

Settings - general settings for the behavior of PageGate

Connectors - settings relating to the PageGate dialers

Carriers - settings for the carriers used by PageGate

Recipients & Groups - settings for recipients and groups

Group Members - members that belong to each group

Messages - messages currently in the database

Advanced Ascii - settings for advanced polling used by the GetAscii module

On-Call - Schedules to send messages to on-call groups

Ad-Hoc Mail - Ranges of PIN numbers used with the AD-Hoc mail option

Mailboxes - Information about the mail settings for each recipient or group

Lookup Table - Lookup table settings used with the GetAscii and GetSerial interfaces

Export - these objects allow you to export information from the PageGate database to comma delimited text files.

Settings - general settings for the behavior of PageGate

Connectors - settings relating to the PageGate dialers

Carriers - settings for the carriers used by PageGate

Recipients & Groups - settings for recipients and groups

Group Members - members that belong to each group

Messages - messages currently in the database

Advanced Ascii - settings for advanced polling used by the GetAscii module

On-Call - Schedules to send messages to on-call groups

Ad-Hoc Mail - Ranges of PIN numbers used with the AD-Hoc mail option

Mailboxes - Information about the mail settings for each recipient or group

Lookup Table - Lookup table settings used with the GetAscii and GetSerial interfaces

Rebuild database - This option will compact and rebuild the PageGate database.  All of PageGate's modules must be stopped in order to rebuild the database (this can be accomplished by clicking on the pager icon in the Windows system tray (next to the clock) and choosing 'Quit' or 'Stop' from the popup menu).  All PageGate GUI Clients must also be exited.  Please note that an external rebuild utility is also included with PageGate.  It is called 'Jetcomp.exe' and can be found in the PageGate program folder.  This utility can be used in cases where the PG Admin program is unable to start due to certain types of database corruption.  After the database is rebuilt, you will have to restart the PageGate Server module.  If PageGate is running as an application or Windows 9x service, restart PageGate by choosing 'PageGate Server' from the Windows start menu.  If PageGate is running as a Windows NT service, click on the pager icon in the Windows system tray (next to the clock) and choose 'Start' from the popup menu.

Password Protect - Entering a database password will restrict access to the database from programs like Microsoft Access.  Without the password, the database cannot be opened.  If this password is set,  the next time each PageGate GUI Client is started, the you will need to enter it (you will be prompted for it automatically).  The PageGate GUI Client will only prompt for this password once, and then it will be stored for future program startups (unless the password is changed again).  DO NOT LOSE THIS PASSWORD.  If this password is lost, the database may become permanently inaccessible!

#### 4.2.4.d.	Scheduler

Description - This object contains settings and status information that relates to the scheduler module.

Functions (accessible by right-clicking on the object)

Start - this will start the scheduler module (if the PageGate application is currently running).

Stop - this will stop the scheduler module (if the PageGate application is currently running).

Settings - the only settings for the scheduler is 'Polling Interval'.  This sets how often (in seconds) the scheduler checks the database for new unscheduled messages.  The scheduler also uses this time interval to check if any of the dialers need a new carrier to connect to.

Status - shows the current state of the scheduler

Log - If logging is enabled, this log shows the activity of the scheduler module. This includes all messages processed by PageGate.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

#### 4.2.4.e.	Interfaces

##### 4.2.4.e.1.	Interfaces

Description - Interfaces are the PageGate modules that accept messages into the paging server.

Functions (accessible by right-clicking on the object)

Start All - this will start all the installed interfaces (if the PageGate application is currently running).

Stop All - this will stop all the installed interfaces (if the PageGate application is currently running).

##### 4.2.4.e.2.	Global

Description - This object contains information that relates to all the PageGate server interfaces.

Functions (accessible by right-clicking on the object)

Start All - this will start all the installed interfaces (if the PageGate application is currently running).

Stop All - this will stop all the installed interfaces (if the PageGate application is currently running).

Status - this displays the current status of all of the PageGate interfaces.

##### 4.2.4.e.3.	GetAscii

Description - This object contains settings and status information for the GetAscii interface.

Functions (accessible by right-clicking on the object)

Start - this will start the GetAscii interface (if the PageGate application is currently running).

Stop - this will stop the GetAscii interface (if the PageGate application is currently running).

Settings

Enabled - check this  box to enable the GetAscii interface.  The interface cannot be enabled unless a valid polling directory is provided

Polling directory - the polling directory is the location where the GetAscii interface looks for text files with paging information in them.  If the sendpage.exe program is being used, this is where it should create its temporary data files.

Polling Interval - the polling interval sets how often (in seconds) the GetAscii interface checks its polling directory(s) for new messages

Advanced Polling Options - advanced polling options can be used when you don't have control over the format of the Ascii text files to be accepted by the GetAscii interface.  It uses file locations and filenames to specify which recipient(s) the messages are for.  Using Advanced Polling Options, PageGate can also detect changes in files, and additions to files.  It can extract message data from the files. See the 'GetAscii (commandline/ascii)' section of this manual for more information about 'Advanced Polling Options'.

Files - set locations for PageGate to watch for files

Symbols - set special characters for searching files for message information

Record - settings to define a record of message data within the files

Recipient - settings to extract recipient information from files

Sender - settings to extract sender information from files

Message - settings to extract message information from files

Status - this displays the current state of the GetAscii Interface

Log - If logging is enabled, this log shows the activities of the GetAscii interface, including all messages accepted and rejected by it.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

##### 4.2.4.e.4.	GetMail

Description - This object contains settings and status information for the GetMail interface.  In addition to receiving messages via email, this module is also responsible forwarding copies of messages and sending outcome messages via email.

Functions (accessible by right-clicking on the object)

Start - this will start the GetMail interface and the mail server service (if the PageGate application is currently running).

Stop - this will stop the GetMail interface and the mail server service (if the PageGate application is currently running).

Settings

Enabled - check this  box to enable the GetMail interface.

Local Domain - This is the domain name to be used by PageGate when sending and receiving email.  This setting should be an internet domain, not a Microsoft networking domain.  Only email with an address in this domain will be accepted by PageGate.  When sending email, this is the domain that PageGate will use when talking to other mail servers.  This setting must be a valid domain name for to work properly (Note: an IP address can be used here instead of a domain name, if email is addressed to match.  This is commonly done during testing when a domain name has not yet been assigned to this machine).

Postmaster - the Postmaster setting is an email address to where all warnings, alerts, bad and undeliverable messages are sent.  It is very useful to set this field to an email address that can be collected by PageGate's administrator.  The email address set here can either be a mailbox maintained by PageGate, or a mailbox on another mail server.  Leaving this field blank will drop all messages that would normally go to the postmaster.

Relay All Mail - check this box to send all of PageGate's outbound email to a single mail server (typically your ISP's mail server).  This mail server (relay server) will then deliver PageGate's mail to its final destination.  This option is commonly used with slow internet connections.

Relay Server - this is the server to send all of PageGate's outbound email to if the 'Relay All Mail' option is checked.  Otherwise, this server will be used only if PageGate is unable to deliver mail directly to its destination.

DNS Server 1 & 2 - PageGate uses these DNS servers to lookup the mail server that hosts the mailboxes that outbound email is addressed to.

Email Outcome - when this box is checked, an email message will be sent back to the message's sender, with the outcome of the delivery attempt to its carrier.

POP Interval - If PageGate is setup to use 'Domain POP Mail Collection' or if any of PageGate's recipients are setup to collect mail from remote POP mailboxes, this is the interval at which collection takes place.

Ad-Hoc - PageGate can be setup to accept email addressed to the ID/PIN number of the recipient (i.e. 1347652@mypagers.com)

Domain POP - PageGate can be setup to collect all the mail for an entire domain from a single POP mailbox.

DUN- PageGate can use Dial-Up Networking to connect to the internet for mail collection, if needed.

Status - this displays the current state of the GetMail Interface

Log - If logging is enabled, this log shows the activities of the GetMail interface, including all messages accepted and rejected by it.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

##### 4.2.4.e.5.	GetSerial

Description - This object contains settings and status information for the GetSerial interface.

Functions (accessible by right-clicking on the object)

Start - this will start the GetSerial interface (if the PageGate application is currently running).

Stop - this will stop the GetSerial interface (if the PageGate application is currently running).

Settings

Enabled - check this  box to enable the GetSerial interface.

Serial Port - choose the serial port (COM port) you want the GetSerial interface to monitor for incoming text.

Baud Rate - set the baud rate to match the speed that the incoming data will be sent at.

Parity - set the parity to match the parity that the incoming data will be sent using.

Data Bits - set the data bits to match the number of data bits that the incoming data will be sent with.

Stop Bits - set the stop bits to match the number of stop bits that the incoming data will be sent with.

Handshaking - the serial connection may use handshaking to control high-speed data.  If the sending device uses handshaking, you should match the handshaking method to match.

Init String - with most direct serial connections the init string setting will be left blank.  It is used mostly if a modem is connected to the port.

Symbols - special characters used for processing incoming data.

Record - settings to define a record of message data from incoming data.

Recipient - settings to extract recipient information from incoming data.

Sender - settings to extract sender information from incoming data.

Message - settings to extract message information from incoming data.

Status - this displays the current state of the GetSerial Interface

Log - If logging is enabled, this log shows the activities of the GetSerial interface, including all messages accepted and rejected by it.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

##### 4.2.4.e.6.	GetTap

Description - This object contains settings and status information for the GetTap interface.

Functions (accessible by right-clicking on the object)

Start - this will start the GetTap interface (if the PageGate application is currently running).

Stop - this will stop the GetTap interface (if the PageGate application is currently running).

Settings

Enabled - check this  box to enable the GetTap interface.

Serial Port - choose the serial port (COM port) you want the GetTap interface to use to accept incoming TAP message sessions.

Baud Rate - set the baud rate to match the speed that the incoming data will be sent at.  The default speed is typically 1200.

Parity - set the parity to match the parity that the incoming data will be sent using.  The default parity is typically Even.

Data Bits - set the data bits to match the number of data bits that the incoming data will be sent with.  The default data bits is usually 7.

Stop Bits - set the stop bits to match the number of stop bits that the incoming data will be sent with.  The default stop bits is usually 1

Init String - the init string is a set of commands sent to the modem to configure it for incoming calls and to turn on or turn of features of the modem.  For example, most systems using the TAP protocol communicate at 1200 baud with not error correction or data compression.

Status - this displays the current state of the GetTap Interface

Log - If logging is enabled, this log shows the activities of the GetTap interface, including all messages accepted and rejected by it.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

##### 4.2.4.e.7.	GetWeb

Description - This object contains settings and status information for the GetWeb interface.

Settings

Enabled - check this box to enable the GetWeb interface.  The interface cannot be enabled unless valid paths are provided for the 'Web Pages Path' and the 'CGI Data Path'

Web Pages Path - sets the location where, on the web server, PageGate will automatically generate web pages for recipients (if desired).

CGI Data Path - sets the location where the GetWeb interface collects the data files generated by the webgate.exe CGI program.

Polling Interval - sets how often (in seconds) the GetWeb interface checks for new messages from the web.

Status - this displays the current state of the GetWeb Interface

Log - If logging is enabled, this log shows the activities of the GetWeb interface, including all messages accepted and rejected by it.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

##### 4.2.4.e.8.	GUI Client

Description - This object contains settings and status information for the GUI (Graphical User Interface) interface.

Settings

Enabled - check this box to enable the GUI interface.

Status - this displays the current state of the GUI Interface

Log - If logging is enabled, this log shows the activities of the GUI interface, including all messages accepted and rejected by it.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

#### 4.2.4.f.	Connectors

##### 4.2.4.f.1.	Connectors

Description - Connectors are the PageGate modules that actually send out messages to the paging carriers.  Connectors can use a modem or a direct cable connection.

Functions (accessible by right-clicking on the object)

Start All - this will start all the installed dialers (if the PageGate application is currently running).

Stop All - this will stop all the installed dialers (if the PageGate application is currently running).

##### 4.2.4.f.2.	Global

Description - This object contains settings and status information that relates to all of PageGate's connectors.

Functions (accessible by right-clicking on the object)

Start All - this will start all the installed dialers, if the PageGate application is currently running.

Stop All - this will stop all the installed dialers, if the PageGate application is currently running.

Settings

Polling Interval - sets how often (in seconds) the dialers check their queues for messages waiting to be sent out.

Dial Threshold - sets the number of messages that can be pending for a single carrier, before an additional dialer is started to assist in delivery.  More than one dialer must be purchased for this setting to have any effect.

Retries - sets the number of times the same message will be retried before the message is marked as bad.  It also sets the number of times a carrier will be retried before all pending messages for it will be marked as bad.

Retry interval - sets how long (in seconds) a failed delivery attempt will wait until it is tried again.

SMTP - settings used by the dialers when sending out SMTP (email) messages.

Local Domain - This is the domain name to be used by PageGate when sending SMTP (email) messages.  This setting should be an internet domain, not a Microsoft networking domain.  When sending email, this is the domain that PageGate will use when talking to other mail servers.  This setting must be a valid domain name for SMTP delivery to work properly.

Postmaster - the Postmaster setting is an email address to where all warnings, alerts, bad and undeliverable messages are sent.  It is very useful to set this field to an email address that can be collected by PageGate's administrator.  The email address set here can either be a mailbox maintained by PageGate, or a mailbox on another mail server.  Leaving this field blank will drop all messages that would normally go to the postmaster.

Relay All Mail - check this box to send all of PageGate's outbound email to a single mail server (typically your ISP's mail server).  This mail server (relay server) will then deliver PageGate's mail to its final destination.  This option is commonly used with slow internet connections.

Relay Server - this is the server to spool all of PageGate's outbound email to if the 'Relay All Mail' option is checked.  Otherwise, this server will be used only if PageGate is unable to deliver mail directly to its destination.

DNS Server 1 & 2 - If you are not using a relay server, then PageGate must deliver email directly to its destination.  PageGate uses these DNS servers to lookup the mail server that hosts the mailboxes that outbound email is addressed to.

Status - shows the state of all the connectors

##### 4.2.4.f.3.	Dialers

Description - These objects contain information for each of PageGate's dialers.  PageGate comes standard with one dialer.  Additional dialers can be purchased separately.

Functions (accessible by right-clicking on the object)

Start - starts an individual dialer (if the PageGate application is currently running).

Stop - stops an individual dialer (if the PageGate application is currently running).

Settings

Dialer - the name of the currently selected dialer

Init String - a modem initialization string needed by this particular dialer.  Init strings needed to connect to a particular carrier can be set under the carrier object.  This setting can be used to help support the use of different model modems on a single PageGate server.

Dialing Prefix - a touch-tone dialing prefix needed to make outside calls for this particular dialer.

Dedicated Carrier - sets as single carrier to be serviced by this dialer.  This is mostly used with a direct cable connection.  When left at (None), the dialer will be able to send messages to any carrier that has pending messages.

Serial Port - the serial or com port used by this dialer.  This is usually the com port for a modem.  A settings of 0 will dedicate the dialer to only use an internet connection for delivery.  Any other number will allow the dialer to send message out the serial port (usually via modem) and out over the internet.

Direct Connect - sets the dialer to use a direct cable connection to a paging terminal (no modem).

Status - shows the state of this dialer

Log - If logging is enabled, this log shows the activities of this dialer, including all messages delivered by it.

Refresh - displays the most current entries in the log

Print - sends a copy of the log file to a printer

Purge - deletes the log's entire contents

Clipboard - copies the log's contents to the Windows clipboard, allowing it to be pasted into another application

#### 4.2.4.g.	Carriers

Description - Carrier objects contain the information needed to connect to paging terminals.  If you will be connecting to outside paging service providers, you may need to contact them to obtain some of the necessary settings.

Functions (accessible by right-clicking on the object)

Add - this allows you to add a new carrier

Cancel Messages - this will cancel all pending messages for all carriers

Carrier(s)

Description - Under the main carrier object you will see carrier objects for each new carrier you have setup.

Functions (accessible by right-clicking on the object)

Delete - this will delete this carrier.  Note: carriers cannot be deleted if there are recipients currently setup to use it.

Cancel Messages - this will cancel all pending messages for this carrier

Settings

Carrier - a name that identifies the carrier.  This name cannot be changed later.

Protocol - PageGate supports seven different protocols for  delivering messages to paging carriers: GSM, SMTP (email), SNPP, TAP, Tone, UCP, and WCTP.  GSM is used when PageGate is connected directly to a wireless modem or mobile phone.  SMTP is used to send messages to carriers over the internet using this email protocol.  SNPP and WCTP are paging protocols designed to use the internet for delivery.  TAP and UCP are used to send alphanumeric and numeric messages to a carrier by connecting to a modem at the paging carrier.  Tone is used to send numeric messages to a carrier by sending touch tones (some carriers don't support TAP with their numeric pagers).  Some of the fields described below don't apply to some of the delivery methods and may not show.

Failover - if a message fails delivery to its original recipient, a failover recipient can be set that will then receive the message(s).

Phone Number - the phone number to connect to the carrier's paging terminal, or if a common phone number is used for touch tone delivery

Host - the host name of the machine at the carrier that will accept the messages over the internet.

Port - the TCP/IP port number that the host at the carrier is accepting messages on (usually 444 for SNPP and 80 for WCTP)

Baud Rate - the baud rate to use when connecting to the paging terminal.  1200 baud is the most common setting

Parity - the type of parity to use when connecting to the paging terminal (e = even, o = odd, n = none). Even parity is the most common setting

Stop Bits - the number of stop bits to use when connecting to the paging terminal.  1 stop bit is the most common setting

Init String - an init string can be set to change modem settings before the carrier is called.  In most cases an init string is necessary to reliably connect to most carriers.  See the section 'Init Strings' for more information

Max Chars - the maximum number of characters that the paging terminal will accept in a single message.  Some carriers accept as little as 80 characters.  Others accept over 1000.  240 is a common limit.

Password - some carriers require a password to accept messages.  Most don't require a password.

Msg Limit - this settings forces PageGate to disconnect from the carrier after sending this number of messages.  This is helpful with some carriers that limit the number of messages they can receive in a single connection, but don't do this correctly.  A setting of 0 (zero) means no limit and should be used in most cases.

Drop not-printable characters - many pagers don't accept formatting characters like carriage returns, line feeds, and tabs.  Check this box if you don't want these 'not printable characters' sent out in the message.

Pulse Dial - If touch-tone dialing is not available in your area, check this box to dial using pulse.

Wait String - this settings is used in conjunction with the Tone delivery method.  See the section 'Sending Touch Tone Messages to Numeric Pagers' for more detailed information.

#### 4.2.4.h.	Recipients

Description - The main recipient object contains objects beneath it for each pager/phone/wireless messaging device setup in PageGate.

Functions (accessible by right-clicking on the object)

Add - this allows you to add a new recipient

Send Message - this will send a single message to all recipients

Requeue Messages - this allows you to re-send messages that have already been sent (to any/all recipients)

Cancel Messages - this will cancel all pending messages for all recipients

Pending Messages - this will show all pending messages for all recipients.  It also allows you to cancel single pending messages.

Recipient(s)

Description - Under the main recipient object, you will see a recipient object for each new recipient that is setup (recipients were called 'Users' in previous versions of PageGate).

Functions (accessible by right-clicking on the object)

Send Message - this will send a message to the selected recipient

Requeue Messages - this allows you to re-send messages that have already been sent to this recipient

Cancel Messages - this will cancel all pending messages for the selected recipient

Pending Messages - this will show all pending messages for the selected recipient.  It also allows you to cancel a single pending message for the recipient

Delete - this will delete the selected recipient.

Settings

Recipient - a short unique name to identify the recipient (no spaces, or symbols with the exception of '-' and '_')

Full Name - the recipients full name or long description (spaces and most symbols can be used)

Enabled Services - check the interfaces that will be allowed as a source of messages for this recipient.

Recipient Type - PageGate supports four different recipient types: Normal, Multi-Page, Ad-Hoc, and Email.  Normal recipients are used most of the time.  They represent a single pager or wireless messaging device.  Multi-Page recipients can be created to accept multiple recipients within a single message (see the Multi-Page section under each of the interfaces for more detailed information).  Ad-Hoc recipients can be created to accept ID/PIN numbers in messages, and use them for delivery to carriers (see the Ad-Hoc section under each of the interfaces for more detailed information.  Please note that the Ad-Hoc feature is only available with licenses of 5000 or greater).  Email recipients are used to setup mailboxes for email use only.  There is no paging capabilities connected to these accounts.  PageGate can be used as a standard mail server for company email by setting up recipients of the 'Email' type.  Depending on the type of recipient chosen, some of the following fields may not be shown.

Carrier - a list of all the available carriers.  Choose the carrier that this recipient uses.

Failover - if a message can not be delivered to this recipient, it is sent the failover recipient (or group)

ID/PIN - the ID or PIN number for the pager, phone, or wireless device

Email To - the email address of the pager, phone, or wireless device

Phone Num - the phone number of the pager

Max Chars - The number of characters PageGate will send to this recipient from a message.  If this number is greater than the Max Characters setting for the carrier, the message will be broken up into multiple messages (according to the carrier's max character settings), numbered, and then sent.

Notify Code - If the 'Notify Only' setting is enabled, this message will be sent instead of the actual message text.

Notify Only - check this if the recipient should receive the 'Notify Code' (above) rather than the actual message.  This setting is commonly used with numeric pagers.

Alpha - check this box if the recipient's device (pager) accepts both letters and numbers

Reliapage - when reliapage is enabled, PageGate will sequentially number each message it sends out to the recipient.  This is useful for detecting if a message has been missed (a number is missing).  Messages then can be re-sent by using the 'Requeue' function.

Don't send 'from' - checking this box will stop PageGate from sending the sender's name (the 'from' field) at the beginning of each message.

Don't send 'from' domain - checking this box will remove the domain name portion of a sender's address, from the 'from' field of the message.  This can save characters especially when a typical message sender has a long domain name in their email address.

Drop non-printable characters - many pagers don't accept formatting characters like carriage returns, line feeds, and tabs.  Check this box if you don't want these 'not printable characters' sent out in the message.

Send Subject - if the message is being delivered via email, PageGate will send a subject line with the message.

Misc - Miscellaneous settings for the recipient

Four Extra Fields - these are user definable data fields that are stored with each recipient.  The labels for these fields can be set under the 'Database' object.  If the GUI Client interface is used, the information in these fields can be viewed by the users, and the recipient lists can be filtered by this information.

Hold Messages - the hold feature makes PageGate put messages, for a recipient or group, on hold until a later time.  This is usually used to hold messages during the night, and then send all the queued messages in the morning at a reasonable hour.  It may be useful to set up two recipients for a single pager/phone.  One which will always get messages at any time, and another for less urgent messages, that will hold messages overnight.

Groups - the groups object allows you to choose which groups (if any) this recipient belongs to

Email - if the email service is enabled for this recipient, this object allows you to setup advanced email settings, like the ability to collect received email with a regular email client program, the ability to collect messages for this recipient from a remote mail server, the ability to forward a copy of each message received by this recipient to another email address, and the ability to create email aliases for this recipient.

Mailbox Name - by default, the email address of a PageGate recipient is the same as the recipient's name.  This can be changed by entering a different mailbox name here.

Forward Copy To - if you would like a copy of every message received by this recipient, sent to an email address, enter an email address here.

Enable POP Access to Messages - messages received by a PageGate recipient can be stored in a mailbox, making them accessible from regular email clients (such as Outlook, Netscape, Eudora, etc.).  Check this box to enable this option.

POP Password - If the 'Enable POP Access to Messages' box is checked, you must provide a password here to protect the mailbox from unauthorized access.

POP Mail Collection - PageGate can collect messages for each recipient from one or more mailboxes hosted elsewhere.  Click on this button to display the setup screen for this feature.

Email Aliases - each PageGate recipient can accept messages from more than one local email address.  These additional names are called 'aliases'.  Click on this button to display the setup screen for this option.

WebPage - If the web service is enabled for this recipient, this object allows you to choose a template, and to enter information for the automatic generation of a personal webpage for this recipient.  See the 'GetWeb' section for more details.

#### 4.2.4.i.	Groups

Description - The main groups object contains objects for each group setup in PageGate.  Groups allow a single message to be sent to several recipients.

Functions (accessible by right-clicking on the object)

Add - this allows you to add a new group to PageGate

Send Messages - this will send a message to all groups

Requeue Messages - this allows you to resend message that have already been sent to any group

Cancel Messages - this will cancel all pending messages for all groups

Group(s)

Description - Under the main group object, you will see a group object for each group that has been setup

Functions (accessible by right-clicking on the object)

Send Messages - this will send a message to all the members of this group

Requeue Messages - this allows you to resend messages that have already been sent to this group

Cancel Messages - this allows you to cancel all pending messages for this group

Delete - this will delete the selected group.  Note: all of PageGate's modules are temporarily paused and then restarted when a group is deleted, so as not to remove a group that is currently being processed.  When removing a large number of groups, it is most efficient to manually stop PageGate's modules, delete the groups, and then restart the modules.

Settings

Group - a short unique name to identify the group

Description - a longer description of the group

Enabled Services - check the interfaces that will be allowed as a source of messages for this group.

Advanced Options

On-Call Group - the on-call group feature allows for users to send a message to a group, and only the recipient(s) that is currently on-call is sent the message.  See 'On-Call Groups' for more information

On-Call Schedule - this button allows you to set a schedule for which recipients in this group are on-call and when.  See 'On-Call Groups' for more information.

Misc - Miscellaneous settings for the group

Four Extra Fields - these are user definable data fields that are stored with each group.  The labels for these fields can be set under the 'Database' object.  If the GUI Client interface is used, the information in these fields can be viewed by the users, and the recipient lists can be filtered by this information.

Hold Messages - the hold feature makes PageGate put messages, for a recipient or group, on hold until a later time.  This is usually used to hold messages during the night, and then send all the queued messages in the morning at a reasonable hour.  It may be useful to set up two recipients for a single pager/phone.  One which will always get messages at any time, and another for less urgent messages, that will hold messages overnight.

Members - the members object allows you to select which recipients belong to this group.  Clicking on a recipient moves it from one list to the other

Email - if the email service is enabled for this group, this object allows you to setup advanced email settings, like the ability to collect received email with a regular email client program, the ability to collect messages for this group from a remote mail server, the ability to forward a copy of each message received by this group to another email address, and the ability to create email aliases for this group.

WebPage - If the web service is enabled for this group, this button allows you to choose a template, and to enter information for the automatic generation of a webpage for this group.  See the 'GetWeb' section for more details.

#### 4.2.4.j.	Logging

Description - The logging object contains settings related to the status and logging functions of PageGate.  Beneath this object you can also access all the logs for each individual PageGate module.

Settings

Log to Disk - checking this box turns on PageGate's logging to disk feature.  PageGate will log to text files information about each message accepted and delivered.  Log to Disk must be enabled to see information in the log objects in the admin program.

Detailed Log - when detailed logging is selected, the history logs will contain more detailed information about what is happening in each of PageGate's modules (especially the dialers).

Archive Logs - when this option is set, PageGate will create a new folder each night named with the date.  It will copy all the day's logs into this folder, and then start new logs for the new day.

Max Log Length - each night the log files are trimmed down so that they are no longer than this setting (measured in characters - previous versions of PageGate measured this in lines)

Use PG Monitor - The PageGate Monitor program allows you to monitor PageGate real-time.  PG Monitor can be run on the local computer, or from any location accessible via TCP/IP.  For more information, refer to the section 'PageGate Monitor'

PG Monitor Port - the TCP/IP port number for PageGate to communicate with PG Monitor.

PG Monitor Host 0 - the Host0 address is always set to 127.0.0.1.  This is the localhost address, which means if PG Monitor is enabled, status information will always be sent to the PG Monitor program running on the local computer.

PG Monitor Host 1-3 - the host name or IP address of computers that will run the PG Monitor program.

Global Log - If logging is enabled, this log shows a summary of the events generated from the PageGate application, including all messages sent.

Scheduler Log - If logging is enabled, this log shows the activity of the scheduler module. This includes all messages processed by PageGate.

GetAscii Log - If logging is enabled, this log shows the activities of the GetAscii interface, including all messages accepted and rejected by it.

GetMail Log - If logging is enabled, this log shows the activities of the GetMail interface, including all messages accepted and rejected by it.

GetSerial Log - If logging is enabled, this log shows the activities of the GetTap interface, including all messages accepted and rejected by it.

GetTap Log - If logging is enabled, this log shows the activities of the GetTap interface, including all messages accepted and rejected by it.

GetWeb Log - If logging is enabled, this log shows the activities of the GetWeb interface, including all messages accepted and rejected by it.

GUI Log - If logging is enabled, this log shows the activities of the GUI interface, including all messages accepted and rejected by it.

Connectors/Dialers Log - If logging is enabled, this log shows the activities of this dialer, including all messages delivered by it.

#### 4.2.4.k.	Reports

Description - Reports about information in the PageGate database.  Reports can be previewed to the screen, printed, or written to a file.

Sent Messages

Carrier - lists all sent messages for a single or group of carriers

Connector - lists all sent messages for a single or a group of dialers

Interface - lists all sent messages that originated from a single or group of interfaces

List - list all sent messages by date

Recipient - lists all sent messages for a single or group of recipients

Sender - lists all sent messages that were sent by a particular user

Station - lists all sent message that were sent from a particular GUI Client station

Pending Messages

Carrier - lists all pending messages for a single or group of carriers

Interface - lists all pending messages that originated from a single or group of interfaces

List - list all pending messages by date

Recipient - lists all pending messages for a single or group of recipients

Sender - lists all pending messages that were sent by a particular user

Station - lists all pending message that were sent from a particular GUI Client station

Statistics

Carrier - prints statistics such as throughput, timing, and delivery results, for a single or group of carriers

Connector - prints statistics such as throughput, timing, and delivery results, for a single or a group of dialers

Interface - prints statistics such as throughput, timing, and delivery results, for messages that originated from a single or group of interfaces

Recipient - prints statistics such as throughput, timing, and delivery results, for a single or group of recipients

Sender - prints statistics such as throughput, timing, and delivery results, for messages that were sent by a particular user

Station - prints statistics such as throughput, timing, and delivery results, for message that were sent from a particular GUI Client station

Carrier List - prints the currently setup carriers and their settings.  Optionally, all the recipients that use the carrier can be printed too.

Recipient List - prints information about all the recipients in the PageGate database.

Group List - prints the groups in PageGate and the members of each group, or prints recipients and all the groups they belong to.

#### 4.2.4.l.	Registration

Description - the registration object allows you to enter registration information to unlock PageGate and turn it into a fully operational, licensed version.

IMPORTANT - when you are entering registration information, ALL of the user and company information must be typed EXACTLY as it appears on the registration you receive. This includes the contact, company, and address information.  The registration information is both case and punctuation sensitive. The registration key also must be typed exactly to be accepted.  If you get a message 'Invalid Registration Key', DOUBLE and TRIPPLE CHECK all the information you have entered.

#### 4.2.4.m.	Help

Description - this object gives access to PageGate's on-line help system.

## 4.3.	PG Monitor

### 4.3.1.	Overview

The PageGate Monitor program (PG Monitor) allows for real-time monitoring of the PageGate Server.  It can be run on the PageGate server, or on any workstation that the server is able to communicate with via TCP/IP (UDP actually).  TCP/IP must be installed on the PageGate server in order for the PG Monitor program to work.

In order to use PG Monitor on a computer other than the server, both the paging server and the monitoring workstation must have TCP/IP installed and operational.  The two systems must be able to communicate with each other via TCP/IP (this can be verified by using the Windows ping utility). The workstation that PG Monitor is to be run on must have a static IP address or host name.

### 4.3.2.	Starting & Stopping

There are two ways to start the PageGate Monitor Program (PG Monitor).  If you are on the paging server, click on the pager icon in the system tray (bottom right corner of the screen, next to the clock).  From the menu choose 'PG Monitor'.  Alternately, you can access PG Monitor form the Windows start menu (Start > Programs > PageGate > PageGate Monitor).

PG Monitor can be exited by choosing 'File' and then 'Exit' from the menu.  Also, clicking on the 'X' control on PG Monitor's main window, will close the program.  If you want the window positions for the log displays saved for the next time you start PG Monitor, check the 'Save Window Positions on Exit' setting on the 'Settings' screen found under the 'File' menu.

### 4.3.3.	Configuration

In PG Admin:

PageGate's Admin program is used to setup the IP address(s) or host name(s) for the workstation(s) that will run PG Monitor

1. Choose the 'Settings' object from under the 'Logging' object

2. Check 'Enable PG Monitor'

3. Enter the TCP/IP port number you want to use (10101 by default) into the 'PG Monitor Port' field.

4. Enter the IP address or host name of the workstation that is to run PG Monitor into one of the 'PG Monitor Host' fields.

5. Apply your changes

***Note: is some cases if a bad IP address or host name is entered, the PageGate Server's performance may suffer.

In PG Monitor

No initial settings are needed unless you are using a different TCP/IP port than the default (10101).  To change the port number, choose 'Settings' from the 'File' menu.

### 4.3.4.	Using

The PageGate Monitor Window

This window shows the state of each of PageGate's modules, and statistics about them.  Each module is listed on this window.  Clicking on the 'Show All Dialers' button will show information about all of PageGate's dialers individually.  'Hide Dialers' will only show summary information for the dialers (this is the default).  The module labels change colors to indicate the state of each module.  A red label means that the module is stopped.  A green label means that the module is running.  If a module is running, it will slowly change shades of green.  Next to the module name are two fields labeled 'Good' and 'Bad'.  These fields show how many messages have been processed by the module (since PG Monitor was started), and whether they have been successful or have failed.  After the module fields is a 'Pending' field.  This field shows how many messages have been accepted, but not yet delivered by PageGate.

In addition to the main PageGate Monitor window, separate real-time log screens can be viewed for each of PageGate's modules.  Log windows can be opened by choosing 'View' from PG Monitor's menu, and then choosing the log you wish to view.  Each log can be moved, resized or closed as desired.  Choose 'Settings' from the 'File' menu and check the 'Save Window Positions on Exit' box if you want to save the window positions and sizes for the next time PG Monitor is run.  Choosing 'Cascade' from the 'View' menu will cascade all the visible log windows so that at least part of each of them can be seen.  Choosing 'Reset Counters' from the 'Tools' menu will reset all the module's counters back to zero. Choosing 'Clear Logs' from the 'Tools' menu will clear all the log windows.

IMPORTANT - by right-clicking inside the log windows, you can toggle between the regular log view and the auto-scroll view.  When auto-scroll is on, the log will always keep the most current information on the screen.

## 4.4.	Service Manager

### 4.4.1.	Overview

The PageGate service manager (PageGate.exe) runs on all computers that run a portion of the PageGate Server application (except PG Admin, PG Monitor, or the PageGate GUI Client).  It runs in the background whenever the PageGate application is active.  The service manager can run as an application, a Windows 9x service, or a Windows NT service.  It is responsible for starting and stopping all the other server modules.  If PageGate is setup to run as a Windows NT system service, it is the only module set to start automatically (it will start the other modules).  This is the only server module that should ever have more than one instance running (only if PageGate is installed across multiple machines).

When PageGate is running as an application or Windows 9x service, the PageGate service manager's icon (small pager) will appear in the Windows system tray (bottom right corner of the screen, next to the clock). Note: the PG Admin and PG Monitor programs can easily be started by clicking on this icon.  Quitting the icon when PageGate is running as an application or Windows 9x service will stop the PageGate Server application.  If the PageGate service manager is running as a Windows NT system service, a separate application (PGSystray.exe) runs in the windows system tray to provide the icon and popup menu.  This program gives similar functionality as the pager icon when PageGate is running as an application or Windows 9x service.  However,  stopping this program has no effect on the rest of PageGate.

### 4.4.2.	Starting & Stopping

The PageGate service manager controls the state of the rest of PageGate's modules.  The entire PageGate application is started or stopped by starting or stopping the PageGate service manager.

Application

Start - If PageGate is setup to run as an application, it can be started by choosing the PageGate icon from the Windows start menu.

Stop - If PageGate is setup to run as an application, it can be stopped by clicking on the pager in the Windows system tray (bottom right corner of the screen next to the clock), and then choosing 'Quit' from the menu.

Windows 9x service

Start - If PageGate is setup to run as a Windows 9x service, the application should start automatically when the computer is started.  If you need to restart it for any reason, PageGate can be restarted the same way you would start it if it were an application (from the Windows start menu).

Stop - If PageGate is setup to run as a Windows 9x service, it can be stopped by clicking on the pager in the Windows system tray (bottom right corner of the screen next to the clock), and then choosing 'Quit' from the menu.

Windows NT service

Start - If PageGate is setup to run as a Windows NT system service, the application should start automatically when the computer is started.  If you need to restart the service for any reason, you may do so by clicking on the pager in the Windows system tray (bottom right corner of the screen next to the clock), and then choosing 'Start' from the menu.  Alternately, the PageGate service manager can be started from the 'Services' icon in the Windows control panel.

Stop - If PageGate is setup to run as a Windows NT system service, it can be stopped by clicking on the pager in the Windows system tray (bottom right corner of the screen next to the clock), and then choosing 'Stop' from the menu.  Alternately, the PageGate service manager can be stopped from the 'Services' icon in the Windows control panel.

## 4.5.	Scheduler

### 4.5.1.	Overview

PageGate's scheduler program is responsible for queuing all messages to be sent out by PageGate's dialers.  Groups, on-call groups, scheduled messages, and repeating messages are also the responsibility of the scheduler.  In addition, the scheduler runs a cleanup routine each night, which trims down log files, purges old message entries in the database, and cleans up other PageGate files.  The scheduler must be running in order for any messages to be delivered by PageGate.

### 4.5.2.	Starting & Stopping

The scheduler can be started or stopped by using the PG Admin program.  Right-click on the 'Scheduler' object and then choose 'Start' or 'Stop' from the popup menu.  The scheduler will start or start within a few seconds.

## 4.6.	Connectors

### 4.6.1.	Overview

PageGate can support up to sixteen simultaneous dialers for message delivery.  Each dialer can have a direct cable connection to a paging terminal, can use a modem to dial out to a paging terminal, or can share an internet connection for message delivery.  Modems used with PageGate must be dedicated to the purpose (i.e. they can't be shared with other applications such as fax programs or internet dialers).

Please note that adding additional dialers may be desirable for not only performance reasons, but also from a redundancy standpoint.  If more than one dialer is used with PageGate, and there is a failure on one dialer (failure with phone line, modem, cabling, serial ports, etc.), PageGate will continue to operate on the remaining dialers, ensuring that there is no outage of paging services.

### 4.6.2.	Starting & Stopping

The PageGate dialers can be started and stopped as a group or individually.  The PG Admin program is used to do either.  Right-click on the 'Connectors' object and then choose 'Start All' or 'Stop All' from the popup menu to start or stop all of the dialers at the same time. An individual dialer can be started or stopped by right-clicking on the dialer object and choosing 'Start' or 'Stop' from the popup menu.  The dialer will start or stop after a few seconds.

### 4.6.3.	Multiple Dialers

PageGate can support up to 16 simultaneous dialers.  PageGate comes with a single dialer by default.  Additional dialers can be purchased separately.  Each dialer has the option to be a direct connection, connected to a modem, or connected to the internet.  Each dialer can also be dedicated to a single carrier if desired.  The dialers are intelligent and will try to deliver messages in the most efficient manner possible.  If more than one dialer is owned, a new dialer will start for each carrier that has messages pending for it.  Once a carrier connection is made, all pending messages for that carrier will be delivered.  If any dialers are still idle, the 'Dialer Threshold' value will be checked to see if any carrier has more pending messages than this value.  If so, an additional dialer will connect to the same carrier to assist in message delivery.

Please note that adding additional dialers may be desirable for not only performance reasons, but also from a redundancy standpoint.  If more than one dialer is used with PageGate, and there is a failure on one dialer (failure with phone line, modem, cabling, serial ports, etc.), PageGate will continue to operate on the remaining dialers, ensuring that there is no outage of paging services.

If only an internet connection is to be used with a dialer, you can set the serial port to 0.  If any other port number is used, the dialer can deliver messages both out the serial port (usually via modem), and out over the internet.

### 4.6.4.	Direct connect

PageGate's dialers support direct cable connection to paging terminals.  If the paging terminal is in-house, this is the most efficient method for connection.  Direct connect dialers and dialers connected to modems can be used together, enabling PageGate to send some messages to an in-house paging terminal and others out to a public carrier.  To make a dialer direct connect: in PG Admin, choose the settings option from under the dialer object.  Check the 'Direct Connect' option, and then choose a 'Dedicated Carrier' from the drop-down list.  In most cases you will need a 'null modem cable' to connect the PageGate computer to a port on the in-house paging terminal.

### 4.6.5.	Dedicated Carriers

Each dialer has the option to be dedicated to a single paging carrier.  This is used with direct cable connections.  This option can also be used if a modem is connected to a phone line with a special phone service that gives beneficial rates, when dialing to a particular carrier.  To make a dialer dedicated to a single carrier: in PG Admin, choose the settings option from under the dialer object and then choose a 'Dedicated Carrier' from the drop-down list.  In most cases the dedicated carrier setting should be left at (None).  This will enable the dialer to send messages out to any carrier that has pending messages.

### 4.6.6.	Touch Tone Delivery

Some paging carriers only accept messages for their numeric pagers by using touch tones.  This is accomplished by dialing the pager's phone number, waiting for a prompt (voice message or series of beeps) and then sending a series of tones via the phone dial-pad.  PageGate  supports delivering numeric messages in this manner, but it is not the preferred method.  This method of message delivery has no provisions for error detection/correction or message confirmation.  It is sometimes known as 'blind dialing' because there is no way to really know if the other end actually received the message.

How to setup a recipient for touch-tone messages:

First a 'Tone' carrier must be setup.  Only a single 'Tone' carrier is needed to send message to all numeric pagers.  To make a carrier a 'Tone' carrier, choose 'Tone' as the 'Delivery Method' on the carrier setup screen.  The default wait string ',,@' should work for most pagers.  If you need to use different wait string for different pagers, you will need to create multiple 'Tone' carriers (one for each unique wait string needed).  The wait string can be changed to support pagers with different input methods.

The comma (,) is used to pause (about one second)

The at sign (@) is used to wait for silence (about five seconds of silence)

The default wait string will dial the pager's number, pause for a couple of seconds, and then wait for a five second period of silence (after the beeps or greeting message) and then send the message (touch tones).

To setup a recipient to use touch-tone delivery, simply put the numeric pager's phone number in the 'ID/PIN' field on the recipient setup screen, and choose the 'Tone' carrier you just setup, as the carrier.  If you have setup several carriers with different wait strings, choose the carrier that will work with this pager's service provider.  With this setup, PageGate will dial the phone number setup in the recipient's ID/PIN field, pause according to the commas or @ in the carrier's 'Wait String' field, and then send the touch-tone message.

Some paging providers use a common phone number for all numeric pagers.  The user must dial this number, wait for a tone, dial their pager's ID number, wait for another message, and then type the numeric message.  This can be accomplished in PageGate by setting the 'Tone' carrier's wait string to wait a predetermined amount of time (using commas and or an @ symbol).  This would  be the wait time between sending the pager's ID number and sending the actual touch-tone message.  On the recipient setup screen, in the recipient's 'ID/PIN' field, put the paging company's common phone number followed by a few commas or an @ symbol, and then the pager's ID number.  This will give the following results:  The phone number will be dialed from the first part of the recipient's ID/PIN field,  PageGate will pause according to the commas or @ in the ID/PIN field, PageGate will then send the second set of number in the recipient's ID/PIN field and then pause according to the carrier's wait field.  PageGate will then send the touch-tone message.

## 4.7.	GetAscii (commandline/ascii)

### 4.7.1.	Overview

The GetAscii module enables PageGate to accept messages from many off-the-shelf, third-party, and custom applications.  It is responsible for accepting messages sent via the sendpage.exe commandline program, and for polling a predetermined directory for Ascii text files containing message information.  This interface is very popular in several different types of applications.  Many third-party programs have the ability to issue a DOS/Windows commandline command (shell command).  The GetAscii interface will enable these programs to send messages to PageGate by using the sendpage.exe program.  Many developers find that using the GetAscii interface can easily add paging capabilities to their applications.  The GetAscii interface can even be used to accept messages from other operating systems.  With an FTP application running, message files can be transferred into the directory being polled by GetAscii.  This would enable MAC's Unix systems, mini computers, etc. to send messages with PageGate.  The Advanced Ascii options enable this interface to watch for changes in files, and to read in data that has been added to the end of existing files.

### 4.7.2.	Settings

Enabled - check this box to enable the GetAscii interface.  The interface cannot be enabled unless a valid polling directory is provided.  If the PageGate server is already running before you check this box, you will need to manually start this interface by right-clicking on 'GetAscii' and choosing 'Start' from the popup menu.  The next time the PageGate server is started, this interface will startup automatically.

Polling directory - the polling directory is the location where the GetAscii interface looks for text files with paging information in them.  If the sendpage.exe program is being used, this is where it should create its temporary data files.

Polling Interval - the polling interval sets how often (in seconds) the GetAscii interface checks its polling directory(s) for new messages

Advanced Polling Options - advanced polling options can be used when you don't have control of the format of the Ascii text files to be accepted by the GetAscii interface or if you want to extract message information from files that change.

### 4.7.3.	Advanced Polling

PageGate version 4.0 has an  'Advanced Ascii Polling' feature.  By using advanced ascii polling, GetAscii can check for files in an unlimited number of different directories, and for files with any name.  In addition, the text files don't need any special header information in them, and can be slotted for a particular recipient  by the file's name, by its location, or by both values.  PageGate can also 'watch' for message information in files that change or that are added to.

Setting Advanced Polling Options

Files - This is where you set the folders and file names that the GetAscii module should monitor.  Fill in each row on the 'Advanced Polling Files' screen with a new set of polling parameters.  An existing rule can be edited or deleted by highlighting the row and choosing 'Edit' or 'Delete' .

Recipient - This field can contain either an asterisk (*) or an existing PageGate recipient's name.  If an asterisk is used, the message's recipient and from fields are extracted from the message file(s).  If a recipient's name is used instead of an asterisk, the from field is used, and any files that are described by the 'File(s)' and 'Path' fields are collected and sent to this recipient.

From - This field is ignored when an asterisk is used in the 'Recipient' field.  Otherwise, the 'From' field becomes part of each message that is collected by this rule.

File(s) - This field is used to describe what files are collected by this rule.  Wildcard characters ('*' and '?') can be used.  For example, an entry of '*.txt' will collect all files that end with '.txt'.  An entry of '?at' will collect files like 'cat' and 'bat' and 'rat', but not 'flat'.

File Type - The Advanced ASCII interface supports four file types:

Standard - the text file is in the standard format that the GetAscii interface looks for.  This is option is usually used when the files are in the regular format, but the file isn't named with the standard .asc file extension, or is in a folder other than the main GetAscii polling folder.

Custom - each text file contains a single message, but the message information is not in the regular format and must be extracted from the text file using the Symbols, Record, Recipient, Sender, and Message objects below.

Adding - the text file periodically has message information added to the end of the file.  With this file type the GetAscii interface will read in the new data and extract the message information using the Symbols, Record, Recipient, Sender, and Message objects below.

Changing - the text file changes periodically.  Each time it changes, the GetAscii interface will try to extract message information from it using the Symbols, Record, Recipient, Sender, and Message objects below.

Path - This field is used to set where (which directory or folder, and on what drive) the ascii text files will be collected from.  Wildcards are not valid in this field.

Symbols - These are characters that have special meanings and are used to help extract message information from the text files.

Single Character Wildcard - when searching for pattern matches in the text files, this character can be used to stand for any single character

Multiple Character Wildcard - when searching for pattern matches in the text files, this character can be used to stand for any character or group of characters.

End Of Record Symbol - when extracting data from the text files, this character stands for the end of the record.  It is usually used as an ending pattern if the data being extracted always goes to the end of the current record.

ASCII Value Delimiter - this symbol is used to help represent characters that normally can't be typed.  This character is always followed three numbers which represent the decimal ascii value for the character.  For example (assuming that the \ character is the ASCII Value Delimiter) these strings represents a carriage-return, linefeed, tab and backslash respectively: \013  \010  \009 \092

Record - These settings are used to define how a record (which typically contains the recipient, sender, and message data) is to be recognized in the text file.

Start Pattern - this is a pattern that is looked for in the text file that will designate the beginning of a new record

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern is where the actual record starts.  A value of -999 always means that the beginning of the record is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know the length of the matching pattern.

End Pattern - this is a pattern that is  looked for in the text file that will designate the end of a new record.  If this pattern is set to the End Of Record symbol, the end of the record will be assumed to be the end of the text file.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern is where the actual record ends.  A value of -999 always means that the end of the record is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Recipient - These settings are used to define who the recipient of the message should be.

Static Recipient - when this option is chosen, all messages read in will be sent to the same recipient or group.

Recipient - when the Static Recipient option is chosen, this is the name of the recipient or group that will receive all the messages that are read in.

Dynamic Recipient - when this option is chosen, the recipient of the message is extracted from the text read in by the GetAscii interface.

Start Pattern - this is a pattern that is looked for in the record text that will designate the beginning of the recipient text.

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern, is where the actual recipient text starts.  A value of -999 always means that the beginning of the recipient text is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know what the length of the matching pattern will be.

End Pattern - this is a pattern that is  looked for in the record that will designate the end of recipient text.  If this pattern is set to the End Of Record symbol, the end of the recipient text will be assumed to be everything up to the end of the record.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern, is where the actual recipient text ends.  A value of -999 always means that the end of the recipient text is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Use Lookup Table - check this box if you want pre-set text substituted for the actual text read in from the file.  This is especially useful if codes are used in the text.  For example, if the text that is read in contains numbers to signify what pager should get the message, the lookup table could have a list of these numbers, and automatically use the actual recipient name instead of the numeric code.

Edit Table - click on this button to edit the lookup table to be used by the GetAscii interface.

Sender - These settings are used to define who the sender of the message should be.

Static Sender - when this option is chosen, all messages read in will use the same text for the sender field.

Sender - when the Static Sender option is chosen, this is the name of the sender that will be used with each message.

Dynamic Sender - when this option is chosen, the sender of the message is extracted from the text read in by the GetAscii interface.

Start Pattern - this is a pattern that is looked for in the record text that will designate the beginning of the sender text.

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern, is where the actual sender text starts.  A value of -999 always means that the beginning of the sender text is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know what the length of the matching pattern will be.

End Pattern - this is a pattern that is  looked for in the record that will designate the end of sender text.  If this pattern is set to the End Of Record symbol, the end of the sender text will be assumed to be everything up to the end of the record.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern, is where the actual sender text ends.  A value of -999 always means that the end of the sender text is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Use Lookup Table - check this box if you want pre-set text substituted for the actual text read in from the file.  This is especially useful if codes are used in the text.  For example, if the text that is read in contains codes to specify a piece of equipment that the message is from, the lookup table could have a list of the numbers, and automatically substitute the equipment name as the sender instead of the number.

Edit Table - click on this button to edit the lookup table to be used by the GetAscii interface.

Message - These settings are used to define the actual message text that should be sent.

Static Message - this option is used when the same message should be sent each time the GetAscii interface reads in a new record

Message - when the Static Message option is chosen, this is the message that will be sent each time.

Dynamic Message - when this option is chosen, the message is extracted from the text read in by the GetAscii interface.

Start Pattern - this is a pattern that is looked for in the record text that will designate the beginning of the message text.

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern, is where the actual message text starts.  A value of -999 always means that the beginning of the message text is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know what the length of the matching pattern will be.

End Pattern - this is a pattern that is  looked for in the record that will designate the end of message text.  If this pattern is set to the End Of Record symbol, the end of the message text will be assumed to be everything up to the end of the record.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern, is where the actual message text ends.  A value of -999 always means that the end of the message text is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Use Lookup Table - check this box if you want pre-set text substituted for the actual text read in from the file.  This is especially useful if codes are used in the text.  For example, if the text that is read in contains numbers to signify different alerts, the lookup table could have a list of these numbers, and automatically substitute the actual alert text for the code.

Edit Table - click on this button to edit the lookup table to be used by the GetAscii interface.

### 4.7.4.	Starting & Stopping

The GetAscii interface can be started or stopped by using the PG Admin program.  Right-click on the 'GetAscii' object (found under the 'Interfaces' object) and then choose 'Start' or 'Stop' from the popup menu.  The GetAscii interface will start or stop within a few seconds.  If the GetAscii interface is enabled, it will be automatically started each time the PageGate server is started.

### 4.7.5.	Using sendpage.exe

The sendpage commandline program can be used to send messages to PageGate.  It can be run from a DOS or Windows commandline, or from within any program that can issue a commandline (shell) command.  Sendpage must be told where to create its temporary message files.  This location must be the same as the GetAscii Polling Directory, which is set in PageGate's Admin program.  The person or application that runs sendpage.exe must have sufficient rights to create files in this directory.  The syntax of sendpage.exe is as follows (typing sendpage.exe /? will also give this information):

sendpage [-dir=dest] to from message

'sendpage' is the name of the program itself

-dir= is a commandline option that sets the directory to where the temporary message files will be stored.

'dest' should be replaced with the actual path to the GetAscii Polled directory

'to' should be replaced with the recipient's name of who the message is to (can not be blank)

'from' should be replaced with who the message is being sent from (can not be blank)

'message' should be replaced with the actual message text (can not be blank)

Example:  sendpage -dir=c:\pages Chris Peter This is a test message

Optionally a DOS environment variable can be set so that the [-dir=dest] option does not need to be included in the command.  The environment variable name is SNDPGDIR.

Example:  SET SNDPGDIR=C:\PAGES

Here is the same example message with the SNDPGDIR environment variable set:

Example:  sendpage Chris Peter This is a test message

If sendpage.exe is run from the same directory which is being polled by PageGate's GetAscii module, the -dir flag and SNDPGDIR variable are not needed.

*** Please note: a program named sendpage32.exe is now included with PageGate's GetAscii interface.  It operates in the exact same way as sendpage.exe.  Sendpage32.exe however, is a 32-bit program.  This means that it may be able to be executed from within some 32-bit application that are unable to run 16-bit programs.  This also means that it may be unable to be executed from some 16-bit applications.  An advantage of using sendpage32.exe over sendpage.exe is that sendpage32 may be able to accept more characters on it's commandline (this can depend on the program running sendpage32.exe).  Due to a DOS/Windows limitation, sendpage.exe can only accept 128 characters on it's commandline.

### 4.7.6.	Using ascii text files

The GetAscii front-end interface can be used to poll a directory for Ascii text files that contain messages.  An Ascii text file is simply a file containing text only.  The DOS EDIT and Window Notepad programs create Ascii text files by default.  Ascii text files can also be created from within virtually any programming environment.  In order for GetAscii to pick-up Ascii text file messages, the file must be put in the Polled Directory specified in PageGate's Admin program.  GetAscii will only look for files with the .asc extension (see 'Advanced ascii polling' for other polling options).  If more than one file with a .asc extension exists in the polled directory, the files are sorted alphabetically and then processed in alphabetical order.  The format of the .asc files is as follows:

to

from

message

Please note that each line must have a carriage return terminating it (or carriage return and line feed).

'to' should be replaced with the recipient's name of who the message is to (can not be blank)

'from' should be replaced with who the message is being sent from (can not be blank)

'message' should be replaced with the actual message text (can not be blank).  The 'message' area is actually everything after the 'from' line.  Therefore multiple lines of message data is valid.

Example file named test.asc:

Chris

Peter

This is a

test.

Only a test.

### 4.7.7.	Ad-Hoc

Ascii text files can be used to send messages to pagers by using their PIN (ID) numbers.  This option is only available with a 5000 pager license or greater.  The advantage of this feature is that a user account for each pager does not have to be setup in PageGate.  Only a single Ad-Hoc recipient needs to be setup for each carrier.  The disadvantage is that the sender must know the PIN (ID) numbers of the pagers (harder to remember than recipient names).  Sending messages using Ad-Hoc is similar to sending Multi-Page messages.  In PageGate's Admin program, create a recipient and set it as an Ad-Hoc recipient.  The carrier information must be correct; it is used to set which carrier the messages are sent to.  If  'ahu' were an Ad-Hoc recipient, this would be a sample text file to send Ad-Hoc messages:

ahu 1234567 7654321 2468086

Peter

This is a test Ad-Hoc message

The above example would dial the carrier specified by the ahu account and send the message to the pagers with PIN (ID) numbers 1234567, 7654321 and 2468086.  Multiple PIN (ID) numbers on the 'to' line should be separated by spaces, commas, or semi-colons.

### 4.7.8.	MultiPage

A single text file can be used to send a message to more than one recipient, by using PageGate's Multi-Page option.  In PageGate's Admin program, create a recipient and set it as a Multi-Page recipient.  This recipient is simply used as a flag to say that more recipients follow.  For example, if a recipient named 'mpu' is setup as a Multi-Page recipient, this would be an example of a message being sent to more than one recipient:

mpu Chris Bob Mary Steve

Peter

This is a test sent to four people

The above example would send the same message to Chris, Bob, Mary, and Steve.  Multiple names on the 'to' line should divided by spaces, commas, or semi-colons.

### 4.7.9.	Additional Notes

The Ascii/Commandline engine will check the directory you setup in the PG Admin program for files that have .asc as an extension (additional polling directories/filenames can be setup using the 'Advanced ascii polling' option.  see the section above for more information).  It will check for the files at the polling interval (in seconds) which is also setup in the PG Admin program.  When the directory is polled, and a file is found, it is immediately moved to the 'process' subdirectory (underneath the polled directory) and given a unique filename (the .asc is replaced with a unique number).  If in the same polling period, two or more files are put into the polled directory with the same filename, the first file will be overwritten by the second.  The files are not given a unique name until the directory is actually polled.  To avoid this, if the files are being written by an application, be sure to use unique filenames.  If the sendpage program is being used, unique filenames are automatically generated.  When the message is polled, if the format of the message or the recipient name is invalid, it will be immediately moved to the 'bad' subdirectory (under the polled directory).  The file will have the same filename it was originally given, but the extension will be changed to a unique number.  If the message is in a valid format and the recipient is also valid, the message will be moved to the 'process' subdirectory (under the polled directory) and then given to the paging engine for delivery.  When the paging engine is done with the message, it will be moved from the 'process' subdirectory to either the 'good' or the 'bad' subdirectory depending on the ability of the engine to send the message to the carrier.  Application programs can check the 'process', 'good' and 'bad' subdirectories for the original filename (with .* as an extension), to verify the status of the message.  Messages are purged from the 'good' and 'bad' directories when they are older (in days) than the 'purge after' setting in the PG Admin program.

## 4.8.	GetMail (email gateway)

### 4.8.1.	Overview

PageGate has email functionality built directly into its GetMail module.  The GetMail module enables PageGate to act as an email to pager gateway.  When acting as a email to pager gateway, PageGate can send the from field, subject line, and body of an email message directly to a pager or other wireless messaging device.  Attachments such as graphics, etc. are ignored.  Optionally, a copy of each message received by PageGate (on any of its interfaces) can be forwarded to an additional email address, for each recipient.  The following information describes several options PageGate has to accept/retrieve email messages.

Using PageGate with a registered domain name, a static IP address, and a dedicated internet connection.

Advantages - This is the most efficient way to use PageGate in regards to both speed of message delivery, and use of computer resources.  When configured this way, PageGate can also act  as the primary mail server for your organization.

Disadvantages - Dedicated internet connections can sometimes be costly.

Using PageGate with a registered domain name and a dialup connection

Advantages - This configuration is relatively efficient as far as computer resources go, and is one of the most economical ways to obtain functionality.  When configured this way, PageGate can also act as the primary mail server for your organization.  Most, but not all, ISPs support this configuration (often know as mail bagging or domain popmail)

Disadvantages - There is a delay in message collection, which is directly proportional to how often PageGate is set to dialup and check for messages.

Using PageGate in conjunction with another mail server (i.e. MS Exchange, Unix based server, etc.)

Advantages - PageGate runs efficiently this way, as it is configured similar to the first option above.

Disadvantages - There is some setup necessary on the existing mail server.  NotePage, Inc. cannot give the exact steps necessary to configure the existing mail server.

Using PageGate without a registered domain name and using POP mail collection

Advantages - Mail can be collected from many mailboxes on different mail servers.  This option can be used in addition to any of the methods described above.  This method can be used with either a dialup connection, or a dedicated internet connection.

Disadvantages - There is a delay in mail collection, which is directly proportional to how often PageGate is set to collect POP mail.   This method is the least efficient in regards to both connection bandwidth and computer resources.  The connection and processor speeds can limit the number of POP accounts that can be collected from within a period of time.

***Note for users upgrading from old versions of PageGate: PageGate no longer uses the MDaemon mail server.  All mail server capabilities are built directly into PageGate.

### 4.8.2.	Settings

Enabled - check this  box to enable the GetMail interface.  If the PageGate server is already running before you check this box, you will need to manually start this interface by right-clicking on 'GetMail' and choosing 'Start' from the popup menu.  The next time the PageGate server is started, this interface will startup automatically.

Local Domain - This is the domain name to be used by PageGate when receiving email.  This is an internet domain name, not a Microsoft networking domain.  Only email with an address in this domain will be accepted by PageGate.  This setting must be a valid domain name for to work properly (Note: an IP address can be used here instead of a domain name, if email is addressed to match.  This is commonly done during testing when a domain name has not yet been assigned to this machine).

Postmaster - the Postmaster setting is an email address to where all warnings, alerts, an bad messages are sent.  It is very useful to set this field to an email address that can be collected by PageGate's administrator.  The email address set here can either be a mailbox maintained by PageGate, or a mailbox on another mail server.  Leaving this field blank will drop all messages that would normally go to the postmaster.

Relay All Mail - checking this option will force PageGate to deliver all outbound email messages to a specific email server rather than trying to deliver them to the actual destination server.  This is commonly used with a slow connection or when PageGate is behind a firewall.  It is then the responsibility of the relay mail server to deliver all the outbound messages to their actual destinations.

Relay Server - if the 'Relay All Mail' option is chosen, you must enter the name of a mail server here.  You may also use this entry if 'Relay All Mail' is not checked.  In this scenario, any messages that are unable to be sent directly to their destination by PageGate, will be sent to the relay mail server.

DNS Server 1 & 2 - If you are not using a relay server, then PageGate must deliver email directly to its destination.  PageGate uses these DNS servers to lookup the mail server that hosts the mailboxes that outbound email is addressed to.

Email Outcome - when this box is checked, an email message will be sent back to the message's sender, with the outcome of the delivery attempt to its carrier.

POP Interval - If PageGate is setup to use 'Domain POP Mail Collection' or if any of PageGate's recipients are setup to collect mail from remote POP mailboxes, this is the interval at which collection takes place.

Dial-Up Networking Settings - this is where you can setup PageGate to use Dial-Up Networking to connect to the internet for mail collection.

Ad-Hoc Mail Settings - if PageGate should accept email addressed to the ID/PIN number of the recipient (i.e. 1347652@mypagers.com), this is where you can set that up.

Domain POP Mail Settings - PageGate can be setup to collect all the mail for an entire domain from a single POP mailbox.  This is where those settings are made.

### 4.8.3.	Ad-Hoc Mail Settings

PageGate can handle Ad-Hoc email in two different ways.  The first way is to accept emails with ID/PIN numbers on the subject line of the mail messages.  The second way is to accept email addressed to the ID/PIN number that the message should use for delivery (i.e. 1234567@pagingserver.com).  Both these methods can be used conjunction with each other at the same time.

To setup the first scenario mentioned above:

1. Create a new recipient and set its 'Recipient Type' to 'Ad-Hoc'.

2. Check the 'Mail' box in the 'Enabled Services' section

3. Choose the carrier that messages addressed to this recipient should be sent to.

To use this new ad-hoc recipient account via email, send email addressed to the ad-hoc recipient just created, and in the subject line enter one or more ID/PIN numbers (separated by spaces or commas).  The carrier associated with this ad-hoc recipient will be connected to and a copy of the email message will be sent to each ID/PIN number on the subject line of the email.

To setup the second scenario mentioned above:

1. Create a new recipient and set its 'Recipient Type' to 'Ad-Hoc'

2. Check the 'Mail' box in the 'Enabled Services' section

3. Choose the carrier that messages should be sent to via ad-hoc.

4. On the 'GetMail' setup screen (found under the 'Interfaces' object) click on the 'Ad-Hoc Mail Settings' button.

5. Choose 'Add' from the next screen.

6. Enter a range of ID/PIN numbers that you want sent to the carrier selected in step 3 above by providing a 'Starting ID/PIN' and an 'Ending ID/PIN'

7. From the 'Ad-Hoc Recipient' drop-down list, choose the recipient that was setup in step 1 above.

8. Apply your settings.

To use this method with email, simply address an email message to the PageGate server with an ID/PIN number (within the range set in setup 6 above) as the mailbox part of the email address (i.e. 1234567@mypagegateserver.com).

### 4.8.4.	Dial-Up Networking Settings

PageGate can use Dial-Up networking to connect to the Internet for collecting email.  A Dial-Up networking account must first be setup in Windows in the usual manor before the account can be used by PageGate.  When Dial-Up networking is enabled, the 'POP Interval' setting on the main GetMail settings screen will be used as the interval at which PageGate will connect to the Internet to collect email.  After PageGate is done receiving, it will automatically disconnect from the Internet and wait for the next 'POP Interval' to arrive.  If the Dial-Up networking account is already active (connected), when it is time for PageGate to collect its email, PageGate will use the current connection, but will not disconnect the connection when done (PageGate assumes that another application is controlling the Dial-Up session and will not interfere with it).

Enable Dial-UP Networking - Check this box to tell PageGate to use Dial-Up networking to connect to the Internet (A Dial-Up account must be selected in the 'Use This Account' box before you will be able to check this box).

Use This Account - Choose and existing Dial-Up networking account from this drop-down list (A Dial-Up networking account must already be setup in Windows in order to show up in this list).

Account Password - Enter the password required by the selected Dial-Up networking account to connect to the Internet.

Coordinate with Dialer1 - If you want to use a single modem and phone line to both connect to the Internet for email collection, and to send out messages to your paging carrier, you must check this box so that PageGate's modules with coordinate the use of the modem with each other.

### 4.8.5.	Domain POP Mail Settings

PageGate can collect the all the email for and entire domain from a single mailbox.  Most Internet Service Providers (ISPs) have the ability to put all email addressed to your domain into a single POP mailbox.  This is commonly known as 'Domain POP Mail' or 'Mailbagging'.  It works well with Dial-Up Internet connections, or Internet connections that don't have a static IP address.  PageGate will try to collect mail from the mailbox according to the 'POP Collection Interval' (found on GetMail's 'Settings' screen).  Once PageGate retrieves the messages, it will sort them into all the correct local mailboxes that you have setup in PageGate.  This setup can be used to give the appearance of having a dedicated in-house mail server to both the Internet, and workstations on your local area network.

Steps to use Domain POP Mail:

1. Obtain a registered domain name (your ISP can usually help with this).

2. Have you ISP setup mailbagging (domain pop mail).

3. If you will be using Dial-Up networking to connect to the Internet see the previous section 'Dial-Up Networking Settings' to configure it.

4. On the 'GetMail' setup screen (found under the 'Interfaces' object) click on the 'Domain POP Mail Settings' button.

5. Check the 'Enabled' box.

6. Enter the name of the mailbox at your ISP you will collect the domain's mail from (do not include the @domain.com part) in the field 'POP Mailbox'

7. Enter the password to enter this mailbox in the 'POP Password' field.

8. Enter the name of your ISP's mail server in the 'POP Server' field.

9. If you want PageGate to remove all the email messages off your ISP's server after it has collected them, check the 'Remove Collected Mail From Server' box.

10. Apply your settings.

11. Setup one or more recipients in PageGate.  By default, the recipient's name will be the name of their email address.  Set the 'Recipient Type' to 'Email' if you want this account to only be used for email (no paging).

12. Some additional recipient email options can be set by clicking on the 'Email' object found beneath the recipient object you just created.

* See the section labeled 'POP Mail Settings' to collect mail from remote mail servers for individual recipients.

### 4.8.6.	Starting & Stopping

The GetMail interface can be started or stopped by using the PG Admin program.  Right-click on the 'GetMail' object (found under the 'Interfaces' object) and then choose 'Start' or 'Stop' from the popup menu.  The GetMail interface will start or stop within a few seconds.  If the GetMail interface is enabled, it will be automatically started each time the PageGate server is started.

### 4.8.7.	Using PageGate with a registered domain name, a static IP address, and a dedicated internet connection.

Advantages - This is the most efficient way to use PageGate in regards to both speed of message delivery, and use of computer resources.  When configured this way, PageGate can also act  as the primary mail server for your organization.

Disadvantages - Dedicated internet connections can sometimes be costly.

Here are the necessary settings to run PageGate this way:

On the 'Interfaces' > 'GetMail' > 'Settings' screen:

- Make sure the 'Enabled' box is checked.

- A valid domain name that is to be used by PageGate mail service should be entered in the 'Domain Name' field.  PageGate will expect all incoming mail to be addressed to this domain name.  PageGate will not relay mail addressed to other domains.

- A 'Postmaster' address can be entered if desired.  This email address can be a local PageGate recipient, or the address of an email account on another mail server.  It will receive information about all warnings, alerts, and failures produced by the email interface.  If this field is left blank, these messages simply won't be sent to anyone.

- If the 'Email Outcome' option is checked, an email message will be sent back to the sender of each message with the result of its delivery to the paging carrier (successful or failed).

- Check 'Relay All Mail' if you don't want PageGate to deliver email to its final destination, but would prefer it to spool all outbound email to another mail server.  This is commonly used with a slow internet connection to spool mail to an ISP's mail server, or if PageGate is used in conjunction with another existing mail server.

- The 'Relay Server' is the name or IP Address of a mail server to be used if the 'Relay All Mail' option is checked.  If this address is set, and the 'Relay All Mail' option is not checked, it is used if PageGate is unable to deliver any messages directly to their destination.

- If PageGate is not set to 'Relay All Mail', then you must provide the IP Address for at least one DNS server.  PageGate requires a DNS server address so it can lookup the addresses of the domains it is to deliver email to.

- The 'POP Interval' settings is not used with this configuration (unless recipients are setup to collect mail from other mail servers).

- The 'Dial-Up Networking Settings' and 'Domain POP Mail Settings' are not necessary for this configuration.

- See the 'Ad-Hoc' section for additional information on the 'Ad-Hoc' setup screen.

On the 'Recipients' > '{recipient_name_here}' > 'Settings' screen:

- Do not choose the 'Email' option in the 'Recipient Type' section.  This is only used to create email accounts that have not paging capabilities (used for everyday email).

- Make sure the 'Mail' box is checked in the 'Enabled Services' section.

On the 'Recipients' > '{recipient_name_here}' > 'Email' screen:

- Set the 'Mailbox Name' to what you want this recipient to use for an email address (don't include the domain name here).  In most cases, this should be left as the recipient's name.

- If you want a copy of all messages that are sent to this recipient forwarded to another email account, enter the email address in the 'Forward Copy To' field.

- Check the 'Enable POP Access to Messages' box if you want PageGate to keep a copy of all messages received by this recipient in a local POP mailbox.  These messages can be collected with any regular email client software (Outlook, Netscape, Eudora, etc.).

- If you checked the above option, enter a password to restrict access to this mailbox.

- For more information about the 'POP Mail Collection' button, see the configuration option below named 'Using PageGate without a registered domain name and using POP mail collection'

- The 'Email Aliases' screen allows you to enter one or more additional local email addresses that should be associated with this recipient.  This enables this recipient to receive mail for more than one email address.

### 4.8.8.	Using PageGate with a registered domain name and a dialup connection

Advantages - This configuration is relatively efficient as far as computer resources go, and is one of the most economical ways to obtain functionality.  When configured this way, PageGate can also act as the primary mail server for your organization.  Most, but not all, ISPs support this configuration (often know as mail bagging or domain popmail)

Disadvantages - There is a delay in message collection, which is directly proportional to how often PageGate is set to dialup and check for messages.

Here are the necessary settings to run PageGate this way:

On the 'Interfaces' > 'GetMail' > 'Settings' screen:

- Make sure the 'Enabled' box is checked.

- A valid domain name that is to be used by PageGate mail service should be entered in the 'Domain Name' field.  PageGate will expect all incoming mail to be addressed to this domain name.  PageGate will not relay mail addressed to other domains.

- A 'Postmaster' address can be entered if desired.  This email address can be a local PageGate recipient, or the address of an email account on another mail server.  It will receive information about all warnings, alerts, and failures produced by the email interface.  If this field is left blank, these messages simply won't be sent to anyone.

- If the 'Email Outcome' option is checked, an email message will be sent back to the sender of each message with the result of its delivery to the paging carrier (successful or failed).

- Check 'Relay All Mail' if you don't want PageGate to deliver email to its final destination, but would prefer it to spool all outbound email to another mail server.  Check this box for the most efficient use of your dialup connection.

- The 'Relay Server' is the name or IP Address of a mail server to be used if the 'Relay All Mail' option is checked.  If this address is set, and the 'Relay All Mail' option is not checked, it is used if PageGate is unable to deliver any messages directly to their destination.  Set this field to the address of your ISP's mail server for the most efficient use of your dialup connection.

- If PageGate is not set to 'Relay All Mail', then you must provide the IP Address for at least one DNS server.  PageGate requires a DNS server address so it can lookup the addresses of the domains it is to deliver email to.

- The 'POP Interval' settings is used as the interval at which PageGate will dial up and collect any waiting email messages from your ISP's mail server.

- See the 'Ad-Hoc' section for additional information on the 'Ad-Hoc' setup screen.

- You will need to use both the 'Dial-UP Networking Settings' and the 'Domain POP Mail Settings' buttons to finish configuring PageGate's email service (instructions follow).

On the 'Interfaces' > 'GetMail' > 'Settings' > 'Dial-Up Networking Settings' screen:

- Make sure the 'Enable Dial-Up Networking' box is checked.

- From the 'Use This Account' drop-down list, select an existing dial-up networking configuration.

- Enter the password required by your ISP when using this dial-up account.

- Check the 'Coordinate with Dialer1' box if you only have one modem and phone line and want PageGate to use it for both connecting to the internet for sending and receiving email, and for sending messages to your paging carrier via modem.

On the 'Interfaces' > 'GetMail' > 'Settings' > 'Domain POP Mail Settings' screen:

- Make sure the 'Enabled' box is checked.

- In the 'POP Mailbox' field, enter the name of the mailbox at your ISP that PageGate should collect all your email from.

- In the 'POP Password' field, enter the password for this mailbox

- In the 'POP Server' field, enter the name or IP Address of your ISP's mail server.

- Check the 'Remove Collected Mail From Server' box if you want PageGate to remove the mail from this mailbox after it has been collected.

On the 'Recipients' > '{recipient_name_here}' > 'Settings' screen:

- Do not choose the 'Email' option in the 'Recipient Type' section.  This is only used to create email accounts that have not paging capabilities (used for everyday email).

- Make sure the 'Mail' box is checked in the 'Enabled Services' section.

On the 'Recipients' > '{recipient_name_here}' > 'Email' screen:

- Set the 'Mailbox Name' to what you want this recipient to use for an email address (don't include the domain name here).  In most cases, this should be left as the recipient's name.

- If you want a copy of all messages that are sent to this recipient forwarded to another email account, enter the email address in the 'Forward Copy To' field.

- Check the 'Enable POP Access to Messages' box if you want PageGate to keep a copy of all messages received by this recipient in a local POP mailbox.  These messages can be collected with any regular email client software (Outlook, Netscape, Eudora, etc.).

- If you checked the above option, enter a password to restrict access to this mailbox.

- For more information about the 'POP Mail Collection' screen, see the configuration option below named 'Using PageGate without a registered domain name and using POP mail collection'

- The 'Email Aliases' screen allows you to enter one or more additional local email addresses that should be associated with this recipient.  This enables this recipient to collect mail for more than one email address.

### 4.8.9.	Using PageGate in conjunction with another mail server (i.e. MS Exchange, Unix based server, etc.)

Advantages - PageGate runs efficiently this way, as it is configured similar to the first option above.

Disadvantages - There is some setup necessary on the existing mail server.  NotePage, Inc. cannot give the exact steps necessary to configure the existing mail server.

There are three ways to use PageGate with an existing mail server.

1. Configure your existing mail server to forward to PageGate a copy of any messages you want sent to a pager.  Typically, an account is setup in PageGate for each account on the existing mail server, that should have paging abilities.  Then on the existing mail server, some sort of forwarding setting or rule is entered so that a copy of each message received in the existing email box is forwarded to their PageGate email account.

To configure PageGate to run this way, make the necessary changes on you existing mail server, and then setup PageGate following the instructions for 'Using PageGate with a registered domain name, a static IP address, and a dedicated internet connection'.

2. Configure PageGate to forward a copy of the email it receives to your existing mail server.  To accomplish this, for each PageGate recipient, you would enter the recipient's email address on your existing mail server in the recipient's 'Forward Copy To' field.

To configure PageGate to run this way, follow the instructions for 'Using PageGate with a registered domain name, a static IP address, and a dedicated internet connection'.  Then:

On the 'Recipients' > '{recipient_name_here}' > 'Email' screen:

- In the 'Forward Copy To' field, enter the email address of the mailbox on your existing mail server that should receive a copy of the email this PageGate recipient receives.

3. Configure each of PageGate's recipients to individually collect (POP) mail from their corresponding account on your existing mail server.  Each PageGate recipient is able to collect email from any number of accounts on other mail servers.  You would need to set each PageGate recipient to collect its mail from an account on your existing server (but not remove the message from that server).  Please note that this options is not as efficient as either of the first two options.

To configure PageGate to run this way, follow the instructions for 'Using PageGate without a registered domain name and using POP mail collection' in the topic following this one.

### 4.8.10.	Using PageGate without a registered domain name and using POP mail collection

Advantages - Mail can be collected from many mailboxes on different mail servers.  This option can be used in addition to any of the methods described above.  This method can be used with either a dialup connection, or a dedicated internet connection.

Disadvantages - There is a delay in mail collection, which is directly proportional to how often PageGate is set to collect POP mail.   This method is the least efficient in regards to both connection bandwidth and computer resources.  The connection and processor speeds can limit the number of POP accounts that can be collected from within a period of time.

To use this option with a dedicated internet connection:

On the 'Interfaces' > 'GetMail' > 'Settings' screen:

- Make sure the 'Enabled' box is checked.

- A domain name that is to be used by PageGate mail service when sending email should be entered in the 'Domain Name' field.  Although you must enter a value in this field, it is not used for incoming mail in this configuration option.

- A 'Postmaster' address can be entered if desired.  This email address can be a local PageGate recipient, or the address of an email account on another mail server.  It will receive information about all warnings, alerts, and failures produced by the email interface.  If this field is left blank, these messages simply won't be sent to anyone.

- If the 'Email Outcome' option is checked, an email message will be sent back to the sender of each message with the result of its delivery to the paging carrier (successful or failed).

- Check 'Relay All Mail' if you don't want PageGate to deliver email to its final destination, but would prefer it to spool all outbound email to another mail server.  This is commonly used with a slow internet connection to spool mail to an ISP's mail server, or if PageGate is used in conjunctions with another existing mail server.

- The 'Relay Server' is the name or IP Address of a mail server to be used if the 'Relay All Mail' option is checked.  If this address is set, and the 'Relay All Mail' option is not checked, it is used if PageGate is unable to deliver any messages directly to their destination.

- If PageGate is not set to 'Relay All Mail', then you must provide the IP Address for at least one DNS server.  PageGate requires a DNS server address so it can lookup the addresses of the domains it is to deliver email to.

- The 'POP Interval' settings is used as the interval at which PageGate will check all the remote mailboxes for waiting messages.

- The 'Dial-Up Networking Settings' and 'Domain POP Mail Settings' are not necessary for this configuration.

- See the 'Ad-Hoc' section for additional information on the 'Ad-Hoc' setup screen.

On the 'Recipients' > '{recipient_name_here}' > 'Settings' screen:

- Do not choose the 'Email' option in the 'Recipient Type' section.  This is only used to create email accounts that have not paging capabilities (used for everyday email).

- Make sure the 'Mail' box is checked in the 'Enabled Services' section.

On the 'Recipients' > '{recipient_name_here}' > 'Email' screen:

- Set the 'Mailbox Name' to what you want this recipient to use for an email address (don't include the domain name here).  In most cases, this should be left as the recipient's name.

- If you want a copy of all messages that are sent to this recipient forwarded to another email account, enter the email address here.

- Check the 'Enable POP Access to Messages' box if you want PageGate to keep a copy of all messages received by this recipient in a local POP mailbox.  These messages can be collected with any regular email client software (Outlook, Netscape, Eudora, etc.).

- If you checked the above option, enter a password to restrict access to this mailbox.

- The 'Email Aliases' screen allows you to enter one or more additional local email addresses that should be associated with this recipient.  This enables this recipient to collect mail for more than one email address.

On the 'Recipients' > '{recipient_name_here}' > 'Email' > 'POP Mail Collection' screen:

- Click on the 'Add' button to add an email account that you want to collect messages from, for this recipient.

- Enter the name of the mailbox on the remote mail server that you want to collect mail from.

- Enter the password needed to collect mail from this mailbox.

- Enter the name or IP Address of the mail server where the mailbox resides.

- Check the 'Remove Collected Mail From Server' if you want PageGate to delete messages off the remote server, after they have been collected.

To use this option with a dial-up connection:

On the 'Interfaces' > 'GetMail' > 'Settings' screen:

- Make sure the 'Enabled' box is checked.

- A domain name that is to be used by PageGate mail service when sending email should be entered in the 'Domain Name' field.  Although you must enter a value in this field, it is not used for incoming mail in this configuration option.

- A 'Postmaster' address can be entered if desired.  This email address can be a local PageGate recipient, or the address of an email account on another mail server.  It will receive information about all warnings, alerts, and failures produced by the email interface.  If this field is left blank, these messages simply won't be sent to anyone.

- If the 'Email Outcome' option is checked, an email message will be sent back to the sender of each message with the result of its delivery to the paging carrier (successful or failed).

- Check 'Relay All Mail' if you don't want PageGate to deliver email to its final destination, but would prefer it to spool all outbound email to another mail server.  Check this box for the most efficient use of your dialup connection.

- The 'Relay Server' is the name or IP Address of a mail server to be used if the 'Relay All Mail' option is checked.  If this address is set, and the 'Relay All Mail' option is not checked, it is used if PageGate is unable to deliver any messages directly to their destination.  Set this field to the address of your ISP's mail server for the most efficient use of your dialup connection.

- If PageGate is not set to 'Relay All Mail', then you must provide the IP Address for at least one DNS server.  PageGate requires a DNS server address so it can lookup the addresses of the domains it is to deliver email to.

- The 'POP Interval' settings is used as the interval at which PageGate will dial up and collect any waiting email messages from your ISP's mail server.

- You will need to use the 'Dial-UP Networking Settings' to finish configuring this option (instructions follow).

- See the 'Ad-Hoc' section for additional information on the 'Ad-Hoc' setup screen.

- 'Domain POP Mail Settings' is not used in this configuration.

On the 'Interfaces' > 'GetMail' > 'Settings' > 'Dial-Up Networking Settings' screen:

- Make sure the 'Enable Dial-Up Networking' box is checked.

- From the 'Use This Account' drop-down list, select an existing dial-up networking configuration.

- Enter the password required by your ISP when using this dial-up account.

- Check the 'Coordinate with Dialer1' box if you only have one modem and phone line and want PageGate to use it for both connecting to the internet for sending and receiving email, and for sending messages to your paging carrier via modem.

On the 'Recipients' > '{recipient_name_here}' > 'Settings' screen:

- Do not choose the 'Email' option in the 'Recipient Type' section.  This is only used to create email accounts that have not paging capabilities (used for everyday email).

- Make sure the 'Mail' box is checked in the 'Enabled Services' section.

On the 'Recipients' > '{recipient_name_here}' > 'Email' screen:

- Set the 'Mailbox Name' to what you want this recipient to use for an email address (don't include the domain name here).  In most cases, this should be left as the recipient's name.

- If you want a copy of all messages that are sent to this recipient forwarded to another email account, enter the email address here.

- Check the 'Enable POP Access to Messages' box if you want PageGate to keep a copy of all messages received by this recipient in a local POP mailbox.  These messages can be collected with any regular email client software (Outlook, Netscape, Eudora, etc.).

- If you checked the above option, enter a password to restrict access to this mailbox.

- The 'Email Aliases' screen allows you to enter one or more additional local email addresses that should be associated with this recipient.  This enables this recipient to collect mail for more than one email address.

On the 'Recipients' > '{recipient_name_here}' > 'Email' > 'POP Mail Collection' screen:

- Click on the 'Add' button to add an email account that you want to collect messages from, for this recipient.

- Enter the name of the mailbox on the remote mail server that you want to collect mail from.

- Enter the password needed to collect mail from this mailbox.

- Enter the name or IP Address of the mail server where the mailbox resides.

- Check the 'Remove Collected Mail From Server' if you want PageGate to delete messages off the remote server, after they have been collected.

### 4.8.11.	MultiPage

A single email message can send its contents to many PageGate recipients by using the Multi-Page feature.  A Multi-Page recipient must first be setup in PageGate's Admin program.  It is simply used as a flag, the recipient information is not important.  To send a Multi-Page message, address the email to the Multi-Page recipient, and then specify all the recipients the message should be sent to in the subject line of the email message.  Recipient names in the subject line should be separated by spaces, commas, or semi-colons.

### 4.8.12.	POP Mail Settings

Each of PageGate's recipients can be setup to collect email messages from one or more mailboxes on remote mail servers.

POP Mail Settings for a recipient can be maintained by clicking on the 'POP Mail Settings' button found on the 'Email' screen below the recipient object that you want to collect POP mail for.

POP Mailbox - the name of the mailbox on the remote mail server (does not include the @domain part)

POP Password - the password necessary to access the mailbox on the remote mail server

Remove Collected Mail From Server - after the messages are collected from the remote mail server, PageGate can delete them from the server.

* See the section labeled 'Domain POP Mail Settings' to collect mail for an entire domain from a single POP mailbox.

### 4.8.13.	Email Aliases

Each PageGate recipient can have more than one email address assigned to it.  Add a new alias to a recipient for each new local email address that should be associated with that recipient.

The aliases setup screen is accessed by clicking on the 'Email Aliases' button found on the 'Email' screen below the recipient object that you want to add aliases to.

### 4.8.14.	Email Filtering

PageGate can filter incoming email messages to decide if messages should be forwarded to the recipient or not.  Filtering is only done on the sender's email address.  Here are the steps necessary to setup email filtering in PageGate.

1. Create a new folder under the PageGate database folder and name it 'Filters' (no quotes)

2. In this folder you can create a text file named 'Global.AllowOnly'.  If this file exists, only mail from the domains and/or mailboxes included in this file will be sent your recipients. For example, if you had 'notepage.com' in the file (no quotes), then the only email that would be sent to the recipients get would be email from anyone at notepage.com. If you put 'chris@notepage.com' (no quotes) in the file, then only email from chris@notepage.com would be sent to your recipients. If you put ".com' in the file, then PageGate would only send messages out that were receive from from .com email addresses (not .net, .org, etc.).

2. in this folder you can create a text file named 'Global.DontAllow'.  If this file exists, then mail from any entries in this file won't be allowed. Fore example if you add 'notepage.com', then no mail from anyone  at notepage.com will be sent to the PageGate recipients. If you put 'chris@notepage.com' in the file, then you can receive email from everyone at notepage.com, except chris. If you add '.com' to the file, you won't be able to receive mail from anyone that has a .com address.

3. You can also create files named <recipient_name>.AllowOnly and <recipient_name>.DontAllow (<recipient_name> would be replaced with the actual recipient's name as it is setup in PageGate). They act the same as the global files, but are only for the individual recipients.

4. All the above files should only have a single entry on each line. Here is an example of a file for the recipient named 'page-chris' that won't get any email from yahoo.com, their mother-in-law, and anyone from Australia (the file would be named 'page-chris.DontAllow')

yahoo.com

mother-in-law@aol.com

.au

### 4.8.15.	Additional Notes

Using PageGate as a regular SMTP/POP3 email server

In addition to PageGate's paging capabilities, PageGate can function as a standard SMTP/POP mail server for regular everyday email.  Email-only accounts can be created which have no paging capabilities, but can act as regular email accounts and can be used with regular email clients (like Outlook, Netscape, Eudora, etc.).  To create an email-only account, create a recipient and choose 'Email' in the 'Recipient Type' section.  Then select the 'Email' object from under the newly created recipient.  Check the 'Enable POP Access to Messages' box, and enter a password in the 'POP Password' field.  Then simply setup your email client to use the PageGate computer as the mail server.

Running PageGate's mail server service on the same machine as an existing mail server

PageGate can be run on the same computer as another mail server.  This section discusses advanced topics and should only be performed by advanced users.  Each mail server must be bound to its own unique IP address for this configuration to work.  You must first bind you existing mail server to one IP address that your computer has (some mail server applications may not have this advanced option).  Next to setup PageGate to use the other IP address (you computer must have at least two IP addresses) you must make a few manual entries into the Windows registry.  Here are the steps:

1. Run the regedit.exe program by clicking on the Windows 'Start' button, choosing 'Run', typing 'regedit' into the entry field, and clicking the 'OK' button.

2. Browse to this registry key: HKEY_LOCAL_MACHINE\Software\NotePage\PageGate.

3. Create a new string value named SMTP_IP, and with a value of the IP address for PageGate's mail service to bind to.

4. Create a new dword value named SMTP_Port, and with a value of 25

5. Create a new string value named POP_IP, and with a value of the IP address for PageGate's mail service to bind to.

6. Create a new dword value named POP_Port, and with a value of 110

7. Exit regedit and restart PageGate's GetMail module (or simply restart the PageGate server).

## 4.9.	GetSerial

### 4.9.1.	Overview

The GetSerial interface enables PageGate to monitor a serial connection for data.  This interface is often used to accept text that would normally be sent to a printer, and extract out message information from that data.

### 4.9.2.	Settings

Enabled - check this box to enable the GetSerial interface.  If the PageGate server is already running before you check this box, you will need to manually start this interface by right-clicking on 'GetSerial' and choosing 'Start' from the popup menu.  The next time the PageGate server is started, this interface will startup automatically.

Serial Port - choose a serial port (COM port) for the GetSerial module to monitor for incoming data.

Baud Rate - choose a baud rate to use for this connection.  This setting must match the baud rate that the sending device uses.

Parity - choose the parity to be used for this connection.  This settings must match the parity setting used by the sending device.

Data Bits - choose the number of data bits to be used on this connection.  This settings must match the number of data bits used by the sending device.

Stop Bits - choose the number of stop bits to be used with this connection.  This settings must match the data bits setting used by the sending device.

Handshaking - if handshaking is used by the sending device for flow control, you must set GetSerial to use the same handshaking method.

Init String - init strings are not typically used with a direct serial connection, but may be used if a modem is attached to the port that GetSerial is set to use.

Symbols - These are characters that have special meanings and are used to help extract message information from the incoming data.

Single Character Wildcard - when searching for pattern matches in the data, this character can be used to stand for any single character

Multiple Character Wildcard - when searching for pattern matches in the data, this character can be used to stand for any character or group of characters.

End Of Record Symbol - when extracting data from the data, this character stands for the end of the record.  It is usually used as an ending pattern if the data being extracted always goes to the end of the current record.

ASCII Value Delimiter - this symbol is used to help represent characters that normally can't be typed.  This character is always followed three numbers which represent the decimal ascii value for the character.  For example (assuming that the \ character is the ASCII Value Delimiter) these strings represents a carriage-return, linefeed, tab and backslash respectively: \013  \010  \009 \092

Record - These settings are used to define how a record (which typically contains the recipient, sender, and message data) is to be recognized in the incoming data.

Start Pattern - this is a pattern of text that is looked for in the incoming data, that will designate the beginning of a new record

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern, is where the actual record starts.  A value of -999 always means that the beginning of the record is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know the length of the matching pattern.

End Pattern - this is a pattern that is  looked for in the incoming data, that will designate the end of a new record.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern is where the actual record ends.  A value of -999 always means that the end of the record is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Recipient - These settings are used to define who the recipient of the message should be.

Static Recipient - when this option is chosen, all messages read in will be sent to the same recipient or group.

Recipient - when the Static Recipient option is chosen, this is the name of the recipient or group that will receive all the messages that are read in.

Dynamic Recipient - when this option is chosen, the recipient of the message is extracted from the data read in by the GetSerial interface.

Start Pattern - this is a pattern that is looked for in the record text, that will designate the beginning of the recipient text.

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern, is where the actual recipient text starts.  A value of -999 always means that the beginning of the recipient text is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know what the length of the matching pattern will be.

End Pattern - this is a pattern that is  looked for in the record text, that will designate the end of recipient text.  If this pattern is set to the End Of Record symbol, the end of the recipient text will be assumed to be everything up to the end of the record.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern, is where the actual recipient text ends.  A value of -999 always means that the end of the recipient text is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Use Lookup Table - check this box if you want pre-set text substituted for the actual text read in by the interface.  This is especially useful if codes are used in the text.  For example, if the text that is read in contains numbers to signify what pager should get the message, the lookup table could have a list of these numbers, and automatically use the actual recipient name instead of the numeric code.

Edit Table - click on this button to edit the lookup table to be used by the GetSerial interface.

Sender - These settings are used to define who the sender of the message should be.

Static Sender - when this option is chosen, all messages read in will use the same text for the sender field.

Sender - when the Static Sender option is chosen, this is the name of the sender that will be used with each message.

Dynamic Sender - when this option is chosen, the sender of the message is extracted from the text read in by the GetSerial interface.

Start Pattern - this is a pattern that is looked for in the record text, that will designate the beginning of the sender text.

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern, is where the actual sender text starts.  A value of -999 always means that the beginning of the sender text is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know what the length of the matching pattern will be.

End Pattern - this is a pattern that is  looked for in the record text, that will designate the end of sender text.  If this pattern is set to the End Of Record symbol, the end of the sender text will be assumed to be everything up to the end of the record.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern, is where the actual sender text ends.  A value of -999 always means that the end of the sender text is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Use Lookup Table - check this box if you want pre-set text substituted for the actual text read in by the interface.  This is especially useful if codes are used in the text.  For example, if the text that is read in contains codes to specify a piece of equipment that the message is from, the lookup table could have a list of the numbers, and automatically substitute the equipment name as the sender instead of the number.

Edit Table - click on this button to edit the lookup table to be used by the GetSerial interface.

Message - These settings are used to define the actual message text that should be sent.

Static Message - this option is used when the same message should be sent each time the GetSerial interface reads in a new record

Message - when the Static Message option is chosen, this is the message that will be sent each time.

Dynamic Message - when this option is chosen, the message is extracted from the text read in by the GetSerial interface.

Start Pattern - this is a pattern that is looked for in the record text that will designate the beginning of the message text.

Start Offset - this value tells how many characters before or how many characters after the beginning of the start pattern, is where the actual message text starts.  A value of -999 always means that the beginning of the message text is one character after the start pattern.  This is helpful when the start pattern contains an * and you don't always know what the length of the matching pattern will be.

End Pattern - this is a pattern that is  looked for in the record that will designate the end of message text.  If this pattern is set to the End Of Record symbol, the end of the message text will be assumed to be everything up to the end of the record.

End Offset - this value tells how many characters before or how many characters after the beginning of the end pattern, is where the actual message text ends.  A value of -999 always means that the end of the message text is one character after the end pattern.  This is helpful when the end pattern contains an * and you don't always know what the length of the matching pattern will be.

Use Lookup Table - check this box if you want pre-set text substituted for the actual text read in by this interface.  This is especially useful if codes are used in the text.  For example, if the text that is read in contains numbers to signify different alerts, the lookup table could have a list of these numbers, and automatically substitute the actual alert text for the code.

Edit Table - click on this button to edit the lookup table to be used by the GetSerial interface.

### 4.9.3.	Starting & Stopping

The GetSerial interface can be started or stopped by using the PG Admin program.  Right-click on the 'GetSerial' object (found under the 'Interfaces' object) and then choose 'Start' or 'Stop' from the popup menu.  The GetSerial interface will start or stop within a few seconds.  If the GetSerial interface is enabled, it will be automatically started each time the PageGate server is started.

## 4.10.	GetTap

### 4.10.1.	Overview

The GetTap interface is used to accept incoming messages via the TAP protocol.  The GetTap interface accepts these message over a serial connection that is usually connected to a modem.  The TAP protocol is the most widely used text messaging protocol for serial and dialup connections.  The GetTap interface enables PageGate to accept in text messages from many sources, and then re-dispatch the messages back out (possible to different recipients or groups).  Also, PageGate can act as a messaging gateway for TAP clients (such as NotePager32 and NotePager Pro), simplifying setup, logging, and providing a single connection for sending messages to a wide variety of carriers.

### 4.10.2.	Settings

Enabled - check this box to enable the GetTap interface.  If the PageGate server is already running before you check this box, you will need to manually start this interface by right-clicking on 'GetTap' and choosing 'Start' from the popup menu.  The next time the PageGate server is started, this interface will startup automatically.

Serial Port - choose a serial port (COM port) for the GetTap module to monitor for incoming TAP sessions.  A modem is typically connected to this port.

Baud Rate - choose a baud rate to use for this connection.  The default baud rate is typically 1200.

Parity - choose the parity to be used for this connection.  The default parity is typically Even.

Data Bits - choose the number of data bits to be used on this connection.  The default data bits is typically 7.

Stop Bits - choose the number of stop bits to be used with this connection.  The default stop bits is typically 1.

Init String - the init string is a set of commands sent to the modem to configure it for incoming calls and to turn on or turn off features of the modem.  For example, most systems using the TAP protocol communicate at 1200 baud with not error correction or data compression.  Here are some sample modem init strings:

AT&FS0=1  - this init string resets the modem to factory defaults and sets the modem to answer on the first ring (this should be the minimum included in an init string).

AT&FS0=1Q0V1X4&D2N0S37=5\N0%C0  - this init string resets the modem to factory defaults, sets the modem to answer on the first ring, locks the baud rate to 1200 baud, turns off data compression, and turns of error correction (and a few other things).

For direct serial connections (no modem) set the init string to the word 'direct' (without the quotes).

### 4.10.3.	Starting & Stopping

The GetTap interface can be started or stopped by using the PG Admin program.  Right-click on the 'GetTap' object (found under the 'Interfaces' object) and then choose 'Start' or 'Stop' from the popup menu.  The GetTap interface will start or stop within a few seconds.  If the GetTap interface is enabled, it will be automatically started each time the PageGate server is started.

## 4.11.	GetWeb (webpage gateway)

### 4.11.1.	Overview

The GetWeb module enables PageGate to take messages that have been entered in a form on a web page, and send them to a pager (or other wireless messaging device).  It is responsible for retrieving these messages from a web server.  A CGI program (webgate.exe) is included with GetWeb to be used with the web server.  Webgate is a Windows CGI program and therefore can only be used with Windows based web servers.  When recipient accounts are added into PageGate, personal web pages, for the recipients, can be automatically created from web page templates (some example templates are included).  Optionally, custom designed web pages can be used for each recipient.  Groups defined within PageGate can also have web pages automatically created for them.  Other web page options are: A generic web page that can be used to send a message to any recipient by entering the recipient's name.  An Ad-Hoc web page where a pager's PIN (ID) number can be entered to send a message.  And, a drop-down list web page where the sender can choose one or more names from a list of recipients.  Web pages can even be hosted on any number of remote servers, and still use the webgate CGI program on the local server to send messages to PageGate.

### 4.11.2.	Settings

Enabled - check this box to enable the GetWeb interface.  The interface cannot be enabled unless valid paths are provided for the 'Web Pages Path' and the 'CGI Data Path'.  If the PageGate server is already running before you check this box, you will need to manually start this interface by right-clicking on 'GetWeb' and choosing 'Start' from the popup menu.  The next time the PageGate server is started, this interface will startup automatically.

Web Pages Path - sets the location where, on the web server, PageGate will automatically generate web pages for recipients (if desired).

CGI Data Path - sets the location where the GetWeb interface collects the data files generated by the webgate.exe CGI program.

Polling Interval - sets how often (in seconds) the GetWeb interface checks for new messages from the web.

### 4.11.3.	Starting & Stopping

The GetWeb interface can be started or stopped by using the PG Admin program.  Right-click on the 'GetWeb' object (found under the 'Interfaces' object) and then choose 'Start' or 'Stop' from the popup menu.  The GetWeb interface will start or stop within a few seconds.  If the GetWeb interface is enabled, it will be automatically started each time the PageGate server is started.

### 4.11.4.	Web server configuration

#### 4.11.4.a.	Overview

PageGate can be used with any Windows based web server.  Although we are unable to give details here on every available web server, integrating PageGate into an existing web server is a simple process with very straight-forward concepts.  Please refer to your web server's documentation for the exact steps necessary to accomplish the requirements outlined in the following sections.

#### 4.11.4.b.	Setup

1.  PageGate can automatically create web pages for each of its recipients and groups.  In order for this to be possible, PageGate must have full rights to the webpage directories used by the web server.  In PageGate's GetWeb settings, the 'Web Pages Path' must be set to this directory.

2.  PageGate uses a CGI program to collect data from the generated (or custom made) web pages.  Most web servers setup, by default, a directory to hold CGI programs (usually named 'Scripts', 'cgi-bin', or 'cgibin').  If not, one must be setup.  The web server program must have sufficient rights to be able to execute the CGI programs.

3.  PageGate's CGI (webgate.exe) creates temporary data files as part of its operation.  This CGI must have sufficient rights to create these files.  In most cases, the temporary files are created in the same directory as the CGI (Microsoft IIS users, please see  the notes regarding IIS at the end of this section).

4.  The PageGate application must have full access to the temporary files produces by the webgate CGI program.  In PageGate's GetWeb settings, the 'CGI Data Path' must be set to this directory.

5.  If the location of the temporary files is different than the location of the CGI program (webgate.exe), you may have to manually copy the webgate.exe program from the PageGate program directory to the web server's CGI directory.

*Notes for Microsoft IIS (web server)

- IIS creates a default location for CGI programs.  It is named 'Scripts' and can be found under the 'inetpub' directory.  This is the best place to put PageGate's CGI program (webgate.exe), as it is already setup with all the proper rights.  If you use a different location for your CGI programs, be sure that this location has all the proper rights to run CGI programs.  If not, you will most likely receive 400 or 500 series errors in you web browser when trying to send messages to PageGate.

- Different version of IIS store the webgate CGI's temporary data files in different locations.  Some versions put them in the same directory as the CGI ('Scripts' by default).  This is how most other web servers work.  Other versions of IIS put the temporary data files in the web pages root directory ('wwwroot' by default).  If PageGate doesn't pickup any messages when the 'CGI Data Path' is set to the 'Scripts' folder, try setting it to the 'wwwroot' folder.

#### 4.11.4.c.	Advanced web server concepts

In order for PageGate to collect data from web pages, the webgate.exe CGI program's data files must be directly accessible by PageGate's GetWeb module.  In most cases, this means that the web server must be running on the same LAN (local area network) as PageGate.  Also, for availability reasons, the web server needs to have a dedicated (see 'Solution3' below for an option that can use a dialup connection) connection to the internet.  This does not mean, however, that all your organization's web pages must reside on this local web server.  Here are some examples:

Although your location has a dedicated internet connection, it is not fast enough to host your organization's web pages (your connection may be a dedicated dialup connection or something else less than a T1).  Because of this, your webpages are stored on a remote web hosting company's server (often a virtual or co-located host) which has multiple high-speed internet connections.

Problem:  PageGate's CGI program can't be put on the remotely hosted web server because its data files are not directly accessible by PageGate.

Solution1:  Run an additional web server in-house that hosts only the web pages used by PageGate and PageGate's CGI (webgate.exe).  The web pages on the remotely hosted website could easily link to the locally hosted web pages when necessary.  This way the CGI's data files will be on the local server and accessible by PageGate.  There would be minimal bandwidth needed for this configuration.

Solution2:  Don't setup PageGate to automatically create web pages for you.  Create your own web pages for sending messages, and keep the pages on the remotely hosted web server.  Put the PageGate CGI on an in-house web server.  In the web pages that reside on the remotely hosted web server, point the CGI to execute from the in-house web server (the path in the 'Form Action' line of the webpage).  This way, the CGI's data files will be on the local server and accessible by PageGate.  There would be very little bandwidth used for this configuration.

Solution3:  Although this solution is not supported by NotePage, Inc., several customers have made this work.  Put PageGate's CGI program (webgate.exe) up on the remotely hosted web server.  Do not set PageGate to automatically generate web pages.  Create your own web pages for sending messages and put them up on the remotely hosted web server too.  The webgate CGI program will create its data files up on the remotely hosted web server (this can only work if the remote web server is Windows based).  Next setup an FTP program at the local site to periodically log onto the remotely hosted website and download the CGI's data files (several shareware applications are available with this capability).  Setup the PageGate GetWeb module's 'CGI Data Path' to the location where the CGI data files are downloaded to by the FTP program.  This configuration can be used with a dialup internet connection if a dedicated connection is not available.

### 4.11.5.	Using the webgate.exe CGI

#### 4.11.5.a.	Overview

When a person fills out a form on a web page and presses the submit button, a program is run by the web server and given the information that was typed into the form.  One kind of program that can be run by the web server is called a CGI.  The webgate.exe CGI program is included with the GetWeb interface for this purpose.  Webgate must be put on the web server in a directory where the server looks for CGI programs.  This is usually the cgi-bin or scripts directory.  When you specify the CGI Data Path in PageGate's Admin program, the webgate.exe program is copied to that location.  The webgate.exe CGI can be named as the form action in any web page to enable the form's data to be sent to it.  Here is a simple web page example that uses webgate.exe:

<HTML>

<HEAD>

<TITLE>

Sample Pager Web Page

</TITLE>

</HEAD>

<BODY>

<H2><CENTER>Enter a message to be sent to a pager</CENTER></H2>

<HR>

<FORM ACTION ="http://www.mydomain.com/cgi-bin/webgate.exe" METHOD=POST>

<CENTER>FROM</CENTER>

<CENTER>

<INPUT SIZE=40 MAXLENGTH=100 NAME="FRM"></TEXTAREA>

</CENTER>

<BR>

<CENTER>MESSAGE</CENTER>

<CENTER>

<TEXTAREA COLS=60 ROWS=5 WRAP=VIRTUAL NAME="MSG"></TEXTAREA>

</CENTER>

<BR>

<CENTER>

<INPUT TYPE=SUBMIT NAME = "SEND" VALUE = "SEND PAGE">

<INPUT TYPE=RESET VALUE = "CLEAR FORM">

</CENTER>

<INPUT TYPE=HIDDEN NAME="USER" VALUE="pageruser">

</FORM>

</BODY>

</HTML>

In this example, please notice the path used for the webgate.exe CGI program.  In web pages the path should be set to the full URL of the CGI program.  You should substitute your domain name for 'mydomain.com'.  The recipient name of the pager in this example is 'pageruser'.  Every web page that is to use webgate must have the following variables included in them.  They can be either entered by the user of the webpage, or hard coded into the form:

FORM ACTION - naming webgate.exe as the form's action

FRM - a text field that will contain the name of who the message is being sent from

MSG - a text field that will contain the actual message text

USER - a text field that will contain the name of the recipient (pager) the message is to be sent to

Here are some optional fields that can be used:

NOPAGE - if this field is set to 'NOPAGE', the message won't actually be sent to the recipient's pager.  It will however be forwarded to the recipient's email forwarding address (if GetMail is setup).  This would turn the web page into a sort of web to email gateway.

SUBJECT - to use the Multi-Page option, a SUBJECT field is used for the recipient names.

This is an example where the pager's recipient is not hard-coded into the web page, and can be typed manually each time (also the NOPAGE option is given):

<HTML>

<HEAD>

<TITLE>

PageGate's Page-A-User Web Page

</TITLE>

</HEAD>

<BODY>

<H2><CENTER>Fill in This Form to Send a Page</CENTER></H2>

<FORM ACTION ="http://www.mydomain.com/cgi-bin/webgate.exe" METHOD=POST>

<CENTER>

USER <INPUT SIZE=40 MAXLENGTH=40 NAME="USER"></TEXTAREA>

</CENTER>

<BR>

<CENTER>

FROM <INPUT SIZE=40 MAXLENGTH=80 NAME="FRM"></TEXTAREA>

</CENTER>

<BR>

<CENTER>MESSAGE</CENTER>

<CENTER>

<TEXTAREA COLS=60 ROWS=5 WRAP=VIRTUAL NAME="MSG"></TEXTAREA>

</CENTER>

<BR>

<CENTER>

<INPUT TYPE=SUBMIT NAME = "SEND" VALUE = "SEND PAGE"><INPUT TYPE=RESET VALUE = "CLEAR FORM">

</CENTER>

</FORM>

</BODY>

</HTML>

#### 4.11.5.b.	Custom webpages

Custom web pages can be used with PageGate instead of pages created from templates.  Simply check the 'Custom Page' option on the recipient's web page setup screen to tell PageGate not to create web pages for that recipient.

#### 4.11.5.c.	Templates

A custom web page could be written for each recipient, however this approach may be overwhelming for a large-scale installation.  PageGate allows you to create web page template files that are used to automatically create web pages for its recipients.  When you are creating a new recipient, or editing an existing recipient in PageGate's Admin program, you can use the 'WebPage' object to create or edit the web page for the current recipient (the GetWeb service must be enabled for this recipient to access this object).  First, you must choose a template file from the drop-down list (template files belong in the 'templates' directory beneath the 'Web Pages Path' directory).  After a template is chosen, enter the text messages to be inserted into the template file.  Below is an example template file that would be used to create a web page similar to the first example above (this template is shipped with PageGate and named basic.htm).

<HTML>

<HEAD>

<TITLE>

~~~text1~~~

</TITLE>

</HEAD>

<BODY>

<H2><CENTER>~~~text2~~~</CENTER></H2>

<HR>

<FORM ACTION ="http://www.mydomain.com/cgi-bin/webgate.exe" METHOD=POST>

<CENTER>FROM</CENTER>

<CENTER>

<INPUT SIZE=40 MAXLENGTH=100 NAME="FRM"></TEXTAREA>

</CENTER>

<BR>

<CENTER>MESSAGE</CENTER>

<CENTER>

<TEXTAREA COLS=60 ROWS=5 WRAP=VIRTUAL NAME="MSG"></TEXTAREA>

</CENTER>

<BR>

<CENTER>

<INPUT TYPE=SUBMIT NAME = "SEND" VALUE = "SEND PAGE">

<INPUT TYPE=RESET VALUE = "CLEAR FORM">

</CENTER>

<INPUT TYPE=HIDDEN NAME="USER" VALUE="~~~user~~~">

</FORM>

</BODY>

</HTML>

PageGate uses text substitution with the template files to insert message text and recipient names into the template files (notice ~~~text1~~~, ~~~text2~~~, and ~~~user~~~ in the above example).  If you chose this template while setting up a new recipient in PageGate, you would set the Text1 field on the recipient's web page setup screen to 'Sample Pager Web Page', and the Text2 field to 'Enter a message to be sent to a pager'.  The ~~~user~~~ field will automatically replaced with the recipient's name.  Up to ten text variables (~~~text1~~~ through ~~~text10~~~) can be used in the template files for automatic substitution.  In addition, up to five image variables(~~~image1~~~ through ~~~image5~~~) can be used to substitute image file names into the template (these files must exist in the \images directory beneath the 'Web Pages Path' directory).

When PageGate creates a web page for a recipient, it creates a directory under the 'Web Pages Path' directory that is specified on the GetWeb's settings screen.  The directory is named the same as the PageGate recipient's name.  The web page is named default.htm (and copied to index.htm) and placed into the newly created directory.  This structure allows for easy addressing of recipient web pages.  For example if the 'Web Pages Path' was set to the root directory of the web server at 'mydomain.com', and a recipient was created with the name 'chris', you could get Chris' web page by going to www.mydomain.com/chris.  If the root directory of the web server were c:\webserver\wwwroot, Chris' web page file would be c:\webserver\wwwroot\chris\default.htm (or index.htm).

#### 4.11.5.d.	Drop-down list pages

Group list web pages enable PageGate to maintain web pages that have a drop-down list of recipient names on them.  These pages allow web users to choose one or more names from a drop-down list to send a message to.  When recipients belonging to groups that have drop-down lists change, the drop-down list web pages are automatically updated.  To enable Group List web pages, you must first have a group setup in PageGate with recipients in it.  Create a web page for the group by choosing the 'WebPage' object below the group's object in PG Admin.  Choose a Group List template file to be used (an example called list.htm is included).  Fill in any needed text or image fields and check the 'Group List' option box.  A Group List web page will be created for that group.  Optionally you can check the 'Full Names' option box.  This option will display the recipient's full name in the web page's drop-down list, in addition to the recipient's user name.  When a message is entered from the web into the Group List web page, only the recipients selected from the list will be sent the message, not the entire group (unlike any other group web page).  If there are groups as members of this group, their entries will be prefixed with a '{G}' in the list. Below is an example of a Group List web page:

<HTML>

<HEAD>

<TITLE>

~~~text1~~~

</TITLE>

</HEAD>

<BODY>

<H2><CENTER>~~~text2~~~</CENTER></H2>

<HR>

<FORM ACTION ="http://www.mydomain.com/cgi-bin/webgate.exe" METHOD=POST>

<INPUT TYPE=HIDDEN NAME="USER" VALUE = "~~~user~~~">

<CENTER>

<SELECT NAME="SUBJECT" SIZE=5 MULTIPLE>

~~~list~~~

</SELECT>

</CENTER>

<BR>

<CENTER>

FROM

</CENTER>

<CENTER>

<INPUT SIZE=40 MAXLENGTH=100 NAME="FRM"></TEXTAREA>

</CENTER>

<BR>

<CENTER>MESSAGE</CENTER>

<CENTER>

<TEXTAREA COLS=60 ROWS=5 WRAP=VIRTUAL NAME="MSG"></TEXTAREA>

</CENTER>

<BR><BR>

<CENTER>

<INPUT TYPE=SUBMIT NAME = "SEND" VALUE = "SEND PAGE"><INPUT TYPE=RESET VALUE = "CLEAR FORM">

</CENTER>

</FORM>

</BODY>

</HTML>

#### 4.11.5.e.	Ad-Hoc

Messages can be sent to pagers by using their PIN (ID) numbers.  This option is only available with the 5000 pager license or greater.  The advantage of this feature is that a user account for each pager does not have to be setup in PageGate.  Only a single Ad-Hoc recipient needs to be setup for each carrier.  The disadvantage is that the sender must know the PIN (ID) numbers of the pagers (harder to remember than recipient names).  Sending messages using Ad-Hoc is similar to sending Multi-Page messages.  In PageGate's Admin program, create a recipient and set it as an Ad-Hoc recipient.  The carrier information must be correct; it is used to set which carrier the messages are sent to. With the Ad-Hoc recipient set as the web page's USER field, the SUBJECT field is used to receive the PIN (ID) numbers.  The PIN (ID) numbers can be separated by spaces, commas, or semi-colons:

<HTML>

<HEAD>

<TITLE>

PageGate's Ad-Hoc Web Page

</TITLE>

</HEAD>

<BODY>

<H2><CENTER>

Fill in This Form to Send a Page

</CENTER></H2>

<FORM ACTION ="http://www.mydomain.com/cgi-bin/webgate.exe" METHOD=POST>

<INPUT TYPE=HIDDEN NAME="USER" VALUE = "put ad-hoc user name here">

<CENTER>

TO <INPUT SIZE=60 MAXLEN=200 NAME="SUBJECT"></TEXTAREA>

</CENTER>

<BR>

<CENTER>

FROM <INPUT SIZE=40 MAXLENGTH=80 NAME="FRM"></TEXTAREA>

</CENTER>

<BR>

<CENTER>MESSAGE</CENTER>

<CENTER>

<TEXTAREA COLS=60 ROWS=5 WRAP=VIRTUAL NAME="MSG"></TEXTAREA>

</CENTER>

<BR><BR>

<CENTER><INPUT TYPE=SUBMIT NAME = "SEND" VALUE = "SEND PAGE"><INPUT TYPE=RESET VALUE = "CLEAR FORM"></CENTER>

</FORM>

</BODY>

</HTML>

#### 4.11.5.f.	MultiPage

A message can be sent to many PageGate recipients (pagers) by using the Multi-Page feature.  A Multi-Page recipient must first be setup in PageGate's Admin program.  It is simply used as a flag, the recipient information is not important. With the Multi-Page recipient set as the web page's USER field, the SUBJECT field is used to receive the actual recipient names.  The recipient names can be separated by spaces, commas, or semi-colons:

<HTML>

<HEAD>

<TITLE>

PageGate's Multi-Page Web Page

</TITLE>

</HEAD>

<BODY>

<H2><CENTER>

Fill in This Form to Send a Page.

More than one username can be input

</CENTER></H2>

<FORM ACTION ="http://www.mydomain.com/cgi-bin/webgate.exe" METHOD=POST>

<INPUT TYPE=HIDDEN NAME="USER" VALUE = "put multi-page username here">

<CENTER>

TO <INPUT SIZE=60 MAXLEN=200 NAME="SUBJECT"></TEXTAREA>

</CENTER>

<BR>

<CENTER>

FROM <INPUT SIZE=40 MAXLENGTH=80 NAME="FRM"></TEXTAREA>

</CENTER>

<BR>

<CENTER>MESSAGE</CENTER>

<CENTER>

<TEXTAREA COLS=60 ROWS=5 WRAP=VIRTUAL NAME="MSG"></TEXTAREA>

</CENTER>

<BR><BR>

<CENTER>

<INPUT TYPE=SUBMIT NAME = "SEND" VALUE = "SEND PAGE"><INPUT TYPE=RESET VALUE = "CLEAR FORM">

</CENTER>

</FORM>

</BODY>

</HTML>

#### 4.11.5.g.	Returning to a specific webpage

When the user submits a message to PageGate, they are given a response webpage indicating the success or failure of their request.  In either case, on the response webpage an option 'Back to Message Form' is given.  This normally returns the user to the webpage form.  By setting the variable BACK_REF in the webpage, you can specify another webpage to return to rather than the form just used.

Example: (this text would be included in the web page)

<INPUT TYPE=HIDDEN NAME="BACK_REF" VALUE="http://www.mydomain.com/UseThisPage">

#### 4.11.5.h.	Custom response webpages

When a user submits a valid message to PageGate, they are given a response webpage indicating the acceptance of the messages.  You can use your own webpage to indicate the message was accepted.  You can create a standard html webpage named webgate.rsp and put it into the same directory as the webgate.exe CGI program.  If it exists, this webpage will be used instead of the standard response page.  If you want to make this file different for each user, it can be saved in the same directory as the recipient's webpage (if it exists in the user's webpage directory, it will be used before the one in the CGI directory)

#### 4.11.5.i.	Nopage option

The webgate CGI recognizes a field called 'nopage'.  If this option is selected on the webpage, the message is accepted into PageGate, but no message is sent to the recipient's pager.  If PageGate's email interface is active and the forwarding option for the recipient is setup, this option can turn the web form into a web-to-email gateway.

Here is some example HTML code to use the 'nopage' option:

<INPUT TYPE=CHECKBOX NAME="NOPAGE" VALUE = "NOPAGE">Send Email Only (do not send page)

#### 4.11.5.j.	Calling webgate.exe directly

For more sophisticated webpages that use javascript, java, or another higher-end language than HTML, the webgate.exe CGI program can be called directly rather than using it only with an HTML form.  Here is a sample calling command that can be used in the URL field in a browser to call webgate.exe:

http://www.mydomain.com/scripts/webgate.exe?USER=pageme&FRM=fromme&MSG=This+is+a+test

This would send a message to the recipient 'pageme', from 'fromme', with a message of 'This is a test'

#### 4.11.5.k.	Additional Notes

Some knowledge of writing web pages is required to write template files (or any other web page for that matter).  The example web page files and templates (anyuser.htm, ad-hoc.htm, basic.htm, list.htm, and multipage.htm) should be used and studied before making your own.  When a web page form is submitted, the webgate CGI program checks for the existence of all required fields, and checks the recipient names against a list of valid PageGate recipients.  If any of these check fail, the web user is given an error message screen telling them what they did wrong.  The web user can then backup, fix the problem, and resubmit the form.  If the form data is accepted, the web user is given a screen stating that the message has been successfully accepted for processing.

## 4.12.	PageGate GUI Client

### 4.12.1.	Overview

The PageGate GUI (Graphical User Interface) Client is the Windows based graphical client for the PageGate Paging application.  It allows workstations on a network to send messages to the PageGate paging server, which in turn delivers the messages (via a modem, a direct cable connection, or email) to your paging carrier(s) for delivery.  Your paging carrier will then send the message to the appropriate pager, phone, or wireless messaging device.  The PageGate GUI Client program has many advanced features such as: personal recipient lists, personal pre-programmed messages, scheduled messages, repeating messages, and spell checking.  The PageGate GUI Client is easy to install and has a very intuitive user interface.

Documentation for the use of the NotePager Net GUI client can be found on-line from within the PageGate GUI Client program, or in the windows start menu on any machine that has the PageGate GUI Client installed on it.

### 4.12.2.	Settings

Enabled - check this box to enable the use of PageGate GUI Clients.

### 4.12.3.	Starting & Stopping

The PageGate GUI Client can be started with the 'PageGate Client' icon in the Windows Start menu.

To stop the PageGate GUI Client, choose 'Exit' from the 'File' menu, or click on the 'X' control on the client program window.

When the PageGate GUI Client is minimized, it will turn into a pager icon in the Windows system tray (bottom right corner of the screen, next to the clock).  The PageGate GUI Client's regular windows can be restored by clicking on this icon.

### 4.12.4.	Additional Notes

Denying Access to the Network recipient List:

PageGate GUI Clients can be restricted to only sending messages to recipients in their local recipient list.  This blocks them from the network recipient list to which they have access to by default.  This option is enabled from the 'Setup' screen inside the PageGate GUI Client program.  Check the 'Local Recipient List Only' box to stop the user from seeing the network recipient list.  You would also want to password protect the setup screen so that the user can't simply uncheck this setting.  A password can be set for the PageGate GUI Client setup screen from the PG Admin program: click on the 'Password Protect' object found under the 'Program' object to access this setting.

Centralized Local Recipient Lists:

The PageGate GUI Client can be setup to use shared local recipient lists.  To use this functionality, specify a local recipient list file in the PageGate GUI Client's local recipient list setup screen.  The local recipient list files are simple ascii files that contain a single recipient on each line of the file.  The local recipient list files can be created and maintained from a central location by using a text editor.

# 5.	Concepts

## 5.1.	Ad-Hoc

Typically, a recipient is entered into PageGate's database (by using the PG Admin program) for each pager (or other wireless messaging device) that messages will be sent to.  For installations with a large number of recipients, maintaining a large database can be cumbersome.  PageGate's Ad-Hoc option enables it to send messages to recipients that aren't setup in PageGate's database.  When this feature is being used, the pager's ID/PIN number must be included in the message.  PageGate then uses this ID/PIN when it delivers the message the paging carrier.  To use Ad-Hoc, a single Ad-Hoc recipient is setup for each carrier.  Messages that should be sent to a carrier are addressed to the Ad-Hoc recipient for that carrier.  Each of PageGate's interfaces has a slightly different way of handling Ad-Hoc messages.  Please refer to the documentation on each interface for details on how it handles the Ad-Hoc feature.

## 5.2.	Carrier

A carrier is the company you pay for your paging service.  They control the antennas, satellites, broadcast, and computer systems that are used to send the messages to your pager.  You may purchase your paging service from a reseller who represents several paging carriers.  They resell the paging service, but are not actually the paging carrier.  When setting up PageGate, you will need some information about the actual carrier that your pager is serviced by (see the installation part of the documentation for specifics).

## 5.3.	Failover

Failover is the ability of PageGate to send an undeliverable message to another recipient, group, or out another connection.  The failover setting at the recipient level allows you to set another recipient or group that should receive the message if it is undeliverable to the original recipient.  Sometimes this is the same physical device that is reached through another method.  The failover settings at the carrier level is used to redirect all the pending messages to another carrier when the original carrier is not able to be contacted.  Usually the second carrier is simply another connection method for the first carrier.  Here are a couple of examples of how it can be used:

Recipient level failover - A message is not able to be delivered to a pager so the message is delivered to the same person's mobile phone.  If the message is unable to be delivered to the mobile phone, an email is sent to a supervisor to notify them of this.

Carrier level failover - Message are normaly sent to the carrier over the internet (SNPP or WCTP).  The internet connection is down and the messages are unable to go out.  The message then are redirected to another carrier setup in PageGate that delivers the message to the same actual carrier, but does it via a dialup modem connection (TAP).

## 5.4.	Group

Groups can be created to help sending the same message to many recipients.  After a group is created you can add several recipients (or other groups) into the group's member list.  Messages can be sent to a group in exactly the same way messages are sent to individual recipients.  The message, however, is delivered to all the members of the group.  *Note: be careful not include any groups within a group that has the original group as a member.  This would create a never-ending circle of messages.

## 5.5.	Hold Messages

PageGate's 'Hold Messages' option can be found in the 'Misc' object under each 'Recipient' object.  It enables messages for a recipient to be put on hold until a later time of day.  This feature is used most often to hold non critical messages during the night, until regular business hours in the morning.

## 5.6.	Init String

What is an init string?

Different modems can't necessarily talk to each other as set right out of the box.  Modems use many different settings and protocols.  In order for two modems to communicate, they must agree on many things such as speed, error correction, compression algorithms, etc...  By sending a modem an init string, you can control some of these parameters.

What do I need to put in my init string?

In general, the modems used by paging carriers run at a speed from 300 to 2400 baud (1200 being the most common).  They don't typically use any form of error correction or data compression protocols.  In many cases, if your modem tries to use either of these protocols, your modem will be unable to communicate with the paging terminal's modem.  In your init string you should include settings to lock you modem's baud rate to that of your carrier's modem, disable error correction protocols, and disable data compression protocols.  In addition PageGate would like your modem to issue English response codes (rather than numeric response codes), and issue complete connection information.

Do all modems use the same init string?

No, many different modem manufacturers use their own set of modem commands.  Init strings may even be different between models by the same manufacturer.

My init string works on one carrier but not the other.

You may need a different init string to connect to each carrier.  In general your init strings will be the same, but not always.

What init string should I use?

There is no easy answer to this.  In general, if your modem is a 2400 baud model or slower (very old models) you won't need an init string.  These older modems sometimes work best when communicating with the paging terminals.  Anything faster (9600, 14.4, 28.8, 33.6, 56K, etc.), will probably require an init string.  Some of the most common init strings are listed below.  If you have a less well known brand modem, you may need to use the documentation include with you modem to create an init string.

Common Init Strings

AT&FQ0V1X4&D2   - Minimal settings.  Works with most 1200 and 2400 baud modems. Some modems use &F0 rather than &F.  All '0' are zeros not ohs.

AT&FQ0V1X4&D2N0S37=5   - Hard coded baud rate.  Works with many 14.4 and 28.8 modems.  The added codes locks the connection baud rate to 1200 baud.  Use S37=3 for a 300 baud connection, and S37=6 for a 2400 baud connection.  Some modems use &F0 rather than &F.  All '0' are zeros not ohs.

AT&FQ0V1X4&D2N0S37=5\N0%C0   - No compression/error correction.  Works with many 14.4, 28.8 and 33.6 modems.  The added codes turn off data compression and error correction.  You can try using only one of the two additional codes (\N0 or %C0) if you modem rejects using both.  Use S37=3 for a 300 baud connection, and S37=6 for a 2400 baud connection.  Some modems use &F0 rather than &F.  All '0' are zeros not ohs.

AT&F0Q0V1X4&D2&B1&K0&M0&N2   - USRobotics (3Com) string.  Works with most USRobotics/3Com 33.6 and 56K modems (and some others too).  Use &N1 (instead of &N2) for a 300 baud connection, or &N3 for a 2400 baud connection.

AT&FQ0V1X4&D2N0+MS=B103  - This string has been found to work with some of the newer winmodems.  Set the baud rate to 300 when using this string.

Note: all '0' characters in the init strings are zeros not ohs

***More init strings may be available on our web site: www.notepage.net

## 5.7.	Interfaces

PageGate has several front-end interfaces, which allows it to accept messages from many sources.  Each interface can be used by itself, or in conjunction with the other interfaces.  Some of the interfaces gain added capabilities when used with each other.  For example, if the email gateway interface is used at the same time as the web interface, a copy of each message received via a web page can be forwarded to the recipient's normal email account (this gives the recipient a record of messages received in his/her email mailbox).  Currently supported front-end interfaces are: Windows GUI client (PageGate Client), Commandline/Ascii Interface, Email Gateway, and Web Gateway.

## 5.8.	Log Files

Each of PageGate's server modules can keep a log of the messages it processes.  PageGate also keeps a general log of all messages that it has sent.  A detailed log option is also available which, when enabled, provides more detailed information in each log.  The log files are accessed from the PG Admin program.

## 5.9.	Message Content

Messages sent by PageGate are sent with more information than just the message itself.  Messages will be prefixed with the reliapage index (if enabled), the block number (if it is part of a large message that has been broken into smaller pieces.  i.e. 1 of 3, 2 of 3, 3 of 3), the sender's name (can be optionally disabled), the subject of the message (optional), and then the message itself.  Each of these fields is separated with a colon (:).  Most pagers have a limited character set usually consisting of upper and lower case letters, numbers, and some symbols (periods, commas, dollar signs, exclamation points, etc.).  Non-printing characters such as carriage returns, line feeds, and tabs can optionally be removed from a recipient's messages.

## 5.10.	Message Length

PageGate supports messages of up to 2000 characters in length.  Not all carriers will support messages of this length.  If a recipient's character limit is set larger than the carrier's character limit, the message will be automatically broken into smaller messages and numbered (i.e. 1of3, 2of3, 3of3), and then sent sequentially.  The numbering of messages is important because, although PageGate sends the messages in the correct order to the paging carrier, the messages may not be received by the pager in the same order (this is possible due to the way the paging systems operate).

## 5.11.	Modularity/Scalability

PageGate has a very modular design that allows for its high scalability.  On busy systems up to 16 intelligent dialers, each controlling its own modem or direct cable connection, can be used.  Each of PageGate's modules can be run on a separate computer (or all on a single computer).  With all of PageGate's modules in use, the PageGate application could actually be spread across more than 10 machines.  This architecture enables PageGate to handle a very large volume of messages per hour.

## 5.12.	MultiPage

PageGate's MultiPage feature enables it to accept a single message that is addressed to multiple recipients.  Only a single MultiPage recipient needs for to use the MultiPage feature.  When a message is sent to a MultiPage recipient, PageGate knows to look for a list of the actual recipients in the message itself.  For example, when using the MultiPage feature with email, you would address the email message to the name of the MultiPage recipient and provide a list of the actual recipients on the subject line of the email.

## 5.13.	Notify Only

Recipients in PageGate can be set to 'Notify Only'.  When this option is enabled, a predefined (settable) message (notification code) is sent instead of the actual message.  This is most commonly used to notify a recipient with a numeric pager that an email message or a message from the web has been received (PageGate's email or web gateways must be running to receive email or web messages).

## 5.14.	Numeric (digital) Pagers

Some pagers are only capable of displaying numbers (no letters).  These are commonly called numeric or digital pagers (beepers).  PageGate fully supports numeric pagers.  If a message is sent to a numeric pager, and the message contains letters, the recipient's notification code (999 by default) is sent instead.  This alerts the recipient that they have been sent a message that was alphanumeric in nature, and they should take steps to retrieve the actual message.  *Note: this option is very popular with PageGate's email gateway.  Recipients with numeric pagers can be notified if they receive an email message (Email Notification).  They can then connect to the Internet and check their email.

## 5.15.	On-Call Group

PageGate has built into it a feature than can be very useful in an on-call service environment.  A schedule of recipients can be setup defining when a recipient is on-call (starting day and time, and ending day and time).  These recipients are then added to a group that has been defined as an on-call group.  When a message is sent to the group, the schedule is consulted to see which recipient or recipients the message should be sent to.  This way, only the recipients that are scheduled to be on-call receive the page.

## 5.16.	Pager (Beeper)

Throughout PageGate's documentation the word 'pager' is used.  'Pager' is better defined as 'wireless messaging device'.  PageGate can be used to send messages to many devices other than pagers.  For example messages can be sent to digital/cellular/mobile/pcs phones, pager cards for portable computers, Personal Information Managers (PIMs), billboards, and many other wireless messaging devices.

## 5.17.	Recipient

In PageGate, 'recipient' is synonymous with 'pager'.  A recipient is the person who carriers the actual pager or wireless messaging device. When PageGate refers to a 'recipient' it is really referring to the recipient's pager or wireless messaging device.  This approach simplifies things for the end users.  For example if a recipient were to replace their pager with a new unit, the recipient name will stay consistent, even though the pager's information would change.  Note: in some previous versions of PageGate the term 'user' was used instead of 'recipient'.

## 5.18.	Reliapage

As you are probably aware, traditional paging uses a one-way transmission.  There is no confirmation sent back from the pager that messages have (or have not) been received.  In poor coverage areas, messages can be, and are missed.  PageGate's Reliapage option sequentially numbers each message sent out.  A numbering sequence is maintained for each individual recipient.  This way, if a recipient receives messages with a number missing, they will know that they have missed a message.  Messages for an individual or group of users can be resent from within PageGate's administration program.

## 5.19.	Repeating Pages

PageGate allows you to send a repeating message.  The message's start date and time is set.  A repeat interval is set (repeat every ? minutes, hours, and days).  Then a stop date and time is set.  This option is currently only available when using the PageGate GUI Client application.

## 5.20.	Reports

PageGate has several useful reports to help you manage the application.  Log files containing PageGate's activity are found under each module's object in the PG Admin program. Other reports include: current recipients and their settings, current carriers and their settings, message usage by recipient, and all messages sent within a time period to recipients.

## 5.21.	Scheduled Pages

PageGate allows you to schedule a message to be sent at a future date and time.  This option is currently only available when using the PageGate GUI Client application.

## 5.22.	Touch-Tone Messages

Touch-tone messages are messages sent by calling a pager's phone number and sending a series of tones (like a touch tone phone) as a message.  The tones are converted to their equivalent numbers and sent to the recipient's pager.  Some paging carriers do not accept messages for numeric pagers via the regular TAP (IXO/PET) protocol.  They only accept touch-tone numeric messages.  PageGate  supports delivering numeric messages in this manner, but it is not the preferred method.  This method of message delivery has no provisions for error detection/correction or message confirmation.  It is sometimes known as 'blind dialing' because there is no way to really know if the other end actually received the message.

How to setup a recipient for touch-tone messages:

First a 'Tone' carrier must be setup.  Only a single 'Tone' carrier may be needed to send message to all your numeric pagers.  To make a carrier a 'Tone' carrier, choose 'Tone' as the 'Delivery Method' on the carrier setup screen.  The default wait string ',,@' should work for most pagers.  If you need to use different wait string for different pagers, you will need to create multiple 'Tone' carriers (one for each unique wait string needed).  The wait string can be changed to support pagers with different input methods.

The comma (,) is used to pause (about one second)

The at sign (@) is used to wait for silence (about five seconds of silence)

The default wait string will dial the pager's number, pause for a couple of seconds, and then wait for a five second period of silence (after the beeps or greeting message) and then send the message (touch tones).

To setup a recipient to use touch-tone delivery, simply put the numeric pager's phone number in the 'ID/PIN' field on the recipient setup screen, and choose the 'Tone' carrier you just setup, as the carrier.  If you have setup several carriers with different wait strings, choose the carrier that will work with this pager's service provider.  With this setup, PageGate will dial the phone number setup in the recipient's ID/PIN field, pause according to the commas or @ in the carrier's 'Wait String' field, and then send the touch-tone message.

Some paging providers use a common phone number for all numeric pagers.  The user must dial this number, wait for a tone, dial their pager's ID number, wait for another message, and then type the numeric message.  This can be accomplished in PageGate by setting the 'Tone' carrier's wait string to wait a predetermined amount of time (using commas and or an @ symbol).  This would  be the wait time between sending the pager's ID number and sending the actual touch-tone message.  On the recipient setup screen, in the recipient's 'ID/PIN' field, put the paging company's common phone number followed by a few commas or an @ symbol, and then the pager's ID number.  This will give the following results:  The phone number will be dialed from the first part of the recipient's ID/PIN field,  PageGate will pause according to the commas or @ in the ID/PIN field, PageGate will then send the second set of number in the recipient's ID/PIN field and then pause according to the carrier's wait field.  PageGate will then send the touch-tone message.

# 6.	Technical Information

## 6.1.	Adding Modules to PageGate

Adding a module to PageGate is easy:

1. If you are adding an additional dialer, you must first check that the new dialer's program files is in the PageGate program directory.  The downloadable evaluation version of PageGate contains only two dialers.  Additional dialers must be copied from the WebGate CD-Rom (you will receive one when you register PageGate), or must be obtained by contacting NotePage, Inc.'s tech support department.  The files are named PGDialX.exe, where the X is the dialer number (i.e. dialer 5 would be named PGDial5.exe).

2. Next, if you are running PageGate as a System Service (on NT, 2000, or XP), you must switch PageGate back to run as an application (temporarily).

3.  In the PG Admin program click on the 'Settings' object under the 'Program' object.  In the section labeled 'Run on this server' check the box(s) for the additional modules you want to run and hit 'Apply'.

4. If you were running PageGate as a System Service, switch it back now.

## 6.2.	Advanced Installation

PageGate's flexibility and scalability is due to its modular design.  The modularity of PageGate allows each of its modules to be run on a separate computer.  Here is an example:

- The PageGate Scheduler module and the PageGate Database run on the production file server.

- The GetMail module runs on a separate server

- The GetWeb module runs on the web server

- 1 to 16 Dialer modules run on a communications server

- The PageGate Administration and Monitoring programs run on an administrator's workstation

- The PageGate GUI Client runs on several workstations on the LAN

In contrast, all of the above modules can run on a single workstation or server.

Steps to install PageGate's modules on additional computers:

1.  Run PageGate's setup program.  Install the PageGate application on the new workstation or server (refer to the regular installation instructions for details).

2.  Run PageGate on the new computer.

3.  The first time PageGate is run, you will be given the 'PageGate Setup Screen' (this screen can be accessed later by choosing 'Settings' from the 'Program' object in the Admin program).

4.  Set the 'Program Directory' to where you installed PageGate on the local machine.

5.  Set the 'Database Directory' to the same location as in the existing PageGate installation (this would need to be a shared folder on the network).

6.  Set the 'Log Directory' to the same location as in the existing PageGate installation (this would need to be a shared folder on the network).

7.  Choose which of PageGate's modules are to run on this computer.  Be sure to disable the modules you choose on any existing PageGate servers.  If you are installing PageGate simply to use the Administration program, leave all the module boxes unchecked.

8.  Apply your changes.  PageGate will now start..

9.  Use the above procedure on each computer that will run one or more of PageGate's modules.

## 6.3.	Integration

Integration Overview

PageGate is designed to easily integrate with other applications, giving them wireless messaging capabilities.  Once integrated, your application will be able to send messages to all the recipients and groups that have been setup in PageGate, and even send messages through PageGate for recipients that aren't actually setup in PageGate's database (see the Ad-Hoc section of this help file for more information).

GetAscii Module

The GetAscii Module, also known as the Commandline/Ascii interface is the most commonly used module to integrate other software with PageGate.  There are four different ways the GetAscii interface can be used to accept messages from other applications: commandline, text file, registry polling, windows messages.

Commandline:

PageGate includes two executable file that can be run to pass messages into its message queue: sendpage.exe and sendpage32.exe.  Sendpage.exe is a 16-bit program, and sendpage32 is a 32-bit program.  Depending on how the programs are to be called, you may need to use one or the other.  To use the executables, run them and provide the recipient, sender, and message information as parameters on the commandline.  For example, to send a message to Joe from Brian with the message Testing 123, you could use this command: sendpage Joe Brian Testing 123.  Detailed information about the use of the sendpage programs can be found in this help documentation in the 'GetAscii' section (under 'Modules').

Text file:

PageGate can extract message information from text files.  By default the GetAscii module looks for text files in a single location, with a predefined file format, and a predefined naming convention.  The GetAscii module also supports 'Advanced Polling Options', which enable GetAscii to read files from multiple folders and from files that aren't in the standard format.  GetAscii can even 'watch' files for additions or changes, and then extract message data from them.  Detailed information about the use of the GetAscii text file interface can be found in this help file in the 'GetAscii' section (under 'Modules').

Registry Polling:

The GetAscii interface can accept messages that are passed to it via the Windows registry.  Messages written to a predefined location in the registry will be picked up by the GetAscii interface.  The application to be integrated with PageGate must be running on the same computer as the GetAscii module to use this integration method.  Here is what you should do:

Create a new key under this existing key: HKEY_LOCAL_MACHINE\Software\NotePage\Transfer\Messages\.  The name of the new key should be unique so it doesn't get overwritten by the next message.  Under the new key, the following values should be created:

Ready - REG_DWORD - should be initially set to zero (0)

To - REG_SZ - the PageGate recipient or group to receive the message

From - REG_SZ - the sender of the message

Message = REG_SZ - the message text

After all the values are written, set the 'Ready' value to one (1).  This signals PageGate that the entry is complete and can be read in.  These new entries are automatically removed by the GetAscii interface after they are read in.

Windows Messages:

This option enables other software to send new messages to the GetAscii interface by sending it a 'Windows Message'.  Windows Messages can be sent by using the SendMessage() Windows API call.  The application to be integrated with PageGate must be running on the same computer as the GetAscii module to use this integration method.  This method may require a little more programming experience that the previous methods, but it is more efficient than the previous methods, and is fairly easy to implement.  Here is the syntax of the API call:

SendMessage(PGHandle, WM_COPYDATA, MyHandle, CopyDataStructure)

Please refer to a Windows API reference for syntax and usage information

The current PageGate handle (PGHandle) can be read from the registry at: HKEY_LOCAL_MACHINE\Software\NotePage\Transfer\Handle.

The CopyDataStructure should contain a string value with the following format:

sendpage recipient sender message

The 'sendpage' text must be a the beginning of the string.  The 'recipient' label should be replaced with the name of an existing PageGate recipient or group.  The 'Sender' label should be replaced with the sender's name.  The 'message' label should be replaced with the actual text of the message to be sent.

How to tell if the GetAscii interface is currently running:

1. Read the PageGate database file (pagegate.mdb).  The current location of the PageGate database can be read from the registry:  HKEY_LOCAL_MACHINE\Software\NotePage\Transfer\DBDir.  It is in a format compatible with MS Access 2000.  In the 'Control' table, the 'getascii_state' field will be True if the GetAscii module is running.

2. Read the registry.  The registry value HKEY_LOCAL_MACHINE\Software\NotePage\Transfer\Running will have a value of 1 when GetAscii is running

3. Use the OpenEvent API call.  When GetAscii starts it creates a Windows event named 'NotePageIsRunning'.  This event is destroyed when GetAscii stops.  The API call OpenEvent(0,0,"NotePageIsRunning") will return a non-zero result if the GetAscii interface is running.

How to get a list of current PageGate recipients and groups:

1. Read the PageGate database file (pagegate.mdb).  The current location of the PageGate database can be read from the registry:  HKEY_LOCAL_MACHINE\Software\NotePage\Transfer\DBDir.  It is in a format compatible with MS Access 2000.  The 'Users' table has an entry for each valid PageGate recipient and group.

2. Read the registry.  The GetAscii interface keeps an up-to-date list of all the PageGate recipients and groups in the Windows registry: HKEY_LOCAL_MACHINE\Software\NotePage\Transfer\Recipients and HKEY_LOCAL_MACHINE\Software\NotePage\Groups.

Direct Database Access

Developers that are familiar with database access can directly integrate with PageGate's database.  This is probably the most efficient way to add messages to the message queue, but may not be an option in all programming environments.  Another downside to this method is that it may be version specific (if the database structure changes in a future version of PageGate, the integration may stop working).  PageGate uses a Jet 4.0 database as it's back end.  This is a MS Access 2000 compatible database.  If you have the ability to write to MS Access 2000 databases, then you can write to the PageGate's database directly.  New messages can be added to the PageGate's message queue by adding a new record to the 'messages' table in the 'pagegate.mdb' database.  The field definitions and descriptions of the 'messages' table follows:

user - text field - name of an existing recipient or group to send message to

pin - text field - not used, but must be set to a zero length string ("") - can't be Null

message - memo field - text of the message to be sent

carrier - text field - not used, but must be set to a zero length string ("") - can't be Null

date_time_requested - date field - date and time to send message (usually the current time)

date_time_sent - date field - not used

frm - text field - name of sender

sent - boolean field - set to false

error - boolean field - set to false

ascii_file - text field - not used, but must be set to a zero length string ("") - can't be Null

mail_file - text field - not used, but must be set to a zero length string ("") - can't be Null

lock - boolean field - set to false

scheduled - boolean field - set to false

reliapage_index - numeric field - not used

repeat_interval - numeric field - not used

repeat_stop - date field - not used

station - numeric field - should be set to a number > 10 can be used to when printing reports to identify source of the messages

message_id - auto number field - not used

retries - numeric field - not used

last_try - date field - not used

transaction - numeric field - must be a unique number (long integer) - best way to handle this is to edit the control database record, read the current value of the 'unique' field, increment it, and then write this value back to the control table.  The transaction number can be reused after about an hour

dialer - numeric field - not used

GetMail Module

More and more modern applications are building in the ability to send email.  If your application supports email, it can easily be integrated with PageGate by using PageGate's email interface.  The GetMail interface can be used to integrate  applications running on different platforms (Unix, Mac, etc.).  Please refer to the GetMail section of this manual for more details.

Web Interface

Webpage developers can use the GetWeb interface to send messages to PageGate from their webpages (HTML, ASP, JavaScript, Java, etc.).  A small CGI program (webgate.exe) is used to accomplish this.  The webgate CGI can be used with either a POST or GET method.  The POST method can be used by creating a form on your webpage and pointing to the webgate.exe program in the FORM ACTION statement.  The GET method is more popular with scripting languages such as JavaScript and Java.  It allows the webgate CGI to be called with a command similar to one that could be entered into a browser's URL field.  The GetWeb interface can be used to enable PageGate to be integrated with applications running on different platforms (Unix, Mac, etc.).  Please refer to the GetWeb section of this manual for more details.

## 6.4.	Run Styles

PageGate can be configured to run in three different styles, depending on the operating system that is on the PageGate computer: Application, Windows  9x (95 or 98) Service,  and Windows NT Service.

Here are the differences:

Application - This style will run on either Windows 9x or Windows NT.  PageGate is started and stopped just like any other program.  PageGate can be automatically run at startup by putting a shortcut to it in the Windows startup group.  The disadvantages to this style, is that if the Windows 9x computer is configured to show a login screen at startup, PageGate  can't automatically start until someone logs onto the computer.

Windows 9x service - This style will run only on a Windows 95  or 98 computer.  This style is the same as the application style with the exception that PageGate is automatically started each time the computer restarts (prior to the login screen).

Windows NT service - This style will run only on a Windows NT, 2000, or XP computer.  This option will run PageGate as a true NT system service.  Like other services, PageGate will startup even if the computer is just sitting at the Windows NT login screen.  Here are a few important notes regarding running PageGate as an NT system service:

1. When switching the run style to an NT service, you will be prompted for a user account and password for PageGate to use to log itself in.  You can leave both these fields blank to use the local computer's System Account (as long as PageGate's doesn't need to open files on another computer).  Otherwise, choose an account that already has 'Log on as a service' and 'Administrator' rights (before switching).  The administrator account does NOT have the 'Log on as a service' right by default.  You must add this right manually to the user account you intend to use.  Follow these instructions to give an existing account 'Log on as a service' rights:

a. Click on 'Start' > 'Programs' > 'Administrative Tools' > 'User Manager'.

b. From the menu choose 'Policies' and then 'User Rights'.

c. Check off 'Show Advanced Rights'

d. Choose 'Log on as a service' from the drop-down listbox.

e. Click the 'Add' button

f. Choose the user account you will be using for PageGate.

g. Click 'Add' and then 'OK'

h. Click 'OK' again

i. Click 'Quit' and then quit out of the User Manager program

2. If you change the password on the account that PageGate is using, you must update it for each of PageGate's services too.  You can use the 'Services' utility from the Windows control panel to do this, or you can switch PageGate to run as an Application and then back to run as a NT service, so you will be re-prompted for the account information.

3. The account you choose to use with the PageGate service must have sufficient rights to access all directories setup in PageGate for its various functions (i.e. program files, database directory, log files, interface directories, etc…).

4. Like many services, PageGate may not be completely started before you log in and get the Windows desktop.  Be patient.  On computers that load several services, it can take several minutes for all the services to start.

5. In the Windows NT 'Services' utility (found in the control panel), you will see several PageGate services installed.  ONLY the one labeled 'PageGate' is set for automatic startup.  This is by design and should not be changed.

6. Starting or stopping PageGate when it is running as a service can take several minutes, especially if it is in the middle of transmitting messages to a carrier.  Please be patient.

7. You can easily start or stop PageGate's services be clicking on the pager icon in the system tray, and choosing 'Start' or 'Stop' from the menu.

8. If you are running PageGate's modules across more than one computer, each computer will run the 'PageGate' service along with the specific module(s) setup to run on that computer.  In other words, there should  only be a single instance of any module running, with the exception of the 'PageGate' service (which will be running on every computer).

## 6.5.	Uninstalling PageGate

To uninstall PageGate you must use the 'Add/Remove Programs' icon in the Windows control panel.

When uninstalling PageGate you are given two choices for what type of uninstall you want to perform: 'Uninstall Application Files Only' and 'Uninstall Entire Application'.  The first choice will leave your database and program settings intact.  This option is typically used to re-install or upgrade PageGate.  The second option removes all programs, data files, and settings from the computer.

PageGate should not be running when attempting to uninstall PageGate.  All PageGate GUI Clients should also be shutdown before attempting to uninstall PageGate.  The uninstall process can take several minutes and may require you to restart your computer when it has finished.

# 7.	Troubleshooting

## 7.1.	Installation

Problem:  Any number of different problems

Solution:  Be sure that there are no other programs running when you install PageGate.  There may be programs running even if you don't see them in your task bar.  Pressing <ctrl><alt><del> will bring up a list of running programs in Windows 95 and 98.  In Windows NT you can choose 'Task Manager' after hitting <ctrl><alt><del> for similar information.  Stop any program that you believe is causing the conflict.  WARNING: this should only be done by an experienced user.  WARNING: Randomly ending tasks can lock up your computer, lose data or worse.  Please consult with and experienced technician before killing any processes.  A clean reboot can sometimes also help.  Holding down the shift key while windows is starting will stop some programs from starting that automatically load at startup.  Try re-running the PageGate setup program.

## 7.2.	Init Strings

What is an init string?

Different modems can't necessarily talk to each other as set right out of the box.  Modems use many different settings and protocols.  In order for two modems to communicate, they must agree on many things such as speed, error correction, compression algorithms, etc...  By sending a modem an init string, you can control some of these parameters.

What do I need to put in my init string?

In general, the modems used by paging carriers run at a speed from 300 to 2400 baud (1200 being the most common).  They don't typically use any form of error correction or data compression protocols.  In many cases, if your modem tries to use either of these protocols, your modem will be unable to communicate with the paging terminal's modem.  In your init string you should include settings to lock you modem's baud rate to that of your carrier's modem, disable error correction protocols, and disable data compression protocols.  In addition PageGate would like your modem to issue English response codes (rather than numeric response codes), and issue complete connection information.

Do all modems use the same init string?

No, many different modem manufacturers use their own set of modem commands.  Init strings may even be different between models by the same manufacturer.

My init string works on one carrier but not the other.

You may need a different init string to connect to each carrier.  In general your init strings will be the same, but not always.

What init string should I use?

There is no easy answer to this.  In general, if your modem is a 2400 baud model or slower (very old models) you won't need an init string.  These older modems sometimes work best when communicating with the paging terminals.  Anything faster (9600, 14.4, 28.8, 33.6, 56K, etc.), will probably require an init string.  Some of the most common init strings are listed below.  If you have a less well known brand modem, you may need to use the documentation include with you modem to create an init string.

Common Init Strings

AT&FQ0V1X4&D2   - Minimal settings.  Works with most 1200 and 2400 baud modems. Some modems use &F0 rather than &F.  All '0' are zeros not ohs.

AT&FQ0V1X4&D2N0S37=5   - Hard coded baud rate.  Works with many 14.4 and 28.8 modems.  The added codes locks the connection baud rate to 1200 baud.  Use S37=3 for a 300 baud connection, and S37=6 for a 2400 baud connection.  Some modems use &F0 rather than &F.  All '0' are zeros not ohs.

AT&FQ0V1X4&D2N0S37=5\N0%C0   - No compression/error correction.  Works with many 14.4, 28.8 and 33.6 modems.  The added codes turn off data compression and error correction.  You can try using only one of the two additional codes (\N0 or %C0) if you modem rejects using both.  Use S37=3 for a 300 baud connection, and S37=6 for a 2400 baud connection.  Some modems use &F0 rather than &F.  All '0' are zeros not ohs.

AT&F0Q0V1X4&D2&B1&K0&M0&N2   - USRobotics (3Com) string.  Works with most USRobotics/3Com 33.6 and 56K modems (and some others too).  Use &N1 (instead of &N2) for a 300 baud connection, or &N3 for a 2400 baud connection.

AT&FQ0V1X4&D2N0+MS=B103  - This string has been found to work with some of the newer winmodems.  Set the baud rate to 300 when using this string.

Note: all '0' characters in the init strings are zeros not ohs

***More init strings may be available on our web site: www.notepage.net

## 7.3.	Connecting to a carrier

### 7.3.1.	Make sure your modem is operating correctly

HyperTerminal is a terminal emulation program that comes with Windows.  It can be used to test communications with your modem.  You should do this test even if other applications work correctly with your modem.  PageGate talks to your modem by addressing its serial port.  Other applications may bypass the port settings and work even if your port settings are incorrect.  HyperTerminal can be found under the 'Accessories' option on your Windows start menu.  Depending on the version of Windows you are running, it may be under a group below 'Accessories' called 'Communications'.

If HyperTerminal is not installed, you can install it by choosing 'Add/Remove Programs' from within the Windows control panel.  Choose the tab labeled 'Windows Setup'.  HyperTerminal can be found under the 'Communications' group.

After choosing HyperTerminal from accessories, invoke the HyperTerminal program by either choosing the 'Hyperterm' icon from the group window, or by choosing 'HyperTerminal' from the next group of programs on the Windows start menu.  If it is the first time you have run HyperTerminal, you may be asked to enter your area code.  If so, do it now.  The next screen will ask you for a name for your new connection.  The name is not important.  Use something like 'test'.  Hit 'OK'. On the next screen, among other things, you are asked for what modem to use ('Connect Using').  Do not choose the name of your modem.  Instead, choose the name of the com port you entered into PageGate (i.e. 'Direct to Com 2').  Hit 'OK'.  The next screen will ask you for 'Port Settings'.  Use the settings that were used when the carrier in question was setup in PageGate.  Choose 'None' for flow control.

You should now get a somewhat blank HyperTerminal window with a blinking cursor.  If you don't, then your modem or com port settings are incorrect, your hardware is not setup correctly, or some other program is using the modem.  Otherwise, you should now be able to type 'AT <cr>' (don't type the ' symbols, and ,<cr> stands for the carriage return or enter key).  The letters 'AT' may or may not appear on the terminal window.  Don't be alarmed if they don't.  However, in either case, your modem should respond with and 'OK'.  If you don't receive an 'OK' then your serial port or modem is not operating correctly and must be fixed before PageGate can use them.  If you do get an 'OK', then continue to the next step.

### 7.3.2.	Make sure you have a dial tone

If you can hear PageGate pick up the phone and get a dial tone before it dials, skip to the next step.  If you don't currently have HyperTerminal running, follow the steps in the above section so that HyperTerminal is running and responds with an 'OK' when you type 'AT<cr>'.

Type 'ATDT<cr>'. You should hear a dial tone.  Some modems may have their speakers turned off.  That is ok.  After a little while you should get one of two messages on the HyperTerminal screen: 'No Carrier' or 'No Dialtone'.  'No Carrier' is a good response, you have dialtone.  'No Dialtone' … well that one needs no further explanation.

### 7.3.3.	Make sure you are connecting to a modem

Using a regular phone, dial the phone number you entered into PageGate.  Now is a good time to note that many modem lines may have different requirements to get an outside line than a phone system does (i.e. just because you dial a '9' to get an outside line on your phone system, you may not need one on your modem line, or vice versa).  After dialing the number, you should hear it ring and then pickup.  Next you should hear a long high-pitched tone (at least 10 seconds and steady).  If you don't, you have the wrong number for your paging service provider's terminal.  Often times if you are given the wrong number, you will hear a voice message or a beep-beep-beep.

### 7.3.4.	Make sure you are connecting to a paging terminal

If you have gotten this far, you are at least calling a modem.  Next, we have to be sure we can connect to the modem on the other end, and that it is a paging terminal that supports the TAP/IXO protocol.

Follow the steps in the first section so that you are running HyperTerminal and are getting an 'OK' response to typing 'AT<cr>'.  Now type 'ATDT<phone number><cr>' where <phone number> should  be replaced with the phone number you entered into the carrier in PageGate.  After the modem dials it should connect to the modem at the other end and you should get a 'CONNECT' message on the HyperTerminal screen.  If you can't connect, either the phone number is wrong (there really isn't a paging terminal at the other end) or you need to use an init string to make a connection to it.  You will find some common init strings in the 'Init Strings' section or on our website (www.notepage.net).  To try an init string, simply type it into HyperTerminal and hit the enter key.  If the string is accepted, you should get an 'OK' back.  If the string is invalid for your modem, you will get and 'ERROR' response.  After entering a valid init string, try connecting to the carrier again ('ATDT<phone number><cr>').  If HyperTerminal can't connect to the carrier, PageGate won't be able to either.

After you receive a 'CONNECT' message, you should receive an 'ID=' message within about five seconds.  If not, try hitting the enter key about every three seconds.  After retrying the carrier a couple of times, if you still don't get and 'ID=' prompt, either you don't have a proper init string, your communications settings are incorrect (baud, data bits, etc.), or you aren't connecting to a paging terminal.  If you do receive the 'ID=' prompt, quit out of HyperTerminal, start PageGate, and go to the next step.

Note:  If you used a different init string here to get this far, be sure to update the carrier's init string in PageGate before continuing.

### 7.3.5.	Check the PageGate Dialer Log

In the PageGate Admin program, select the 'Settings' object found underneath the 'Logging' object.  Make sure 'Log to Disk' and 'Detailed Log' are both checked.  Apply your changes.  You can view the dialer's log file by selecting the 'Log' object from under the 'Dialer1' object, which is found under the 'Connectors' object.  Next, send a test message and then review the dialer log.  Here are failure messages for the dialers and their possible causes:

Unable to send dialer init string - the dialer init string was sent to the modem, but not accepted.  Verify the syntax of the init string, or try a new one.

Unable to send carrier init string - the carrier init string was sent to the modem, but not accepted.  Verify the syntax of the init string, or try a new one.

Unable to connect to carrier - the dialer didn't receive a 'CONNECT' message from the modem.  This is usually caused by a wrong phone number, incorrect modem settings, or improper init string.

Checksum error - your message was transmitted to the carrier, but the data received didn't match the data you sent.  This can be caused by a bad connection or improper init string.

Bad message content - your message was received by the carrier without any errors.  However, the carrier rejected the message due to invalid data in it.  This is almost always caused by an incorrect ID/PIN number, or by sending a message that is longer than what the carrier will accept in a single message (try lowering the 'maxchars' setting for the carrier).

Disconnect requested by carrier - this message could mean a number of different things.  It is sometimes simply given because the carrier has a limit to the number of messages it will accept in a single session, and wants you to hang up.  In most other cases, the terminal has detected a condition which prohibits it from continuing to accept messages, and wants to terminate the session.

No response from carrier - somewhere in the middle of transmitting messages, contact was lost with the carrier.  This is usually caused by a bad phone connection, or the need for a better init string.

Message retry limit reached - the message has been retried the number of times specified in the global dialer settings, without being sent successfully.  The message at this point is marked as bad, and not retried again.  Please check the error message previous to this one in the log for causes.

Carrier retry limit reached - This means that a specific carrier has been unable to be reached after being retried for the number of times set in the global dialer settings.  Check the log for failure messages prior to this one for more information on how the connection failed.  This message is usually due to an outage of service at the carrier or a modem/phoneline/hardware failure.  All pending messages currently queued for this carrier are marked as bad, so as not to delay the delivery of message to other carriers that are still accessible.

## 7.4.	Misc

### 7.4.1.	I receive the error 'Unable to access modem'

This message is given if PageGate is unable to talk to your modem.  Check the port you have set PageGate to use for the dialer in question.  If you have an external modem, make sure it is connected and turned on. There may be another program running on your system that is using the modem. For example, you will get this error if you have a fax program running in the background waiting to receive faxes, or you may have a remote control program such as PCAnywhere running in the background waiting for a call. PageGate requires exclusive use of the modem. For more information, please refer to the section 'Connecting to a carrier' in the 'Troubleshooting' chapter of the PageGate help file.

### 7.4.2.	I receive the message 'Warning unable to access the database'

This message is warning you that one of PageGate's modules has temporarily lost contact with the PageGate database.  If PageGate is running on a real busy computer or network, you may intermittently receive this message.  This can be normal in a busy environment.  PageGate is simply letting you know that it did not get through right away, and it will keep trying.  However, if the module is unable to re-connect to the database, this error will eventually lead to the error 'Program halted. Database error...' (see the information that follows).

### 7.4.3.	I receive the error 'Program halted.  Database error...'

This error is the result of one of two different problems.  Either one of PageGate's modules has lost contact with the PageGate database, or there is some corruption in the PageGate database.  If the PageGate module that reports this error is running on a different computer than where the PageGate database file resides, chances are you have a network connectivity problem (possible an intermittent one).  If the PageGate module that reports this error is running on the same computer as the PageGate database file, there is most likely corruption in the PageGate database.  Corruption is caused by one of the following conditions:

1. Abnormally terminating PageGate

-  PageGate should ALWAYS be shut down in an orderly manner.  PageGate's administration program and ALL of the running PageGate GUI Clients should be exited before shutting down the computer that PageGate runs on.  Next, each of PageGate's modules should be set to a 'stopped' state (red in PG Monitor).  The PageGate service manager should be stopped by clicking on the pager icon in the Windows system tray and then choosing 'Quit' or 'Stop' from the popup menu.  All of the above should be done before shutting down or restarting the computer that PageGate is running on.  Not following these procedures can in some cases cause database corruption.

-  All PageGate GUI Client programs should be closed by choosing 'File' and then 'Exit' from their menus, or by clicking on the 'X' control on their windows (Note: when NotePager Net is minimized, it appears as a pager icon in the Windows system tray.  Be sure not to shut down or restart with it still there.).  Not following this procedure can in some cases cause database corruption.

2. Freezing, locking, abnormal shutting down/rebooting, or other hardware failures

-  If any computer that is running ANY part of PageGate or its GUI client (NotePager Net) freezes, locks up, reboots, shuts down, catches on fire, etc. without using normal shutdown procedures, in some cases can cause corruption in the PageGate database.

3. Operating system errors

-  If computers running ANY part of PageGate (including the PageGate GUI Client) receive errors such as: 'the blue screen of death', general exception errors, processor exception errors, illegal instruction errors, page fault errors, etc., in some cases the PageGate database can become corrupt.

4. Incorrect or out-of-date database support files

-  PageGate uses the Microsoft Jet database engine (common with MS Access 97) for data access.  If any of the database engine files are out of date (older versions are buggy), corruption can appear in the PageGate database.  Please refer to the section 'File version problems' in the 'Troubleshooting' chapter of the PageGate help file for more details.

5. Bad or out-of-date operating system files.

-  If PageGate is running on Windows NT, be sure that the NT Service Pack 5 or later is installed on all computers that run ANY part of PageGate or the PageGate GUI Client.

-  If PageGate is running on Windows 95 or 98, make sure ALL computers that run ANY part of PageGate or the PageGate GUI Client have the current Windows redirector and OLE files.  In particular, there is a serious problem with the redirector shipped with some versions of Windows 95 that will almost certainly cause database corruption.  Please refer to the section 'File version problems' in the 'Troubleshooting' chapter of the PageGate help file for more details.

### 7.4.4.	When I start the PG Admin program, I get the error 'Error 35602 key is not unique in collection...'

PageGate will never allow two recipients (users) with the same user name.  If you are inserting records directly into the PageGate database by your own means, be sure that you don't allow duplicates.  This is the message you will get if there is one.

### 7.4.5.	I have a US Robotics (3 Com) modem. I have tried the init string but still get inconsistent results.

Some model US Robotics modems have difficulty communicating with the computer at low baud rates (i.e. under 9600 baud). Try setting the baud rate on the carrier setup screen to 19200. Also use the init string given to lock the baud rate between the modems to a lower rate.

### 7.4.6.	I am certain that everything is set correctly, but PageGate won't dial with my modem.

We have seen problems with some of the new faster modems communicating with the computer at lower baud rates. Set the carrier's baud rate 19200 and use a modem initialization string to lock the modem to modem connection speed to 1200 baud.

### 7.4.7.	I have a 56K modem and am unable to connect to the paging carrier.

Many new 56K modems are 'voice' modems, which means that they support features such as speakerphone, voice mail, sound recording, etc.  In order for these features to be supported, the modem locks itself in a high-speed connection mode which can't be over-ridden by using init strings.  To disable the voice capabilities of the modem follow these instructions:

1. From the Windows start menu choose: 'Start' > 'Settings' > 'Control Panel'

2. From the control panel double-click on the icon labeled sYstem.

3. On the 'System Properties' screen click on the tab labeled 'Device Manager'

4. Find the entry called 'Sound, video and game controllers' (if there isn't one,

you don't have a voice modem)

5. Click on the '+' sign next to the entry

6. Select the entry under 'Sound, video and game controllers' called: 'Wav device for voice modem' (if there isn't one there, you don't have a voice modem).

7. Click on the 'Properties' button.

8. The properties screen for the device will appear.  Check the box labeled 'Disable in this hardware profile'.

9. Click 'OK' and then 'OK' again.

10. Restart your computer.

### 7.4.8.	My modem does not hang up after sending a message.

You may need to use an init string (or a different one) with your modem.

### 7.4.9.	My pager keeps receiving garbled messages, why?

Garbled messages are caused by a low battery, poor reception (inside some buildings or in fringe areas), or a defective paging unit.

### 7.4.10.	PageGate says the message was sent, but it is never received on the pager, why?

If PageGate indicates that the message has been sent successfully, it means that the paging terminal has confirmed receipt of the message. If the message is not received, you may be out of your paging coverage area (or in a fringe area), your account may be incorrectly set up by the carrier, or you may have an incorrect ID/PIN number.

### 7.4.11.	When I try to install, I get a "missing runtime DLL" message.

Try clearing out the files in your c:\windows\temp directory and then reinstalling.  If this doesn't help, re-download the product.  The download may not have completed successfully.

### 7.4.12.	When I try to send a message through a web page nothing happens. My server is NT 4.0 and my web server is Microsoft IIS.

Put the webgate.exe (CGI) in inetpub/scripts directory this is where IIS is expecting to find the CGI programs, and be sure to point the getweb interface to the same directory.

### 7.4.13.	Messages aren't being acknowledged by either the scheduler or the dialers.

The scheduler module and the dialer modules currently only support US date format (mm/dd/yy). Check your date format setting in the control panel under 'Regional Settings' to be sure this is the current format.

### 7.4.14.	PageGate stopped sending messages at the end of the month

The scheduler module and the dialer modules currently only support US date format (mm/dd/yy). Check your date format setting in the control panel under 'Regional Settings' to be sure this is the current format.

### 7.4.15.	I'm using commandline/ascii interface and I am trying to send message from notepad.

The notepad program, always appends a .txt extension to its files. PageGate requires .asc as the last extension on the file. Rename the file manually.

### 7.4.16.	With Windows 2000, I can't get GetMail to work

Some installations of Windows 2000 installs a 'Simple Mail Transport Protocol (SMTP) service by default.  Although you probably aren't using this service, it will conflict with PageGate's email capabilities.  To disable Microsoft's SMTP service go to the Windows control panel. Double-click on the 'Administrative Tools' icon.  Double-click on the 'Services' icon.  Double-click on the entry: 'Simple Mail Transport Protocol (SMTP)'.  Change the 'Startup type' to 'Manual' or 'Disable'.  Click on the 'Stop' button.  Then click on the 'OK' button.

### 7.4.17.	With Windows 2000, I can't get PageGate to use my modem

Some installations of Windows 2000 installs a 'Fax Service' by default.  If this 'Fax Service' is running, you will be unable to use the modem it is using for PageGate.  To disable the 'Fax Service' go to the Windows Control Panel.  Double-click on the 'Administrative Tools' icon.  Double-click on the 'Services' icon.  Double-click on the entry: 'Fax Service'.  Change the 'Startup type' to 'Manual' or 'Disable'.  Click on the 'Stop' button.  Then click on the 'OK' button.

# 8.	FAQs

## 8.1.	How does your software treat email messages with graphic encapsulation e.g. attachments, jpg, MIME ?

Only the text portions of the email body are sent to the pager. If the user account set to forward messages to an email account. The full message will be forwarded to that account.

## 8.2.	Will PageGate support more than one modem per server ?

PageGate supports up to 16 dialers and direct cable connections.

## 8.3.	Will PageGate queue messages on the PageGate server for when dialout lines are busy?

Yes

## 8.4.	Can I send the same messages to multiple pagers ?

PageGate v1.4 (and higher) supports groups, which allows groups of pagers to share a single web page.

## 8.5.	Can PageGate run as a service an NT server?

Yes, PageGate version 4.x will run as a true Windows NT system service under Windows NT,  2000, and XP.

## 8.6.	Will PageGate work with a direct connection to the paging terminal (a direct serial connection, no modem) ?

Yes, a direct cable connection can be used in conjunction with other dialers that have modems, to allow pages to be sent externally too.

## 8.7.	Can PageGate be modified to adhere to a different paging protocol, that requires '' character before inputting text ?

Yes, please provide details to products@notepage.com and a price quote for modifications can be provided.

## 8.8.	What operating systems operating systems will PageGate work on?

Although the PageGate engine must run on a Windows 95, 98, ME, NT, 2000, or XP system, pages can be sent from DOS, windows 3.x, OS/2, Mac's and even Unix stations, if they are networked to the PageGate system. In addition with the email or web front-ends, pages may be sent from anything that can do email or www.

## 8.9.	Can PageGate's modules be run on separate machines?

Yes, each of PageGate's modules may be run on a different system. They must all be networked together.

## 8.10.	What mail servers does PageGate work with?

PageGate has all the necessary functionality to be its own mail server.  However, PageGate can also work in conjunction with any existing mail server.  Please refer to the section 'GetMail (email gateway)' for more information on the different ways PageGate's mail server can be configured.

## 8.11.	How do I get technical support ?

PageGate technical support is handled through email. If you have a question that is not answered here e-mail tech@notepage.net.

## 8.12.	What web servers will PageGate work with?

PageGate will work with any windows based web server.

9x Service	81

Additional Notes	51, 59, 70, 71

CGI	70

Commandline/Ascii	51

Email	59

GetAscii	51

GetMail	59

GetWeb	70

getweb.exe	70

GUI Client	71

PageGate GUI Client	71

Ad-Hoc	50, 68, 72

Commandline/Ascii	50

Concepts	72

GetAscii	50

GetWeb	68

webgate.exe	68

Ad-Hoc Mail	52

Advanced ascii polling	46

Advanced Installation	78

Aliases	59

Application	81

Automated Installation	14

Automatic Installation	14, 23

Automatic Startup	81

Beeper	75

Bind to IP	59

Bind to Port	59

Buy	8

Capabilities	4, 5, 6, 7

Commandline/Ascii	5

Email	6

GetAscii	5

GetMai	6

GetWeb	6

GUI Client	7

PageGate	4

Web	6

Capture Printer	60

Capture Serial Data	60

Carrier	72

Troubleshooting 1	84

Troubleshooting 2	84

Troubleshooting 3	84

Troubleshooting 4	85

Troubleshooting 5	85

Carrier Failover	72

Carriers	36, 76

Numeric Pagers	76

Object	36

Touch Tone Delivery	76

CGI	66, 67, 68, 69, 70

Additional Notes	70

Ad-Hoc	68

Custom webpages	66

Drop-down list webpages	67

Group list webpages	67

MultiPage	69

Nopage	70

Response webpages	70

Return webpages	69

Templates	67

Collecting POP Mail	58

Commandline/Ascii	5, 46, 49, 50, 51

Additional Notes	51

Ad-Hoc	50

Advanced ascii polling	46

Capabilities	5

Features	5

MultiPage	50

Overview	46

sendpage.exe	49

Starting & Stopping	49

Text Files	49

Company	8

Company Email	59

Configuration	42

PG Monitor	42

Connecting to the Internet	53

Connection Troubleshooting 1	84

Connection Troubleshooting 2	84

Connection Troubleshooting 3	84

Connection Troubleshooting 4	85

Connection Troubleshooting 5	85

Connectors	35, 44, 45

Dedicated Carriers	45

Direct Connect	45

Multiple Dialers	44

Object	35

Overview	44

Starting & Stopping	44

Touch Tone Delivery	45

Contact	8

Context sensitive help	26

Copyright	9

Custom webpages	66

Database	29

Object	29

Dedicated Carriers	45

Connectors	45

Description	4

Dialers	36, 44, 45

Dedicated Carriers	45

Direct Connect	45

Multiple Dialers	44

Object	36

Overview	44

Starting & Stopping	44

Touch Tone Delivery	45

Dial-Up Networking	53

Digital Pagers	45, 75, 76

Direct Connect	45

Connectors	45

Domain Mail Collection	56

Domain Name	54

Domain POP Mail Collection	53

Domain POP Mail Settings	53

Drop-down list webpages	67

Email	6, 8, 51, 52, 54, 58, 59

Additional Notes	59

Ad-Hoc mail settings	52

Aliases	59

Capabilities	6

Features	6

Filtering	59

MultiPage	58

Overview	51

Starting & Stopping	54

Email Client	59

Email Notification	75

Email Rules	59

End User License Agreement	9

Eudora	59

EULA	9

Failover	72

Fax	8

Features	4, 5, 6, 7

Commandline/Ascii	5

Email	6

GetAscii	5

GetMail	6

GetWeb	6

GUI Client	7

PageGate	4

Web	6

Filtering	59

Email	59

SPAM	59

Forwarding between servers	56

front-ends	73

Full Names	67

General	4

Capabilities	4

Features	4

GetAscii	5, 31, 46, 49, 50, 51

Additional Notes	51

Ad-Hoc	50

Advanced ascii polling	46

Capabilities	5

Features	5

MultiPage	50

Object	31

Overview	46

sendpage.exe	49

Settings	46

Starting & Stopping	49

Text Files	49

GetMail	6, 32, 51, 52, 54, 55, 56, 58, 59

Additional Notes	59

Capabilities	6

Dialup	55

Domain Mail Collection	56

Domain Name	54

Existing Mail Server	56

Features	6

MultiPage	58

Object	32

Overview	51

Settings	52

Starting & Stopping	54

Static IP	54

GetSerial	33, 60, 62

Object	33

Overview	60

Settings	60

Starting & Stopping	62

GetTap	33, 62, 63

Object	33

Overview	62

Settings	62

Starting & Stopping	63

GetWeb	6, 34, 63, 64, 66, 67, 68, 69, 70

Additional Notes	70

Ad-Hoc	68

calling webgate.exe	70

Capabilities	6

Custom Webpages	66

Drop-down list webpages	67

Features	6

Group list webpages	67

MultiPage	69

Nopage	70

Object	34

Overview	63

Response webpages	70

Return webpages	69

Settings	63

Starting & Stopping	64

Templates	67

Global	31, 35

Object (Connectors)	35

Object (Interfaces)	31

Group	72

Group list webpages	67

Groups	39

Object	39

GUI	34, 71

Object	34

Settings	71

GUI Client	7, 21, 22, 23, 24, 71

Additional Notes	71

Capabilities	7

Features	7

Installation (.exe)	22

Installation (.zip)	23

Installation (CD-Rom)	22

Installation (promptless)	23

Overview	71

Post-installation	24

Pre-installation	21

Starting & Stopping	71

Help	26, 41

Object	41

History	74

in-house paging terminal	45

Init String	73

Initialization String	73

Installation	11, 12, 13, 14, 16, 17, 19, 20, 22, 23, 25, 26, 78, 83

Advanced	78

First Message	26

GUI Client	22, 23

Multiple Servers	78

One-time configuration	25

Overview	11

PageGate (.exe)	13

PageGate (.zip)	13

PageGate (CD-Rom)	12

PageGate (promptless)	14

PageGate GUI Client (.zip)	23

PageGate GUI Client (CD-Rom)	22

PageGate GUI Client (promptless)	23

PageGate GUI Clientt (.exe)	22

PG Admin (.exe)	17

PG Admin (.zip)	17

PG Admin (CD-Rom)	16

PG Monitor (.exe)	19

PG Monitor (.zip)	20

PG Monitor (CD-Rom)	19

Starting & Stopping	25

Troubleshooting	83

Integrating	79

Integration	79

Interfaces	31, 73

Object	31

Internet Dialing	53

ISP Settings	53

IXO	62

Java	70

JavaScript	70

Key	41

Length	74

License Agreement	9

Licensing	7

Local Recipient List File	71

Log Files	74

Logging	40

Object	40

Mail bagging	55

Mailbagging	53

Manual	26

Message Content	74

Message Length	74

Minimum Requirements	11, 16, 18, 21

GUI Client	21

PageGate	11

PG Admin	16

PG Monitor	18

Modem Commands	73

Modem Init String	73

Modem Troubleshooting 1	84

Modem Troubleshooting 2	84

Modem Troubleshooting 3	84

Modem Troubleshooting 4	85

Modem Troubleshooting 5	85

Modularity	74

Modules	28

Overview	28

Monitoring PageGate's Modules	27

MultiPage	50, 58, 69, 74

Commandline/Ascii	50

Concepts	74

Email	58

GetAscii	50

GetMail	58

GetWeb	69

webgate.exe	69

Multiple Dialers	44

Connectors	44

Multiple email addresses	59

Multiple IP Addresses	59

Multiple Mail Servers	59

Multiple Mailboxes	58

Netscape	59

Nopage	70

Notify Only	75

NT Service	81

Numeric Messages	76

Numeric Pagers	45, 75, 76

Object	28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 39, 40, 41

Carriers	36

Connectors	35

Database	29

Dialers	36

GetAscii	31

GetMail	32

GetSerial	33

GetTap	33

GetWeb	34

Global (Connectors)	35

Global (Interfaces)	31

Groups	39

GUI	34

Help	41

Interfaces	31

Logging	40

PageGate	28

Program	29

Recipients	37

Registration	41

Reports	41

Scheduler	30

On-Call Groups	75

Outlook	59

Overview	4, 11, 28, 42, 43, 44, 46, 51, 60, 62, 63, 64, 65, 71

Commandline/Ascii	46

Connectors	44

Dialers	44

Email	51

GetAscii	46

GetMail	51

GetSerial	60

GetTap	62

GetWeb	63

GUI Client	71

Installation	11

Modules	28

PageGate	4, 43

PageGate GUI Client	71

PG Admin	28

PG Monitor	42

Scheduler	44

Serial	60

Service Manager	43

TAP	62

Web	63

Web Server	64

webgate.exe	65

PageGate	4, 11, 12, 13, 14, 28

Capabilities	4

Description	4

Features	4

Installation (.exe)	13

Installation (.zip)	13

Installation (CD-Rom)	12

Installation (promptless)	14

Object	28

Overview	4

Post-installation	14

Pre-installation	11

Requirements	11

PageGate GUI Client	22, 23, 71

Additional Notes	71

Installation (.exe)	22

Installation (.zip)	23

Installation (CD-Rom)	22

Installation (promptless)	23

Overview	71

Starting & Stopping	71

PageGate Service	43

Overview	43

Starting & Stopping	43

PageGate:	4

Pager	75

Paging Carrier	72

PET	62

PG Admin	16, 17, 18, 28

Installation (.exe)	17

Installation (.zip)	17

Installation (CD-Rom)	16

Overview	28

Post-installation	18

Pre-installation	16

Requirements	16

Starting & Stopping	28

Using	28

PG Monitor	18, 19, 20, 21, 42

Configuration	42

Installation (.exe)	19

Installation (.zip)	20

Installation (CD-Rom)	19

Overview	42

Post-installation	21

Pre-installation	19

Requirements	18

Starting & Stopping	42

Using	42

Phone Number	8

PIN Mail	52

PIN paging	72

POP Mail Collection	56

POP Mail Collection (Domain)	53

POP Mail Settings	58

POP Server	59

POPMail collection	55

Post-installation	14, 18, 21, 24

GUI Client	24

PageGate	14

PG Admin	18

PG Monitor	21

Pre-installation	11, 16, 19, 21

GUI Client	21

PageGate	11

PG Admin	16

PG Monitor	19

Price	8

Program	29

Object	29

Promptless Installation	14

Purchase	8

Quiet Installation	14, 23

Recipient	75

Recipient Failover	72

Recipients	37

Object	37

Registering	41

Registration	7, 41

Object	41

Reliapage	75

Removing PageGate	82

Repeating Messages	76

Reports	41, 76

Object	41

Response webpages	70

Return webpages	69

RS232	60

Run Styles	81

Running the first time	25, 26

First Message	26

One-time configuration	25

Starting & Stopping	25

Scalability	74

Scheduled Messages	76

Scheduler	30, 44

Object	30

Overview	44

Starting & Stopping	44

sendpage.exe	49

sendpage32.exe	49

Serial	60

Overview	60

Serial Printer	60

Server	11, 12, 13, 14

Installation (.exe)	13

Installation (.zip)	13

Installation (CD-Rom)	12

Installation (promptless)	14

Post-installation	14

Pre-installation	11

Requirements	11

Service Manager	43

Overview	43

Starting & Stopping	43

Service Provider	72

Settings	46, 52, 60, 62, 63, 71

GetAscii	46

GetMail	52

GetSerial	60

GetTap	62

GetWeb	63

GUI	71

Setup	11, 12, 13, 14, 16, 17, 19, 20, 22, 23, 25, 26, 64

First Message	26

GUI Client	22, 23

One-time configuration	25

Overview	11

PageGate (.exe)	13

PageGate (.zip)	13

PageGate (CD-Rom)	12

PageGate (promptless)	14

PageGate GUI Client (.exe)	22

PageGate GUI Client (.zip)	23

PageGate GUI Client (CD-Rom)	22

PageGate GUI Client (promptless)	23

PG Admin (.exe)	17

PG Admin (.zip)	17

PG Admin (CD-Rom)	16

PG Monitor (.exe)	19

PG Monitor (.zip)	20

PG Monitor (CD-Rom)	19

Starting & Stopping	25

Web Server	64

Shareware	7

SMTP Server	59

SPAM	59

Starting & Stopping	28, 42, 43, 44, 49, 54, 62, 63, 64, 71

Commandline/Ascii	49

Connectors	44

Dialers	44

Email	54

GetAscii	49

GetMail	54

GetSerial	62

GetTap	63

GetWeb	64

GUI Client	71

PageGate	43

PageGate GUI Client	71

PG Admin	28

PG Monitor	42

Scheduler	44

Service Manager	43

Web	64

Startup	25, 26, 81

First Message	26

One-time configuration	25

Starting & Stopping	25

Static IP	54

System Requirements	11, 16, 18, 21

GUI Client	21

PageGate	11

PG Admin	16

PG Monitor	18

System Service	81

TAP	62

Templates	67

Text Files	49

Touch Tone Delivery	45, 76

Connectors	45

Touch Tone Messages	76

Troubleshooting	83, 84, 85

Carriers 1	84

Carriers 2	84

Carriers 3	84

Carriers 4	85

Carriers 5	85

Connections 1	84

Connections 2	84

Connections 3	84

Connections 4	85

Connections 5	85

Init Strings	83

Installation	83

Modem 1	84

Modem 2	84

Modem 3	84

Modem 4	85

Modem 5	85

Two Mail Servers	59

Unattended Installation	14, 23

Uninstalling PageGate	82

Unlocking Key	41

Updating	12

Upgrading	12

User	75

Using	28, 42

PG Admin	28

PG Monitor	42

Version	9

Web	6, 63, 64

Capabilities	6

Features	6

Overview	63

Starting & Stopping	64

Web Server	64, 65

Advanced Concepts	65

Overview	64

Setup	64

webgate.exe	65, 66, 67, 68, 69, 70

Additional Notes	70

Ad-Hoc	68

Custom webpages	66

Drop-down list webpages	67

Group list webpages	67

MultiPage	69

Nopage	70

overview	65

Response webpages	70

Return webpages	69

Templates	67

Webpage templates	67

Website	8

Windows 9x System Service	81

Windows NT System Service	81